3GPP TSG-SA5 Meeting #139-e 	S5-215256

- e-meeting, 11 - 20 October 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0086 | rev | 1 | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Technical errors in use case descriptions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some use cases define actors that are not used. Some new text was added in CR 0039, but the corresponding old text was not removed. Some new text was added in CR 0064, but the corresponding old text was not removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove unnecessary text from use case descriptions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect use case descriptions may cause incorrect implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 5.1.3, 5.1.7, 5.1.12, 5.1.13, 5.1.14, 5.1.17, 5.1.19, 5.1.20, 5.1.22, 5.1.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.1.1	Network slice instance allocation

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To satisfy request for allocation of a network slice instance with certain characteristics, by creation of new or using existing network slice instance; the request includes the network slice related requirements. |  |
| Actors and Roles | A network slice provisioniong management service consumer. |  |
| Telecom resources | Network slice instance Network slice subnet instance Transport network A network slice provisioning management service provider. A network slice subnet provisioning management service provider. |  |
| Assumptions | N/A |  |
| Pre-conditions | N/A |  |
| Begins when | The network slice provisioning management service provider receives the request for allocation of the network slice instance with certain characteristics; the request contains network slice related requirements and the information indicating whether the requested NSI could be shared with other consumers. |  |
| Step 1 (M) | If the requested NSI can be shared and if an existing NSI can be used, the network slice provisioning management service provider decides to use the existing NSI. Modification of the existing NSI may be needed to satisfy the network slice instance related requirements. Use case is completed go to “Step 8". Otherwise, the network slice provisioning management service provider triggers to create a new NSI, for which the following steps 2 – 8 are needed. |  |
| Step 2 (M) | The network slice provisioning management service provider decides on the constituent NSSIs and the topology of the NSI to be created using the information from service profile [6]. For the constituent NSSIs, the network slice provisioning management service provider derives network slice subnet related requirements from the network slice related requirements. If reconfiguration of the transport network is needed, the network slice provisioning management service provider derives transport network related requirements (e.g. latency, bandwidth) from the network slice related requirements. |  |
| Step 3 (M) | For the required NSSI(s), the network slice provisioning management service provider sends network slice subnet related requirements to the network slice subnet provisioning management service provider to request allocation of the required NSSI(s). | Network slice subnet instance allocation use case |
| Step 4 (M) | The network slice provisioning management service provider receives the information of the allocated NSSI(s) (e.g. the management identifier of NSSI, service access point information of NSSI, external connection point information of NSSI) from NSSMF. |  |
| Step 5 (M) | The network slice provisioning management service provider, via the network slice subnet provisioning management service provider, sends the transport network related requirements (e.g. external connection point, latency and bandwidth) to the TN Manager. The TN manager reconfigures the TN accordingly and responds to the network slice provisioning management service provider via the network slice subnet provisioning management service provider. |  |
| Step 6 (M) | The network slice provisioning management service provider receives the response from TN Manager via the network slice subnet provisioning management service provider. |  |
| Step 7 (M) | The network slice provisioning management service provider associates the NSSI(s) with the corresponding NSI (e.g. allocation of the management identifier of NSI and mapping the management identifier of NSI with the received management Identifier of NSSI(s)) and triggers to establish the links between the service access points of the NSSI(s). |  |
| Step 8 (M) | The network slice provisioning management service provider notifies the network slice instance information of NSI (e.g., the management identifier of NSI). |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | An NSI is ready to satisfy the network slice related requirements. |  |
| Traceability | REQ-PRO_NSSI-FUN-1, REQ-PRO_NSI-FUN-3. |  |

Next change

### 5.1.3	Network slice instance deallocation

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To terminate or modify an existing NSI which was used to support a particular service, but is no longer needed. |  |
| Actors and Roles | A network slice provisioning management service consumer. |  |
| Telecom resources | Network slice instance Network slice subnet instances A network slice provisioning management service provider. A network slice subnet provisioning management service provider. |  |
| Assumptions | N/A |  |
| Pre-conditions | N/A |  |
| Begins when | The network slice provisioning management service provider receives the request indicating that an existing NSI is no longer needed to support particular service. The NSI identification is included in the request. |  |
| Step 1 (M) | Based on the request, the network slice provisioning management service provider checks if there are no other services to be supported by the NSI. If there are none the network slice provisioning management service provider may decide to terminate the NSI; then proceed to Step 2. Otherwise, the network slice provisioning management service provider may decide to trigger to modify the NSI or to do nothing. The use case is completed; skip the remaining steps. | NSI modification use case |
| Step 2 (M) | If the NSI to be terminated is active, the network slice provisioning management service provider de-activates the NSI. Then, the NSI to be terminated is inactive. | NSI de-activation use case |
| Step 3 (M) | The network slice provisioning management service provider identifies the network slice subnet instances used by the NSI, and for every such NSSI sends the request to the corresponding network slice subnet provisioning management service provider (s) indicating that the NSSI(s) are no longer needed for the NSI. The network slice subnet provisioning management service provider (s) may decide to terminate or modify the NSSI(s) based on the request and disassociates them with the NSI. |  |
| Step 4 (M) | The network slice provisioning management service provider receives the response from the network slice subnet provisioning management service provider (s) and terminates the NSI. |  |
| Step 5 (M) | The network slice provisioning management service provider notifies its consumer of the NSI termination. |  |
| Ends when | All the steps identified above are successfully completed or skipped per condition in the Step 1. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The NSI has been or modified. |  |
| Traceability | REQ-PRO_NSI-FUN-3 |  |

Next change

### 5.1.7	Network slice instance activation

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To activate an existing network slice instance which is inactive |  |
| Actors and Roles | Network slice provisioning management service consumer. For example, CSP providing NSaaS plays the role of Network slice management service consumer. |  |
| Telecom resources | Network slice instance Network slice provisioning management service provider. Network slice subnet provisioning management service provider. |  |
| Assumptions | N/A |  |
| Pre-conditions | An NSI has already been created and it is inactive. |  |
| Begins when | The network slice provisioning management service provider decides to activate an NSI based on the received network slice related request from its authorized consumer. |  |
| Step 1 (M) | The network slice provisioning management service provider checks whether NSSIs associated with the NSI are all active, if there is an inactive NSSI, the network slice provisioning management service provider requests the network slice subnet provisioning management service provider to activate the corresponding NSSI. | Network slice subnet instance activation use case |
| Step 2 (M) | The network slice provisioning management service provider receives response from the network slice subnet provisioning management service provider indicating that the NSSI is active. |  |
| Step 3 (M) | The network slice provisioning management service provider activates the NSI and sends response to the requesting consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | An NSI has been activated. |  |
| Traceability | REQ-PRO_NSI–FUN-4 |  |

Next change

### 5.1.12	Network slice subnet instance modification

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To modify an existing network slice subnet instance |  |
| Actors and Roles | Network slice subnet provisioning management service consumer. |  |
| Telecom resources | Network slice subnet instance Network slice subnet provisioning management service provider. |  |
| Assumptions | N/A |  |
| Pre-conditions | N/A |  |
| Begins when | The network slice subnet provisioning management service provider receives request from its authorized consumer with new sets of network slice subnet related requirements and decides to modify an existing NSSI. |  |
| Step 1 (M) | The network slice subnet provisioning management service provider identifies the NSSI constituents as well as the transport network (TN) part within the NSSI that needs to be modified, and generates new sets of requirements for the NSSI constituents and transport network if needed. |  |
| Step 2 (M) | The network slice subnet provisioning management service provider checks whether the requirements for the identified NSSI constituents managed by itself could be satisfied, and then triggers the modification of the corresponding NSSI constituents if needed. |  |
| Step 3 (M) | If the NSSI consists of constituent NSSI managed by other network slice subnet provisioning management service provider, and the constituent NSSI is identified to be modified, the network slice subnet provisioning management service provider sends modification request to other network slice subnet provisioning management service provider which manages the constituent NSSI with new sets of constituent NSSI requirements. | Network slice subnet instance modification use case |
| Step 4 (M) | If the NS instance associated with the NSSI needs to be modified, the network slice subnet provisioning management service provider derives the new sets of NS related requirements and triggers corresponding NS instance request to NFVO with Os-Ma-nfvo interface as described in clause 6.4.3 in TS 28.525 [2]. | TS 28.525 [2] Clause 6.4.3 NS instance use cases |
| Step 5 (M) | If the related TN part of the NSSI is identified to be modified, the network slice subnet provisioning management service provider derives new sets of requirements for the TN part and coordinates with the corresponding TN management system. |  |
| Step 6 (M) | The network slice subnet provisioning management service provider generates the modification result based on the received response and send response to its authorized consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The NSSI is modified. |  |
| Traceability | REQ-PRO_NSSI-FUN-11 |  |

### 5.1.13	Network slice subnet configuration

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To provide service for slice-specific (re)configuration of NSSI. |  |
| Actors and Roles | Network slice subnet provisioning management service consumer (e.g., the operator) |  |
| Telecom resources | Network slice subnet provisioning management service provider Network slice subnet instance NF(s) |  |
| Assumptions | Authorized network slice subnet provisioning management service consumer provide slice operation information (see 4.2) for (re-)configuring NSSI constituents. |  |
| Pre-conditions | NSSI exists. |  |
| Begins when | The network slice subnet provisioning management service consumer wants to (re-)configure the constituents of a NSSI. |  |
| Step 1 (M) | The network slice subnet provisioning management service consumer sends requests to the network slice subnet provisioning management service provider with slice operation information for (re-)configuring a network slice subnet. |  |
| Step 2 (M) | The network slice subnet provisioning management service provider (derives and) decomposes the received slice operation information, and then makes them as separate CM requests for each constituent if necessary and applicable. These (decomposed) requests may be delegated to other provisioning management service providers (e.g., other network slice subnet provisioning service providers, NF provisioning management service providers) with corresponding slice operation information. These requests may contain configuration for specific NFs such as 1) Configuration of dedicated NFs (e.g., configure the SMF with the information of new instantiated UPFs, see 5.3.2, 5.3.3 in [6]) and 2) Configuration of shared NFs (see 4.2 so that this information can be accessed by other constituents of the NSS (e.g., NSSF, AMF, SMF). |  |
| Step 3 (M) | The network slice subnet provisioning management service provider sends the processing result to the network slice subnet provisioning management service consumer (might be based on applicable processing results from other CM service providers). |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The required (re)configuration is configured at the corresponding constituent(s). |  |
| Traceability | REQ-PRO_NSSI-FUN-16 |  |

### 5.1.14	Exposure of network slice management data

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | Enable network slice management service consumer to obtain network slice management data (e.g. PM data, FM data). |  |
| Actors and Roles | Network slice management service consumer. Network slice management service provider. |  |
| Telecom resources | Network slice instance |  |
| Assumptions | The network slice management service consumer is authorized to obtain the network slice management data from the network slice management service provider. |  |
| Pre-conditions | NSI is created. |  |
| Begins when | The network slice management service consumer wants to obtain the network slice management data. |  |
| Step 1 (M) | The network slice management service consumer sends a request to the network slice management service provider to obtain the network slice management data. |  |
| Step 2 (M) | The network slice management service provider provides the network slice management service consumer with the network slice management data . |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The network slice management service consumer obtained the network slice management data. |  |
| Traceability | REQ-PRO_NSI-FUN-7 |  |

Next change

### 5.1.17	Creation of a 3GPP NF

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to request creation of an instance of 3GPP NF. |  |
| Actors and Roles | An authorized consumer of the NF creation provisioning management service. |  |
| Telecom resources | VNF package(s) of the virtualized part of 3GPP NF; ETSI NFV MANO system; NF provisioning management service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The VNF package(s) of the virtualized part of 3GPP NF have been on-boarded to ETSI NFV MANO system. |  |
| Begins when | The authorized consumer needs to create a new instance of 3GPP NF. |  |
| Step 1 (M) | The authorized consumer requests the NF provisioning management service producer to create a new instance of 3GPP NF. |  |
| Step 2 (M) | The NF provisioning management service producer checks the subject 3GPP NF contains virtualized part and/or non-virtualized part. If it contains virtualized part, then the NF instantiation service producer performs the step 3 and 4 to instantiate the virtualized part of the subject 3GPP NF. How to instantiate the non-virtualized part of the subject 3GPP NF is out of scope of present specification. |  |
| Step 3 (M) | The NF provisioning management service producer interacts, or requests another NF provisioning management service producer to interact, with ETSI NFV MANO system to instantiate the VNF(s) that are realizing the virtualized part of subject 3GPP NF. |  |
| Step 4 (M) | If all of the contained parts (i.e., virtualized part and non-virtualized part if any) of the 3GPP NF have been successfully instantiated, the NF provisioning management service producer informs the consumer(s) (who have subscribed to the notifications for NF creation) that the instance of 3GPP NF has been created, and creates the MOI(s) for the subject 3GPP NF. |  |
| Step 5 (M) | Created MOI(s) may be maintained by a Management Function which has the NF provisioning management service or the 3GPP NF. When the MOI(s) is maintained by the 3GPP NF, the NF provisioning management service producer sends a request of creating the MOI(s) to the corresponding NF provisioning management service producers in the created NF. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The instance of 3GPP NF has been created, and the MOI(s) of the 3GPP NF have been created. |  |
| Traceability | REQ-PRO_NF-FUN-1, REQ-PRO_NF-FUN-2, REQ-PRO_NF-FUN-7 |  |

Next change

### 5.1.19	Creation of a 3GPP sub-network

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to request creation of a 3GPP sub-network. |  |
| Actors and Roles | An authorized consumer of the sub-network provisioning management service. |  |
| Telecom resources | VNF package(s) of the virtualized part of 3GPP NF(s); NSD(s) of the NS(s); ETSI NFV MANO system; Network provisioning service producer; NF provisioning service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The non-virtualized part of the NFs (including completely non-virtualized NFs) constituting the 3GPP sub-network have been deployed; The VNF package(s) of the virtualized part of 3GPP NF(s) have been on-boarded to ETSI NFV MANO system; The NSD(s) of the NS realizing the 3GPP sub-network have been on-boarded to ETSI NFV MANO system. |  |
| Begins when | The authorized consumer needs to create a 3GPP sub-network. |  |
| Step 1 (M) | The authorized consumer requests the sub-network provisioning management service producer to create a 3GPP sub-network. The request needs to indicate the network capacity (e.g., the number of instances of each kind of NFs, and the capacity of each NF instance, for example, number of flows with certain QoS attributes to be supported), network topology information (e.g., the connections between NF instances), and the network QoS requirements (e.g., bandwidth and latency requirements of the interface between two NF instances). |  |
| Step 2 (M) | The network provisioning management service producer interacts, or requests another network provisioning management service producer to interact, with ETSI NFV MANO system to instantiate the NS(s) realizing the sub-network. |  |
| Step 3 (M) | ETSI NFV MANO system informs the NF provisioning management service producer about the instantiation of the NSs and the new VNFs. |  |
| Step 4 (M) | The NF provisioning management service producer creates the MOI(s) of the 3GPP NFs that are realized by the newly instantiated VNF(s); there may be MOI(s) that specify the topology of the instantiated NSs. |  |
| Step 5 (M) | The sub-network provisioning management service producer is using the NF provisioning management service to configure the 3GPP NF instance(s) that are constituting the subject 3GPP sub-network. | NF configuration service |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The 3GPP sub-network has been created. |  |
| Traceability | REQ-PRO_NW-FUN-1, REQ-PRO_NW-FUN-2 |  |

### 5.1.20	Configuration of a 3GPP sub-network

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to request configuration of a 3GPP sub-network. |  |
| Actors and Roles | An authorized consumer of the network provisioning management service. |  |
| Telecom resources | 3GPP network; 3GPP NFs; ETSI NFG MANO system; Network provisioning management service producer. |  |
| Assumptions | N/A |  |
| Pre-conditions | The 3GPP sub-network has been created; The MOI(s) related to the sub-network has been created. |  |
| Begins when | The authorized consumer needs to configure a 3GPP sub-network. |  |
| Step 1 (M) | The authorized consumer requests to configure a 3GPP sub-network. |  |
| Step 2 (M) | The consumer requests the network provisioning management management service producer to modify the attribute of the MOI(s) related to the 3GPP sub-network. |  |
| Step 3 (O) | If the 3GPP network is realized by NS(s) (ETSI ISG NFV concept), the network provisioning management service producer requests (directly or indirectly via another) ETSI NFV MANO system to update the NS(s) realizing the 3GPP sub-network. |  |
| Step 4 (O) | If there are new VNFs instantiated by the NS update, ETSI NFV MANO system informs the NF provisioning management service producer about the instantiation of VNFs. |  |
| Step 5 (O) | The NF provisioning management service producer creates the MOI(s) of the 3GPP NFs that are realized by the newly instantiated VNF(s). |  |
| Step 6 (M) | The network provisioning management service producer consumes the NF provisioning management service to configure the impacted 3GPP NF instance(s). | NF configuration service |
| Step 7 (M) | The network provisioning management service producer configures the 3GPP sub-network, per the MOI attribute modification request received from the consumer. |  |
| Step 8 (M) | The NF provisioning management service producer modifies the attributes of the MOI(s) of the 3GPP network and informs the consumer that the 3GPP sub-network has been configured successfully. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The 3GPP network has been configured. |  |
| Traceability | REQ-PRO_NW-FUN-3, REQ-PRO_NW-FUN-4 |  |

Next change

### 5.1.22	Network slice resource capacity planning

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To calculate capacity of network slice instances and network slice subnet instances. |  |
| Actors and Roles | Network slice management service consumer. |  |
| Telecom resources | Network slice instance Network slice subnet instance Network slice management service provider. |  |
| Assumptions | The network slice management service consumer has decided to perform network slice resource capacity optimization process. |  |
| Pre-conditions | Network slice resource capacity optimization objectives are set by the network slice management service consumer. |  |
| Begins when | The network slice management service consumer requests resource capacity planning of the NSIs and/or NSSIs when the pre-set resource optimization objectives need to be satisfied. |  |
| Step 1 (M) | The network slice management service provider obtains information needed for the optimization process such as slice provisioning requirements, existing active or non-active NSI and/or NSSI resource information, and performance measurement data by requesting feasibility check operation. |  |
| Step 2 (M) | The network slice management service provider performs resource optimization process based on the information obtained in Step 1. The goal of the process is to find an optimal capacity availability against the target objective |  |
| Step 3 (M) | The network slice management service provider proceeds with network slice (NSI and/or NSSI) provisioning or modification processes until it meets the resource capacity optimization objective. | Network slice instance creation or modification/network slice subnet instance creation or modification use cases |
| Step 4 (M) | The network slice management service provider updates capacity availability information after provisioning or modification processes. |  |
| Ends when | The capacity resource planning ends when it meets the optimization objective. |  |
| Exceptions | One of the mandatory steps fails. |  |
| Post-conditions | Capacity planning policy for either provisioning or modification is generated. |  |
| Traceability | REQ-PRO_NSSI-FUN-3, REQ-PRO_NSI-FUN-9 |  |

### 5.1.23	Network slice subnet management with assigned priority

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To assign priority on existing network slice subnet instance(s). |  |
| Actors and Roles | Network slice subnet provisioning management service consumer. Network slice subnet provisioning management service provider |  |
| Telecom resources | Network slice subnet instance (i.e. NSSI) Network slice subnet provisioning management service provider |  |
| Assumptions | Network slice subnet instance is deployed to support a communication service with priority, set by the operator. |  |
| Pre-conditions | This use case is based on the condition that operator requires a priority assigned to network slice subnet. |  |
| Begins when | The NSSI(s) should have been assigned with priority set by the operator. |  |
| Step 1 (M) | The network slice provisioning management service provider identifies the NSSI(s) that needs to be associated with the priority, requested by authorized network slice subnet provisioning management service consumer. The network slice provisioning management service provider assigns priority to the identified NSSI(s) | Network slice subnet instance modification use case |
| Step 2 (M) | The network slice provisioning management service provider sends response to its authorized consumer about assigned priority on identified NSSI(s). |  |
| Ends when | All the steps identified above are successfully completed. Network slice subnet priority was assigned by the network slice subnet management service provider. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions |  |  |
| Traceability | REQ-PRO_NSI-FUN-x |  |

End of changes