3GPP TSG-SA5 Meeting #139-e 	S5-214711rev2

- e-meeting, 11 - 20 October 2021

- Source:	Huawei

- Title:	pCR Add ECS charging information aggregation and technical solution

- Document for:	Approval

- Agenda Item:	7.5.1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.815 “Study on charging aspects of edge computing”

# 3	Rationale

Resubmission of S5-214711.

The S5-214711 was approved in SA5 #138e meeting, while it is not captured in the current TR 28.815 v1.2.0.

# 4	Detailed proposal

1st Modified Section

#### 7.4.4.X	Potential charging architectures

The possible solution to support the key issue #4c are based on the following potential converged charging architecture.

Figure 7.4.4.X-1: Converged charging architecture with ECS supporting CEF

In this architecture, the CEF could be integrated with the ECS as depicted in Figure 7.4.4.X-1, the ECS:

-	subscribes notification from the EES to receive required information for charging.

-	aggregate the information for charging received from the EES.

-	generates charging event when the trigger conditions are met.

Alternatively, the CEF could be deployed as standalone functionality and the CEF:

-	subscribes notification from the EES to receive required information for charging.

-	aggregate the information for charging received from the EES.

-	generates charging event when the trigger conditions are met.

#### 7.4.4.Y	Possible solutions for aggregated charging

The possible solutions to support key issue #4c is illustrated in the Figure 7.4.4.Y-1. In this solution, the CEF is integrated with the ECS.

Figure 7.4.4.Y-1: Aggregated charging for edge enabling services

- The ECS subscribes notification from the EES to receive required information for charging. The subscription request may include EAS ID and a reporting rule of how a certain measurement shall be reported (e.g., when detecting a service/event had been invoked by the EAS for N times).

- The EAS invokes edge enabling service as defined in clause 7.4.1.

- The EES sends notification to the ECS when the reporting rule is met.

- The ECS aggregates the information for charging (e.g. amount of event) reported by one or multiple EES.

5.ch-a) Charging Data Request [Event]: The ECS generates charging information related to the collected information and sends the charging data request to the CHF when the triggers condition is met.

5.ch-b) Create CDR: The CHF stores received charging information, creates a CDR related to the event.

5.ch-c) Charging Data Response [Event]:  The CHF sends the response to the ECS on the result of the request.

End of Modified Sections