3GPP TSG SA WG5 Meeting 139-e		S5-215370rev1

- E-meeting, 11th - 20th Oct. 2021

- Source:	AsiaInfo, Alibaba Group

- Title:	Use case for Publishing rule for management capability exposure

- Document for:	Approval

Agenda Item:	6.5.4

# 1	Decision/action requested

The group is asked to discuss and approve the proposal..

# 2	References

[1]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

# 3	Rationale

The operator allows the vertical to use the provided network slice instance and optionally manage it. In some cases (e.g. RAN sharing scenario), a network slice can offer connection to a number of endpoints with a set of shared or dedicated network resources. However, the Operator may not want to expose certain MnS regarding certain network resources (e.g. RAN related resource) to externals. Therefore, an intrinsic rule for exposure may be needed for this purpose.

# 4	Detailed proposal

Start of 1st Change

## 5.X Use case for Publishing rule for management capability exposure

### 5.X.1	Description

A use case for publishing rule for management capability exposure can be described as follows:

1. In a RAN sharing scenario, MNO A owns the RAN resource, which is shared with an MNO B. The MNO A and MNO B have their own 5GC which uses the shared RAN for wireless access.

2. Due to the RAN sharing situation, MNO A and MNO B have an agreement that the management service for RAN is not allowed to be exposed to any other 3rd party.

3. An external customer subscribes to the network slice service from MNO A. The external customer wishes to sign a contract with MNO A for obtaining certain MnSs (e.g. KPI information of its application).

4. Due to the agreement (i.e. intrinsic rule for exposure) between MNO A and B, the management system of MNO A only provides MnS discovery service without the MnS data of RAN.

5. External customer identifies the available product via the BSS (e.g. Service Catalog API) without any MnS information regarding RAN provided in the product.

### 5.X.2 Issue and gaps

Gaps:

The definition and the format of intrinsic rule for network management capability exposure is not specified in the existing 3GPP management system.

End of Change