- 3GPP TSG-SA5 Meeting #139e	S5-215377rev2

- 11 Oct to 20 Oct 2021, E-meeting

- Source:	China Telecom, AsiaInfo

- Title:	Discussion paper on necessary inputs and outputs for MDA assisted energy saving analysis

- Document for:	Endorsement

- Agenda Item:	6.4.18

# 1	Decision/action requested

The group is asked to discuss and endorse on the proposal.

# 2	References

[1] 	3GPP TS 28.104-000 “Management and orchestration; Management Data Analytics”.

[2] 	3GPP TR 28.809 “Study on enhancement of Management Data Analytics (MDA)”.

[a]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[b]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[c]	3GPP TS 28.406: "Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport".

[d]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[e]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[f]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[g]	3GPP TS 28.532: "Management and orchestration; Generic management services".

# 3	Rationale

In SA5#138e, we submited the S5-214168 on adding the MDA capability for MDA assisted energy saving analysis. But it was noted since the structure of TS 28.104 [1] was not approved. This discussion paper is to discuss the analytics inputs and outputs for energy saving issue analysis.

3.2			Discussion on the analytics inputs and outputs for energy saving issue analysis

The MDA assisted energy saving analysis has been studied in the TR 28.809 [2]. The data category in the table X.1-1 and the information elements in the table X.2.1 have been discussed in the TR 28.809.

##### X.1   Analytics inputs

Table X.1-1: Analytics inputs for energy saving issue analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance Measurements | PNF Power Consumption: Power consumed over the measurement period; | 5.1.1.19.2 of TS 28.552 [a] |
|  | PNF Energy consumption: The energy consumed; | 5.1.1.19.3 of TS 28.552 [a] |
|  | PNF Temperature: The temperature over the measurement period; | 5.1.1.19.4 of TS 28.552 [a] |
|  | PNF Voltage: The voltage; | 5.1.1.19.5 of TS 28.552 [a] |
|  | PNF Current: The current; | 5.1.1.19.6 of TS 28.552 [a] |
|  | PNF humidity: The percentage of humidity during the measurement period; | 5.1.1.19.7 of TS 28.552 [a] |
|  | PDCP Data Volume of NR cells: The transmitted PDCP data volume; | 5.1.2.1 and 5.1.3.6 of TS 28.552 [a] |
|  | Virtual resource usage of NF: The resource usage of virtual network functions; | 5.7.1 of TS 28.552 [a] |
|  | Data volume of UPF; | 5.4 of TS 28.552 [a] |
|  | Delay related measurements of UPF; | 5.4 of TS 28.552 [a] |
|  | Number of PDU sessions with SSC mode 1 on UPF. | TS 28.552 [a] |
|  | Traffic load variation: PRB utilization rate, RRC connection number, etc | 5.1.1.2 and 5.1.1.4 of TS 28.552 [a] |
|  | Network data: UE throughput, the mobility management data (e.g., rate of successful handover executions), packet delay data (e.g., air-interface delay), etc. | 5.1.1.1, 5.1.1.3 and 5.1.1.6 of TS 28.552 [a] |
| QoE Data | The measurements that are collected are DASH and MTSI measurements. | TS 28.406 [c], TS 26.247 [d], TS 26.114 [e] |
| NRM | MOIs of the cells, UPFs and SMFs. | TS 28.541 [f] |
| Network analytics data | The control plane analysis result from the NWDAF, e.g., observed service experience related network data analytics. | TS 23.288 [b] |
| ML model information | The information of the ML model, which may include the ML model file address. It can be used by MDA to derive predictions of the energy saving related information (e.g., base station energy-saving scenarios, traffic load). | Editor’s note: to be defined in MDA capability specific procedures in TS 28.104 |

##### X.2   Analytics outputs

Table X.2-1: Analytics output for energy saving issue analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| Location | The list of NR cells or the NF instances that have the potential to execute energy-saving commands through analysis. | M | FFS |
| Root cause | The root cause of the part of the energy consumption that may be conserved. The allowed value is one of the enumerated values: Ultra-lowTrafficLoadAreaEnergyConsumption, Other, Unknown. | O | FFS |
| Trends of traffic load | Predictions of traffic load and network performance trend (PRB utilization rate, RRC connection success rate, etc.). | M | FFS |
| Current Energy Saving State of the cells | The information of the current energy saving state of the cells. The allowed value is one of the enumerated values: notEnergySavingState, EnergySavingState. | O | FFS |
| ML model information | The information of the optimal ML model for energy saving related information prediction. The ML model information can be used by MDAS consumer to retrieve the ML model. The ML model information can be ML model file address. | O | FFS |
| Energy saving recommendations | For ES on NR cells. It may contain a set of - recommended NR Cell (ES-Cell) to enter energySaving state. - recommended candidate cells with precedence for taking over the traffic of the ES-Cell. - a time period, during which energy saving is or not allowed. For ES on UPFs. It contains a set of - recommended UPF (ES-UPF) to conduct energy saving; - recommended candidate UPFs with precedence for taking over the traffic of the ES-UPF. - a time period, during which energy saving is or not allowed. | O | FFS |

# 4	Detailed proposal

It is proposed to endorse the following as the way forwards in the discussion on the data definition for MDA assistd energy saving in the TS 28.104:

- the information elements of the analytics output for MDA assisted energy saving described in TR 28.809 (e.g., Location, RootCause, Trends of the traffic load, Current Energy Saving State of the cells, ML model information and Energy saving recommendations) are necessary for the MDA assisted energy saving analysis, which may be included in the table of analytics output for MDA assisted energy saving in the TS 28.104 based on the discussion result of the structure of TS 28.104. How to define and format these information elements may be discussed later.