3GPP TSG-SA5 Meeting #139-e 	S5-215411rev2

- e-meeting, 11 - 20 October 2021

- Source:	Lenovo, Motorola Mobility

- Title:	Solution for external discovery

- Document for:	Approval, Information, Discussion

Agenda Item:	6.5.4

# 1	Decision/action requested

Please approve

# 2	References

# 3	Rationale

# The text below proposes a high-level solution to registering an exposed MnS to an external discovery system.

# 4	Detailed proposal

Start of changes

# 7	Possible solutions for network management capability exposure

7.x Possible solution for “exposed MnS support to discovery systems”

The steps of the solution are as follows:

- A MnS consumer configures using the appropriate MnS (for example the generic provisioning service) the details of the external discovery service location and other supporting details (for e.g. authentication). Further, the detail on which parts of which MnS (component A)  IOCs and which instances of the corresponding MOI (component B) and corresponding data (component C) can be registered with the discovery service/system.  As an example, the operator may want to register her the ability to provision (Management object A) a particular slice type (NSSAI-ID) at a certain coverage area (coverageArea item 6.3.3 TS28.541) with some additional details (example: supported latency or maxNumberofUEs or delayTolerance) to an external discovery service or system.

In addition to the information of what is externally registered, information relating to where the appropriate management service to access the exposed MnS also needs to be provided. This could be default information based on the authentication of the operator in the exernal discovery system or in case of a trusted discovery the address of where to access the management service.

- The 3GPP Management System registers the exposed management component A, B and C as configured in step 1 to the appropriate external discovery service/system.

- Eventually, if any of the exposure details change – for example the same slice type can now be supported in a new coverageArea – the registration to the external discovery system may now need to be updated.

End of changes