3GPP TSG-SA5 Meeting #139-e	S5-215332

# e-meeting, 11-20 October 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0324 | rev | - | Current version: | 17.4.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of average value of scheduled MIMO layers per PRB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, ZTE, Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ePM_KPI_5G |  |  |  |  |  | Date: |  |  | 2021-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - With MIMO technology, NR cell can allocate one PRB to several different users at the same time, namely several layers can multiplex one PRB to transit traffics simultaneously. Thus, with the same radio resources (i.e. the same number of PRBs), MIMO can increase the cell throughput (i.e. capacity) obviously. - The capacity of cell with MIMO depends on the Number of PRBs and the Number of Layers. TS28.552 has defined the former, but not the latter. - This CR proposed an indicator " Average value of scheduled MIMO layers per PRB" to measure the average Layers (capacity improvement) that is achieved, and to support the analyse of cell capacity for MIMO scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new indicator " Average value of scheduled MIMO layers per PRB " to accurately measure the NR cell capacity for MIMO scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of indicator to measure the cell capacity improvement brought by MIMO technology. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.30.x(new), 5.1.1.30.y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

5.1.1.30.x	Average value of scheduled MIMO layers per PRB on the DL

a)	This measurement provides the average value of allocated MIMO layers on the downlink per PRB per cell, for MIMO scenario within the measurement period.

b)	SI.

c)	This measurement is obtained by computing the average value of scheduled MIMO layers among all used PRBs that are used within the measurement period in the cell. The average value is obtained by this formula:

LaveDL=∀j∀kM1kjT*LkjT∀j∀kM1kjT,

where LaveDL denotes the average value of scheduled MIMO layers per PRB per cell on the DL. T denotes the measurement period (e.g. 1 hour). And j denotes the sampling occasion j (e.g. 1 symbol). And k denotes the number of kinds of MIMO layers (e.g. 2 kinds). Lkj(T) denotes the number of MIMO layers (e.g. 1 layers, 4layers, etc.) scheduled for traffic transmission  at sampling occasion j. M1kj(T) denotes the number of PDSCH PRBs used for transmission corresponding to  Lkj(T), at sampling occasion j. For example, a cell has 10 PRBs in total for one sampling occasion (j=1), within which 9 PRBs are used and 1 left spare. Among 9 used PRBs, one is multiplexed by 4 layers, three is multiplexed by 2 layers, and five only has 1 layer (no multiplexing). So the LaveDL in this case is: (1*4+3*2+5*1)/(1+3+5) = 1.67 layers per PRB.

d)	Each measurement is a real value.

e) The measurement name has the form CARR.AverageLayersDl

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i）One usage of this measurement is to monitor the cell capacity for MIMO scenario, on the DL.

Second Change

5.1.1.30. y	Average value of scheduled MIMO layers per PRB on the UL

a)	This measurement provides the average value of allocated MIMO layers on the uplink per PRB per cell, for MIMO scenario within the measurement period.

b)	SI.

c)	This measurement is obtained by computing the average value of scheduled MIMO layers among all used PRBs that are used within the measurement period in the cell. The average value is obtained by this formula:

LaveUL=∀j∀kM1kjT*LkjT∀j∀kM1kjT,

where LaveUL denotes the average value of scheduled MIMO layers per PRB per cell on the UL. T denotes the measurement period (e.g. 1 hour). And j denotes the sampling occasion j (e.g. 1 symbol). And k denotes the number of kinds of MIMO layers (e.g. 2 kinds). Lkj(T) denotes the number of MIMO layers (e.g. 1 layers, 4layers, etc.) scheduled for traffic transmission  at sampling occasion j. M1kj(T) denotes the number of PUSCH PRBs used for transmission corresponding to  Lkj(T), at sampling occasion j. For example, a cell has 10 PRBs in total for one sampling occasion (j=1), within which 9 PRBs are used and 1 left spare. Among 9 used PRBs, one is multiplexed by 4 layers, three is multiplexed by 2 layers, and five only has 1 layer (no multiplexing). So the LaveUL in this case is: (1*4+3*2+5*1)/(1+3+5) = 1.67 layers per PRB.

d)	Each measurement is a real value.

e) The measurement name has the form CARR.AverageLayersUl

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i）One usage of this measurement is to monitor the cell capacity for MIMO scenario, on the UL.

Third Change

A.X	Monitoring of Average value of scheduled MIMO layers per PRB

The average value of scheduled MIMO layers per PRB should be monitored, as it reflects the capacity improvement brought by MIMO in the 5G business network, quantitatively. The operators can use this information to compare cell capacity among different areas and support capacity enhancement decision-making. Besides, the information can be used to optimize the MIMO equipment performance and other OAM works.

End of Change