3GPP TSG-SA5 Meeting #140-e 	S5-216363

# e-meeting, 15 - 24 November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | 0044 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add beam specific handover counters to MRO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSON_5G |  |  |  |  |  | Date: |  |  | 2021-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Add new PM to support analytics related to intra-system mobility. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce new PM related to handover failure per beam. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Use case for beam handover mobility cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.25, A.66 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 28.541 CR 0635, TS 28.552 CR 0340 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 6.4.1.2	MRO (Mobility Robustness Optimisation)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To automatically configure the handover parameters in cells or beams in order to improve the handover performance. |  |
| Actors and Roles | D-SON management function to support MRO function. |  |
| Telecom resources | - gNB; - The producer of provisioning MnS |  |
| Assumptions | N/A |  |
| Pre-conditions | - 5G NR cells are in operation. - MRO is in operation. |  |
| Begins when | The D-SON management decides to enable MRO function. |  |
| Step 1 (M) | The D-SON management function requests the producer of provisioning MnS to set the targets, HO offset ranges, and control information for the MRO function. |  |
| Step 2 (M) | The D-SON management function requests the producer of provisioning MnS to enable the MRO function. |  |
| Step 3 (M) | The MRO function detects handover issues (e.g. too late HO, too early HO and HO to a wrong cell) in intra-RAT or inter-RAT mobility by analysing reports from UEs and network side information, and acts to mitigate the HO issues by adjusting HO related parameters. |  |
| Step 4 (M) | The D-SON management function collects MRO related measurements, and analyses them to evaluate the MRO performance. |  |
| Step 5 (M) | The D-SON management function performs the following action, if the MRO performance does not meet the target: 1. Update the targets for MRO function. 2. Update the ranges for MRO function. 3. Update the control information for MRO function. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The MRO performance has been optimized. |  |
| Traceability | REQ-MRO-FUN-1, REQ- MR-FUN-2, REQ-MRO-FUN-3, REQ-MRO-FUN-4 |  |

Next change

### 7.1.2	MRO (Mobility Robustness Optimisation)

#### 7.1.2.1	MnS component type A

Table 7.1.2.1-1: MRO type A

| MnS Component Type A | Note |
| --- | --- |
| Operations and notifications defined in clause 11.1.1 of TS 28.532 [3]: - createMOI operation - getMOIAttributes operation - modifyMOIAttributes operation - deleteMOI operation - notifyMOIAttributeValueChanges operation - notifyMOICreation - notifyMOIDeletion - notifyMOIChanges | It is supported by Provisioning MnS for NF, as defined in TS 28.531 [11]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in TS 28.550 [12]. |

#### 7.1.2.2	MnS Component Type B definition

##### 7.1.2.2.1	Targets information

The targets of MRO are shown in the Table 7.1.2.2.1-1.

Table 7.1.2.2.1-1: MRO targets

| Target Name | Definition | Legal Values |
| --- | --- | --- |
| Total handover failure rate | (the number of failure events related to handover) / (the total number of handover events) | [0..100] in unit percentage |
| Total intra-RAT handover failure rate | (the number of failure events related to intra-RAT handover) / (the total number of handover events) | [0..100] in unit percentage |
| Total inter-RAT handover failure rate | (the number of failure events related to inter-RAT handover) / (the total number of handover events) | [0..100] in unit percentage |

##### 7.1.2.2.2	Control information

The parameter is used to control the MRO function.

Table 7.1.2.2.2-1: MRO control

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| MRO function control | This attribute allows the operator to enable/disable the MRO functionality. See attribute mroControl in TS 28.541 [13]. | Boolean On, off |

##### 7.1.2.2.3	Parameters to be updated

Table 7.1.2.2.3-1: Ranges of handover parameters

| Control parameters | Definition | Legal Values |
| --- | --- | --- |
| Maximum deviation of Handover Trigger | This parameter defines the maximum allowed absolute deviation of the Handover Trigger, from the default point of operation (see clause 15.5.2.5 in TS 38.300 [7] and clause 9.2.2.61 in TS 38.423 [17]). | [-20..20] in unit 0.5 dB |
| Minimum time between Handover Trigger changes | This parameter defines the minimum allowed time interval between two Handover Trigger change performed by MRO. This is used to control the stability and convergence of the algorithm (see clause 15.5.2.5 in TS 38.300 [7]). | [0.. 604800] in unit Seconds |
| Tstore_UE_cntxt | The timer used for detection of too early HO, too late HO and HO to wrong cell. Corresponds to Tstore_UE_cntxt timer described in clause 15.5.2.5 in TS 38.300 [7]. | [0..1023] in unit 100 milliseconds |

#### 7.1.2.3	MnS Component Type C definition

##### 7.1.2.3.1	Performance measurements

Performance measurements related MRO are captured in Table 7.1.2.3.1.-1:

Table 7.1.2.3.1-1. MRO related performance measurements

| Performance measurements | Description | Related targets |
| --- | --- | --- |
| Number of handover events | Includes all successful and unsuccessful handover events (see clause 5.1.1.6 in TS 28.552 [5]). | Total handover failure rate |
| Number of handover failures | Includes unsuccessful handover events with failure causes (see clause 5.1.1.6 in TS 28.552 [5]). | Total handover failure rate |
| Number of intra-RAT handover events | Includes all successful and unsuccessful intra-RAT handover events (see clauses 5.1.1.6.1 and 5.1.1.6.2 in TS 28.552 [5]). | Total intra-RAT handover failure rate |
| Number of intra-RAT handover failures | Includes unsuccessful intra-RAT handover events with failure causes (see clauses 5.1.1.6.1 and 5.1.1.6.2 in TS 28.552 [5]). | Total intra-RAT handover failure rate |
| Number of inter-RAT handover events | Includes all successful and unsuccessful inter-RAT handover events (see clause 5.1.1.6.3 in TS 28.552 [5]). | Total inter-RAT handover failure rate |
| Number of inter-RAT handover failures | Includes unsuccessful inter-RAT handover events with failure causes (see clause 5.1.1.6.3 in TS 28.552 [5]). | Total inter-RAT handover failure rate |
| Number of intra-RAT too early handover failures | Detected when an RLF occurs after the UE has stayed for a long period of time in the cell (see clause 5.1.1.25.1 in TS 28.552 [5]). |  |
| Number of intra-RAT too late handover failures | Detected when an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure (see clause 5.1.1.25.1 in TS 28.552 [5]). |  |
| Number of intra-RAT handover failures to wrong cell | Detected when an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure (see clause 5.1.1.25.1 in TS 28.552 [5]). |  |
| Number of intra-RAT too early handover failures per source beam | Detected when an RLF occurs after the UE has stayed for a long period of time in the cell (see clause 5.1.1.25.x in TS 28.552 [5]). |  |
| Number of intra-RAT too late handover failures per source beam | Detected when an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure (see clause 5.1.1.25.x in TS 28.552 [5]). |  |
| Number of intra-RAT handover failures to wrong cell per source beam | Detected when an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure (see clause 5.1.1.25.x in TS 28.552 [5]). |  |
| Number of inter-RAT too early handover failures | Detected when an RLF occurs after the UE has stayed in an E-UTRAN cell which connects with 5GC for a long period of time (see clause 5.1.1.25.2 in TS 28.552 [5]). |  |
| Number of inter-RAT too late handover failures | Detected when an RLF occurs shortly after a successful handover from an E-UTRAN cell which connects with EPC to a target cell in a E-UTRAN cell which connects with 5GC (see clause 5.1.1.25.2 in TS 28.552 [5]). |  |
| Number of unnecessary handover to another RAT | Detected when a UE is handed over from NG-RAN to other system (e.g. UTRAN) even though quality of the NG-RAN coverage was sufficient for the service used by the UE (see clause 5.1.1.25.3 in TS 28.552 [5]). |  |
| Number of inter-RAT handover ping pong | Detected when an UE is handed over from a cell in a source system (e.g. NG-RAN) to a cell in a target system different from the source system (e.g. E-UTRAN), then within a predefined limited time the UE is handed over back to a cell in the source system, while the coverage of the source system was sufficient for the service used by the UE (see clause 5.1.1.25.4 in TS 28.552 [5]). |  |

End of changes