3GPP TSG-SA5 Meeting #140-e 	S5-216083

- e-meeting, 15 - 24 November 2021

- Source:	Ericsson

- Title:	Adding SBI architecture to clause 7.2.4

- Document for:	Approval

Agenda Item:	7.5.7

# 1	Decision/action requested

Include the proposed changes in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: " Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

Adding SBI architecture to clause 7.2.4.

# 4	Detailed proposal

First change

### 7.2.4	Potential solutions

#### 7.2.4.1 	Solution #2b.1: V-CHF communicating with H-CHF for retail charging of 5G data connectivity

##### 7.2.4.1.1	General

A possible solution for retail charging for 5G data connectivity provided to the home MNO’s users by the visited MNO, in the case of local breakout.

##### 7.2.4.1.2	Reference architecture

Figure 7.2.4.1.2-1: Roaming 5G data connectivity scenario in service-based interface representation

The visited CHF does converged charging for interconnect, while the home CHF does converged charging for the subscriber.

Figure 7.2.4.1.2-2: Roaming 5G data connectivity scenario in reference point representation

##### 7.2.4.1.3	Message flows

Figure 7.2.4.1.3-1 shows a scenario for session based charging (SCUR) with CHF to CHF communication.

Figure 7.2.4.1.3-1: SCUR - Session based charging with CHF to CHF communication

1)	Request for service delivery: A request for session establishment is received in the NF (CTF). The service is configured to be authorized by the vCHF before start.

2)	Units determination: the NF (CTF) determines the number of units depending on the service requested by the UE in "Decentralized Units determination" scenario.

3)	Charging data request [Initial, Quota requested]: The NF (CTF) sends the request to the vCHF for the service to be granted authorization to start, and to reserve the number of units if determined in item 2.

4)	Determine hCHF: the vCHF determines that it need to interact with hCHF and which hCHF it should contact.

5)	Charging home request [Initial, Quota requested]: The vCHF sends the request to the hCHF for the service to be granted authorization to start, and to reserve the number of units if determined in item 2.

6)	Account, Rating, Reservation control: the hCHF rates the requests either based on the number of units requested or on internal unit determination, checks if corresponding funds can be reserved on the user's account balance. If the account has sufficient funds, the hCHF performs the corresponding reservations.

7)	Open CDR: based on policies, the hCHF opens a CDR related to the service.

8)	Charging home response [Initial, Quota granted]: The hCHF grants authorization to vCHF for the service to start, with the reserved number of units.

9)	Open CDR: based on policies, the vCHF opens a CDR related to the service.

10)	Charging data response [Initial, Quota granted]: The vCHF grants authorization to NF (CTF) for the service to start, with the reserved number of units.

11)	Granted units supervision: the NF (CTF) monitors the consumption of the granted units.

12)	Start of service delivery: the NF (CTF) starts to deliver the content/service based on the reserved number of units.

13)	Quota management trigger: A trigger associated to quota management is met. Unit determination is performed when applicable.

14)	Charging data request [Update, Unit used, Quota requested]: the NF (CTF) sends the request to the vCHF, for more units to be granted for the service to continue and reporting the used units.

15)	Charging home request [Update, Unit used, Quota requested]: the vCHF sends the request to the hCHF, for more units to be granted for the service to continue and reporting the used units.

16)	Account, Rating, Reservation control: The hCHF performs the process related to the reported usage and the requested reservation, involving rating entity and user's account balance.

17)	 Update CDR: based on policies, the hCHF updates the CDR with charging data related to the service.

18)	Charging home response [Update, Quota granted]: The hCHF grants quota to vCHF for the service to continue, with the reserved number of units.

19)	 Update CDR: based on policies, the vCHF updates the CDR with charging data related to the service.

20)	Charging data response [Update, Quota granted]: The vCHF grants quota to NF (CTF) for the service to continue, with the reserved number of units.

21)	Granted units supervision: the NF (CTF) monitors the consumption of the granted units.

22)	Service delivery ongoing: the NF (CTF) continues the service delivery based on the granted quota.

23)	Service released: the service is terminated or released.

24)	Charging data request [Termination, Unit used]: the NF (CTF) sends the request to the vCHF, for charging data related to the service termination with the final consumed units.

25)	Charging home request [Termination, Unit used]: the vCHF sends the request to the hCHF, for charging data related to the service termination with the final consumed units.

26)	Account, Rating control: The hCHF performs the service termination process involving rating entity and user's account balance.

27)	 Close CDR: based on policies, the hCHF closes the CDR with charging data related to the service termination and the last reported units.

28)	Charging home response [Termination]: The hCHF informs the vCHF on the result of the request.

29)	 Close CDR: based on policies, the vCHF closes the CDR with charging data related to the service termination and the last reported units.

30)	Charging data response [Termination]: The vCHF informs the NF (CTF) on the result of the request.

#### 7.2.4.2 	Solution #2b.2: Visited NF (CTF) communicating with both H-CHF and V-CHF

##### 7.2.4.2.1	General

A possible solution for retail charging for 5G data connectivity provided to the home MNO’s users by the visited MNO. Roaming 5G data connectivity scenario in service-based interface representation is the same with Figure 7.2.4.1.1-1.

The visited CHF does converged charging for interconnect, while the home CHF does converged charging for the subscriber.

##### 7.2.4.2.2	Reference architecture

Figure 7.2.4.2.2-1: Roaming 5G data connectivity scenario in service-based interface representation

The visited CHF does converged charging for interconnect, while the home CHF does converged charging for the subscriber.

Figure 7.2.4.2.2-2: Roaming 5G data connectivity scenario in reference point representation

7.2.4.2.3	Message flows

Figure 7.2.4.2.3-1 shows a scenario for session based charging (SCUR) with CTF to both CHFs communication.

Figure 7.2.4.2.1-2: SCUR - Session based charging with CTF to two CHFs communication

1)	Request for service delivery: A request for session establishment is received in the NF (CTF). The service is configured to be authorized by both the vCHF and hCHF before start.

2)	Units determination: the NF (CTF) determines the number of units depending on the service requested by the UE in "Decentralized Units determination" scenario.

3)	Charging data request [Initial]: The NF (CTF) sends the request to the vCHF for the service to be granted authorization to start.

4)	Open CDR: based on policies, the vCHF opens a CDR related to the service.

5)	Charging data response [Initial]: The vCHF grants authorization to NF (CTF) for the service to start.

6)	Charging data request [Initial, Quota requested]: The NF (CTF) sends the request to the hCHF for the service to be granted authorization to start, and to reserve the number of units if determined in item 2.

7)	Account, Rating, Reservation control: the hCHF rates the requests either based on the number of units requested or on internal unit determination, checks if corresponding funds can be reserved on the user's account balance. If the account has sufficient funds, the hCHF performs the corresponding reservations.

8)	Open CDR: based on policies, the hCHF opens a CDR related to the service.

9)	Charging home response [Initial, Quota granted]: The hCHF grants authorization to NF (CTF) for the service to start, with the reserved number of units.

10)	Granted units supervision: the NF (CTF) monitors the consumption of the granted units.

11)	Start of service delivery: the NF (CTF) starts to deliver the content/service based on the reserved number of units.

12)	Quota management trigger: A trigger associated to quota management is met. Unit determination is performed when applicable.

13)	Charging data request [Update, Unit used]: the NF (CTF) sends the request to the vCHF, reporting the used units.

14)	 Update CDR: based on policies, the vCHF updates the CDR with charging data related to the service.

15)	Charging data response [Update]: The vCHF grants authorization to NF (CTF) for the service to continue.

16)	Charging data request [Update, Unit used, Quota requested]: the NF (CTF) sends the request to the hCHF, for more units to be granted for the service to continue and reporting the used units.

17)	Account, Rating, Reservation control: The hCHF performs the process related to the reported usage and the requested reservation, involving rating entity and user's account balance.

18)	 Update CDR: based on policies, the hCHF updates the CDR with charging data related to the service.

19)	Charging data response [Update, Quota granted]: The hCHF grants quota to NF (CTF) for the service to continue, with the reserved number of units.

20)	Granted units supervision: the NF (CTF) monitors the consumption of the granted units.

21)	Service delivery ongoing: the NF (CTF) continues the service delivery based on the granted quota.

22)	Service released: the service is terminated or released.

23)	Charging data request [Termination, Unit used]: the NF (CTF) sends the request to the vCHF, for charging data related to the service termination with the final consumed units.

24)	 Close CDR: based on policies, the vCHF closes the CDR with charging data related to the service termination and the last reported units.

25)	Charging home request [Termination, Unit used]: the NF (CTF) sends the request to the hCHF, for charging data related to the service termination with the final consumed units.

26)	Account, Rating control: The hCHF performs the service termination process involving rating entity and user's account balance.

27)	 Close CDR: based on policies, the hCHF closes the CDR with charging data related to the service termination and the last reported units.

28)	Charging home response [Termination]: The hCHF informs the NF (CTF) on the result of the request.

29)	 Close CDR: based on policies, the hCHF closes the CDR with charging data related to the service termination and the last reported units.

30)	Charging data response [Termination]: The hCHF informs the NF (CTF) on the result of the request.

End of changes