3GPP TSG-SA5 Meeting #140-e 	S5-216174

- e-meeting, 15 - 24 November 2021

- Source:	Huawei

- Title:	pCR 28.556 update PolicyContent definition

- Document for:	Approval

- Agenda Item:	6.4.11

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	TS 28.556 v0.4.0 Policy management for 5G mobile networks

# 3	Rationale

The PolicyContent type as described in the table of clause 5.2.4.2 is not defined. It is suggested to add the PolicyContent data type definition.

# 4	Detailed proposal

It is proposed to apply following changes in [1].

1st modified section

- 5.2.3	Class definitions

- 5.2.3.1	Policy

- 5.2.3.1.1	Definition

This IOC represents a network policy.

- 5.2.3.1.2	Attributes

The Policy IOC includes attributes inherited from Top IOC (defined in TS 28.622) and the following attributes:

| Attribute name | Support Qualifier | isReadble | isWriteble | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| policyPriority | M | T | T | F | T |
| policyStatus | M | T | T | F | T |
| policyType | M | T | T | F | T |
| policyContent | M | T | T | F | T |

- 5.2.3.3	Notifications

TBD.

5.2.3.4	PolicyContent <<dataType>>

- 5.2.3.4.1	Definition

This data type represents the content of a network policy.

- 5.2.3.4.2	Attributes

| Attribute name | Support Qualifier | isReadble | isWriteble | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| condition | M | T | T | F | T |
| action | M | T | T | F | T |

2nd modified section

- 5.2.4	Information attribute definitions

- 5.2.4.1	Introduction

This clause defines the semantics of the attributes used in IOCs.

- 5.2.4.2	Definitions and legal values

| Attribute Name | Definition | Legal Values |
| --- | --- | --- |
| policyPriority | It specifies the priority of Policy. allowedValues:LOW,Medium,High | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| policyStatus | It specifies the status of Policy. If a policy is activated, and then its status is active. If it is deactivated ,then its status is inactive allowedValues:ACTIVE,INACTIVE | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: NONE isNullable: False |
| policyType | It idIt specifies the type of Policy. allowedValues: N/A NOTE: Its value is not defined in this TS. | type: string multiplicity: 0..N isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| policyContent | It identifies the content of a network policy allowedValues: N/A | type: PolicyContent multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: False |
| condition | It identifies the condition of the policy. The type of condition depends on the concrete policy. allowedValues: N/A | type: None multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: False |
| action | It identifies the action of policy. The type of action depends on the concrete policy. allowedValues: N/A | type: None multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: False |