3GPP TSG-SA5 Meeting #140-e 	S5-216187rev1

- e-meeting, 15 - 24 November 2021

- Source:	Huawei

- Title:	pCR Add alarm incident analysis solution

- Document for:	Approval

- Agenda Item:	6.4.18

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	3GPP TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	3GPP TS 28.104-020 “Management and orchestration; Management Data Analytics”

# 3	Rationale

This contribution is proposed to add MnS component for fault management related issues analysis capability of MDAS in [2].

# 4	Detailed proposal

It is proposed to apply following changes in [2].

1st modified section

### X.X.1	Fault management related issues

#### X.X.1.X	Fault Analysis

##### X.X.1.X.1	MDA type

The MDA type for Fault analysis is: FaultManagement. FaultAnalysis.

##### X.X.1.X.2	Analytics output

The specific information elements of the analytics output (MDA report) for Fault analysis, in addition to the common information elements of the analytics outputs (see clause X.Y), are provided in table X.X.1.X.2-1.

Table X.X.1.X.2-1: Analytics output for Fault analysis

| Information | Description | Support qualifier | Properties |
| --- | --- | --- | --- |
| FaultID | The Fault id or name for correlated alarms and performance measurements, e.g., Xn transmission alarm incident. | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CorrelatedAlarmInfos | List of Alarm name or alarm ID. | O | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| FaultRaisedtime | Indicates the date and time when the Fault is first raised. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| FaultClearedtime | Indicates the date and time when the Faultis cleared. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| RootCause | Indicates the root cause of Fault. Editor's Note: the data type of RootCause is FFS. | O | type: FFS multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| SeverityLevel | It indicates the severity level of the Fault. Values: Critical, Major, Minor, Warning, Indeterminate, Cleared | O | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End modified section