3GPP TSG-SA5 Meeting #140-e 	S5-216193rev3

- e-meeting, 15 - 24 November 2021

- Source:	Huawei

- Title:	PCR TS 28.104 add MDA analysis request and reporting workflow

- Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	3GPP TS 28.104-020 “Management and orchestration; Management Data Analytics”

# 3	Rationale

This contribution propose to add generic MDA analysis request and reporting workflow in [1].

# 4	Detailed proposal

It is proposed to apply following changes in [1].

1st modified section

# X	Workflow for MDA management

## X.1	MDA analysis request and reporting workflow

Figure X.1-1: Generic MDA analysis request/reporting workflow

1	MDAS Producer receives a Create MDAReportControl MOI request (see createMOI operation defined in TS 28.532 [11]) from MDAS Consumer with MDA request control related information.

Editor’s Note1: the reference of information model definition will be added, when they are available.

Editor’s Note2: the MDA request control related information is FFS.

2	The MDAS Producer creates the MOI for the MDAReportControl to be created.

3	The MDAS Producer sends the Create MDAReportControl MOI response (see createMOI operation defined in TS 28.532 [11]) to MDAS Consumer with identifier of MOI.

4 	The MDAS Procedure perform MDA anlaysis and generate MDA report.

Alt1: For stream-based MDA reporting

5.1	Establish the streaming connection containing the stream information, by invoking the establishStreamingConnection operation;

5.2	The MDAS producer sends the MDA report to the MDAS consumer via data streams, by invoking the reportStreamData operation.

Alt2: For file-based MDA reporting

6.1~6.2 The MDAS consumer create a file-ready notification subscription operation by invoking the subscribe operation.

6.3~6.5 The MnS producer sends notifyFileReady notification to subscribed MnS consumers when a MDA report file becomes ready (available) on the MnS producer for upload by MnS consumers.

End of modified section