3GPP TSG-SA5 Meeting #140-e 	S5-216194rev4

- e-meeting, 15 - 24 November 2021

- Source:	Huawei

- Title:	Add MDA analysis report reporting related service component

- Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

This contribution is for approval.

# 2	References

[1]	3GPP TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2] 	3GPP TS 28.104-000 Management and orchestration; Management Data Analytics (MDA)

# 3	Rationale

This pCR is proposed to add MDA analysis report reporting related service component in TS 28.104[2].

# 4	Detailed proposal

It is proposed to add the following chapter in TS 28.104[2].

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.500: "Telecommunication management; Self-Organizing Networks (SON); Concepts and requirements".

[3]	3GPP TS28.535: “Management services for communication service assurance; Requirements”.

[4]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[5]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[6]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[7]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[8]	3GPP TS 28.405: "Telecommunication management, Quality of Experience (QoE) measurement collection; Control and configuration".

[9]	3GPP TS 28.406: "Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport".

[10]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[11]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[y]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[z]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

Next Change

# X	Information model definitions for MDA

## X.1   Imported and associated information entities

### X.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [y], IOC, ManagedFunction | ManagedFunction |
| TS 28.622 [y], IOC, ManagedElement | ManagedElement |
| TS 28.622 [y], IOC, SubNetwork | SubNetwork |

### X.1.2	Associated information entities and local labels

X.2  Class diagram

### X.2.1  Relationships

This clause provides the relationships of relevant classes in UML.

Figure X.2.1-1: NRM fragment for MDA Analytics control

Note1: The Subnetwork can be RAN Subnetwork or CN Subnetwork or Cross-domain Subnetwork, depends on the detailed use case.

X.2.2  Inheritance

Figure X.2.2-1: Inheritance Hierarchy

## X.3  Class definitions

### X.3.1  ClassName <<MDAReportControl>>

#### X.3.1.1  Definition

This <<IOC>> MDAReportControl represents the MDA analysis report request Control and Configuration parameters of MDA analysis service capabilities.

To request a desired MDA analysis report, a MDAS consumer has to create MDAReportControl object instance on the MDAS producer.

#### X.3.1.2  Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mDAType | M | T | T | F | T |
| reportingFilters | M | T | T | F | T |

Editor's Note: whether the reportingmethod can be contained in MDAReportControl IOC is FFS.

#### X.3.1.3  Attribute constraints

None.

#### X.3.1.4  Notifications

The common notifications defined in clause 4.5 as defined in TS 28.541[z] are valid for this IOC, without exceptions or additions.

## X.4  Attribute definitions

### X.4.1  Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| mDAType | It indicates the type of MDA capability for analysis. AllowedValues: Editor's Note: the allowed value is FFS which depends on the detailed analytic use case. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| reportingFilters | It indicates the analysis reporting filter of dedicated MDA Type to be analysis. Editor's Note: the data type of ReportingFilters is FFS which depends on the detailed MDA Type. | type: ReportingFilters multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of changes