3GPP TSG SA WG5 Meeting 140-e		S5-216238

- Electronic meeting, online, 15 - 24 Nov. 2021

- Source:	Huawei

- Title:	Add service experience analysis solution

- Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

This contribution is for approval.

# 2	References

[1]	TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	3GPP TS 28.104-020 “Management and orchestration; Management Data Analytics”

# 3	Rationale

Service experience of end user is key indicator which directly reflects the user satisfaction degree. In 5G system, the diversity of network service is explored and the reuqirements of different service especially form vertical users are standardized. For example, the users who require URLLC service have high requirements on the network latency and reliability, while the users who require NB-IoT service considers the maximum access UE numbers as the first priority for network requirement.

This contribution is proposed to provide the service experience analysis capability of MDA in [2].

# 4	Detailed proposal

It is proposed to apply following changes in [2].

First of changes

- x.x.x	SLS analysis

- x.x.x.x	  Service experience analysis

- x.x.x.x.1	   MDA type

The MDA type for Capability-Service experience analysis is: ServiceExperienceAnalysis.SLSAnalysis

- x.x.x.x.2	Service experience analysis output

Table x.x.x.x.2-1:  Service experience analysis output

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| Service experience identifier | The identifier indicates the analytics report is related with service experience analysis. | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Indication of issue type | Indication of the service experience issue type, e.g. RAN issue, CN issue, UE issue, service provider issue. | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Managed Objects of service experience | The object instances where the service experience is applicable, e.g., SubNetwork Instance, NetworkSlice Instance, S-NSSAI. | O | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Service experience level | The level of service experience, e.g. there are five levels which are represented by 1, 2, 3, 4, 5 where level 1 represents the users are enduring bad experience while level 5 represents the users’ requirements are perfectly satisfied. | O | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Info of service experience | Statistics or predictions of the service experience, may be analysed in different aspects, e.g. per S-NSSAI, per network slice, per network slice subnet, per 5QI, per UE etc. | O | type: list multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Root Cause | The root cause of the service experience issues | O | type: FFS multiplicity: FFS isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of changes