3GPP TSG SA WG5 Meeting 140-e		S5-216242

- Electronic meeting, online, 15 - 24 Nov. 2021

- Source:	Huawei

- Title:	Add network slice load analysis solution

- Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

This contribution is for approval.

# 2	References

[1]	TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	3GPP TS 28.104-020 “Management and orchestration; Management Data Analytics”

# 3	Rationale

Multiple SLS related attributes are defined in ServiceProfile, e.g., maxNumberofUEs, latency, throughput or etc, which are of great importance and should be satisfied. SLS analysis issues are studied in [1], including not only latency analysis, load analysis, jitter analysis.For different network services or vertical users, the requirements may varies a lot. This contribution is proposed to add the network slice load analysis capability of MDA in [2].

# 4	Detailed proposal

It is proposed to apply following changes in [2].

First of changes

- x.x.x	SLS analysis

- x.x.x.x	  Network slice load analysis

- x.x.x.x.1	  MDA type

The MDA type for Capability- Network slice load analysis is: NetworkSliceLoadAnalysis.SLSAnalysis

- x.x.x.x.2	Analytics output

Table x.x.x.x.2-1:  Network slice load analysis output

| Information | Description | Support qualifier | Properties |
| --- | --- | --- | --- |
| Network slice load analysis Identifier | The identifier indicates the output is for Network slice instance load analysis | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Domain of network slice load issue | Indicates the domain of the network slice instance load issue, e.g. RAN issue, CN issue | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Phase of network slice load issue | Indicates the phase of the network slice instance load issue, e.g. historic/ongoing/potential network slice load issue | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| State of network slice load issue | Indicates the state of the network slice instance load issue, e.g. overload/underutilized network slice load issue | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| List of network entities | Lists the network entities involved in the network slice instance load problem | O | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Network slice load distribution | Describes the detailed load distribution or predictive distribution, e.g. load distribution for network slice instances, different locations and/or time periods etc. | O | type: list multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Root cause | The root cause of the network slice instance load problem | O | type: FFS multiplicity: FFS isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Recommended actions | Describes the recommended actions to solve the network slice instance load problem | O | type: FFS multiplicity: FFS isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of changes