3GPP TSG SA WG5 Meeting 140-e		S5-216242

- Electronic meeting, online, 15 - 24 Nov. 2021

- Source:	Huawei

- Title:	Add network slice load analysis solution

- Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

This contribution is for approval.

# 2	References

[1]	TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	3GPP TS 28.104-020 “Management and orchestration; Management Data Analytics”

# 3	Rationale

Multiple SLS related attributes are defined in ServiceProfile, e.g., maxNumberofUEs, latency, throughput or etc, which are of great importance and should be satisfied. SLS analysis issues are studied in [1], including not only latency analysis, load analysis, jitter analysis.For different network services or vertical users, the requirements may varies a lot. This contribution is proposed to add the network slice load analysis capability of MDA in [2].

# 4	Detailed proposal

It is proposed to apply following changes in [2].

First of changes

- 8.4.x	SLS analysis

- 8.4.x.2	  Network slice load analysis

- 8.4.x.2.x	  MDA type

The MDA type for Capability- Network slice load analysis is: NetworkSliceLoadAnalysis.SLSAnalysis

- 8.4.x.2.y	Analytics output

The specific information elements of the analytics output (MDA report) for network slice load analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.x.2.y-1.

Table 8.4.x.2.y-1:  Network slice load analysis output

| Information | Description | Support qualifier | Properties |
| --- | --- | --- | --- |
| Network slice load analysis Identifier | The identifier indicates the output is for Network slice instance load analysis | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Domain of network slice load issue | Indicates the domain of the network slice instance load issue, e.g. RAN issue, CN issue | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Phase of network slice load issue | Indicates the phase of the network slice instance load issue, e.g. historic/ongoing/potential network slice load issue | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| State of network slice load issue | Indicates the state of the network slice instance load issue, e.g. overload/underutilized network slice load issue | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| List of network entities | Lists the network entities involved in the network slice instance load problem | O | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Network slice load distribution | Describes the detailed load distribution or predictive distribution, e.g. load distribution for network slice instances, different locations and/or time periods etc. | O | type: list multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Root cause | The root cause of the network slice instance load problem, e.g. number of UEs accessing the network, number of PDU sessions, end user’s distribution | O | type: FFS multiplicity: FFS isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Recommended actions | Describes the recommended actions to solve the network slice instance load problem,e.g. UE access control policy modification, handover policy modification | O | type: FFS multiplicity: FFS isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of changes