| 3GPP TS 28.316 V0.2.0 (2021-11) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Plug and Connect; Data formats (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Data formats for Plug and Connect	7

4.1	Client identification in DHCP requests	7

4.1.1 	General	7

4.1.1.1	DHCPv4	7

4.1.1.2	DHCPv6	8

4.2	Entities information in DHCP replies	9

4.2.1	General	9

4.2.1.1	DHCPv4	9

4.2.1.2	DHCPv6	9

4.2.2	Certification Authority (CA/RA) server	9

4.2.3	Security Gateway (SeGW)	9

4.2.4	Management system	9

4.3	Entities Fully Qualified Domain Names (FQDN)	9

4.3.1	General	9

4.3.2	Certification Authority (CA/RA) server	9

4.3.3	Security Gateway (SeGW)	9

4.3.4	Management system	9

Annex <X> (informative): Change history	9

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The present document is part of a TS family covering the 3rd Generation Partnership Project Technical Specification Group Services and System Aspects, Management and orchestration; as identified below:

TS 28.314: "Plug and Connect; Concepts and requirements".

TS 28.315: "Plug and Connect; Procedure flows".

TS 28.316: "Plug and Connect; Data formats".

# 1	Scope

The present document specifies data formats for Plug and Connect NE in 3GPP systems.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.314: " Management and orchestration; Plug and Connect; Concepts and requirements”.

[3]	3GPP TS 28.315: "Management and orchestration; Plug and Connect; Procedure flows".

[4]	IETF RFC 3925: "Vendor-Identifying Vendor Options for Dynamic Host Configuration Protocol version 4 (DHCPv4)".

[5]	IETF RFC 8415: "Dynamic Host Configuration Protocol for IPv6 (DHCPv6)".

[6]	IETF RFC 2132: "DHCP Options and BOOTP Vendor Extensions".

[7]	IANA, "Private Enterprise Numbers", <http://www.iana.org/assignments/enterprise-numbers>.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1], TS 28.314 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1] and in TS 28.314 [2].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1], TS 28.314 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1] and TS 28.314 [2].

# 4	Data formats for Plug and Connect

## 4.1	Client identification in DHCP requests

### 4.1.1 	General

This clause describes DHCP options for use with DHCP for IPv4 (DHCPv4) and DHCP for IPv6 (DHCPv6) that NE uses to identify itself in DHCP requests in Plug and Connect (PnC) IP autoconfiguration service as outlined in TS 28.315 [3].

#### 4.1.1.1	DHCPv4

The NE performing the Initial IP Autoconfiguration procedure specified in clause 5.2 of 3GPP TS 28.315 [3], using IPv4 based network stack, may identifiy itself using Vendor Class Identifier DHCPv4 option 60 as specified in RFC 2132 [6]. The format of Vendor Class identifier shall follow the rules specified in clause 9.13 of RFC 2132 [6], which is illustrated in table 4.1.1.1.1:

Table 4.1.1.1.1: Format of Vendor Class Identifier

| Code | Length | Vendor Class Identifier |  |  |
| --- | --- | --- | --- | --- |
| 60 | n | i1 | i2 | … |
| octet | octet | octet | octet | octet |

The NE may identify itself as Mulit-Vendor Plug and Connect (MvPnC) compatible DHCPv4 client using DHCPv4 option 60 in the following way:

- DHCPv4 option code 60;

- Length 5 bytes;

- Vendor class identifier "MvPnC".

The use of Vendor Class Identifier DHCPv4 option 60 with specific value MvPnC is illustrated in table 4.1.1.1.2.

Table 4.1.1.1.2: Use of Vendor Class Identifier

| Code | Length | Vendor Class Identifier |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 60 | 5 | M | v | P | n | C |

The NE may alternatively identify itself by using the Vendor-Identifying Vendor Class DHCPv4 option 124 as specified RFC 3925 [4]. This option contains one or more Vendor-Identifying Vendor class information each identified by Enterprise Number as registered with IANA [7]. The format of Vendor-Identifying Vendor Class shall follow the rules specified in clause 3 of RFC 3925 [4], which is illustrated in table 4.1.1.1.3:

Table 4.1.1.1.3: Format of Vendor-Identifying Vendor Class

| Code | Length 1-255 | Enterprise Number 1 | Vendor Class Data 1 |  | Enterprise Number 2 | Vendor Class Data 2 |  | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Data Length 1 | Opaque data 1 |  | Data Length 2 | Opaque data 2 | … |
| 124 | n | x | n | Data | y | n | Data | ... |
| octet | octet | 4 octets | octets | n octets | 4 octets | octet | n octets |  |

The NE may identify itself as MvPnC compatible DHCPv4 client using DHCPv4 option 124 in the following way:

- DHCPv4 option code 124;

- Vendor enterprise number 10415 for “3GPP”;

- Vendor class data length 5 bytes;

- Vendor class data "MvPnC".

The use of Vendor- Identifying Vendor Class DHCPv4 option 124 with specific value MvPnC is illustrated in table 4.1.1.2.4.

Table 4.1.1.1.4: Use of Vendor-Identifying Vendor Class

| Code | Length 1-255 | Enterprise Number 1 | Vendor Class Data 1 |  |  |  |  |  | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Data Length 1 | Opaque data 1 |  |  |  |  | … |
| 124 | n | 10415 | 5 | M | v | P | n | C | ... |
| octet | octet | 4 octets | octet | octet | octet | octet | octet | octet | … |

The order of vendor-identifying vendor class contained in option 124 does not matter, and any other vendor-identifying vendor class data with a different IANA enterprise number, if required by the vendor, may appear before or after the 3GPP vendor class.

#### 4.1.1.2	DHCPv6

The NE performing the Initial IP Autoconfiguration procedure specified in clause 5.2 of 3GPP TS 28.315 [3], using IPv6 based network stack, shall identify itself using the Vendor Class DHCPv6 option 16 as specified in RFC 8415 [5]. The format of Vendor Class shall follow the rules specified in clause 21.16 of RFC 8415 [5], which is illustrated in table 4.1.1.2.1:

Table 4.1.1.2.1: Format of Vendor Class

| Code | Length | Enterprise Number | Vendor Class Data |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Vendor Class Length 1 | Opaque Data 1 | Vendor Class Length 2 | Opaque Data 2 | … |
| 16 | n | x | n | Data | n | Data | … |
| 2 octets | 2 octets | 4 octets | 2 octets | n octets | 2 octets | n octets | … |

The NE may identify itself as MvPnC compatible DHCPv6 client by using the Vendor Class DHCPv6 option 16 in the following way:

- DHCPv6 option code 16;

- Data length 11 bytes;

- Vendor enterprise number 10415 for “3GPP”

- Vendor class length 5 bytes;

- Vendor class data "MvPnC".

The use of Vendor Class DHCPv6 option 16 with specific value MvPnC is illustrated in table 4.1.1.2.1.

Table 4.1.1.2.2: Use of Vendor Class

| Code | Length | Enterprise Number | Vendor Class Data |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Vendor Class Length 1 | Opaque data 1 |  |  |  |  |
| 16 | 11 | 10415 | 5 | M | v | P | n | C |
| 2 octets | 2 octets | 4 octets | 2 octets | octet | octet | octet | octet | octet |

## 4.2	Entities information in DHCP replies

### 4.2.1	General

#### 4.2.1.1	DHCPv4

#### 4.2.1.2	DHCPv6

### 4.2.2	Certification Authority (CA/RA) server

### 4.2.3	Security Gateway (SeGW)

### 4.2.4	Management system

## 4.3	Entities Fully Qualified Domain Names (FQDN)

### 4.3.1	General

### 4.3.2	Certification Authority (CA/RA) server

### 4.3.3	Security Gateway (SeGW)

### 4.3.4	Management system

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |