3GPP TSG-SA5 Meeting #141-e 	S5-221393

# e-meeting, 17 -26 January 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0105 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify handling of modified network slice profile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNETSLICE_PRO |  |  |  |  |  | Date: |  |  | 2022-01-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - For the allocateNsi operation it is not so clear on how to handle in case the operation includes an already existing serviceProfileId. - For the allocateNssi operation it is not so clear on how to handle in case the operation includes an already existing sliceProfileId. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update 6.5.1.1 AllocateNsi description with handling when ServiceProfileId is included in the request Update 6.5.2.1 AllocateNssi description with handling when SliceProfileId is included in the request Update 7.6 figure 7.6-1 replace ModifyNsi with AllocateNsi and update steps accordingly Update 7.7 Update figure 7.7-1 replace ModifyNssi with AllocateNssi, update steps accordingly |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misunderstanding of how to handle when allocateNsi/allocateNssi includes an existing or known serviceProfileId or sliceProfileId which may lead to incompatible software implementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No impact on stage 3 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

## 6.5.	Operations of provisioning

### 6.5.1	AllocateNsi operation

#### 6.5.1.1	Description

This operation is invoked by network slice provisioning MnS consumer to request the provider to allocate a network slice instance to satisfy network slice related requirements. The provider may create a new NSI or using existing NSI to satisfy the request. The requirements in the request are compared/matched against the actual capabilitites of all candidate NSIs by the provider. If an existing NSI can be found e.g. with the right coverage and with good enough latency, it is eligible for allocation. In case not, or if networkSliceSharingIndicator is equal to "non-shared", a new NSI is created with capabilities to host the service, provided that required NSSIs can be created.

The following scenarios are supported by the AllocateNsi operation:

- when the AllocateNsi operation does not include a serviceProfileId, the producer will treat the request as a create request and create a new serviceProfileId or;

- when the AllocateNsi operation includes a known serviceProfileId, the producer will treat the request as a modification request and the corresponding ServiceProfile is updated provided that the constituent NSSIs can be modified or;

- when the AllocateNsi operation includes an unknown serviceProfileId, the producer will reject the request.

#### 6.5.1.2	Input parameters

| Parameter Name | Support Qualifier | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListIn | M | LIST OF SEQUENCE< attribute name, attribute value> | This parameter specifies the network slice related requirements defined in ServiceProfile in Clause 6.3.3 in TS 28.541 [6]. |

#### 6.5.1.3	Output parameters

| Parameter name | Support Qualifier | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListOut | M | LIST OF SEQUENCE< attribute name, attribute value> | This list of name/value pairs contains all of the attributes of the ServiceProfile [6] which have been allocated or modified and the actual value assigned to each as a result of the input requirements. |
| status | M | ENUM (OperationSucceeded, OperationFailed) | An operation may fail because of a specified or unspecified reason. |
| networkSliceDN | M | The DN of NetworkSlice MOI uniquely identifying the network slice instance. |  |

### 6.5.2	AllocateNssi operation

#### 6.5.2.1	Description

This operation is invoked by network slice subnet provisioning MnS consumer to request the provider to allocate a network slice subnet instance to satisfy the network slice subnet related requirements. The provider may create a new NSSI or using existing NSSI to satisfy the request. The requirements in the request are compared/matched against the actual capabilitites of all candidate NSSIs by the provider. If an existing NSSI can be found e.g. with the right coverage and with good enough latency, it is eligible for allocation. In case not, a new NSSI is created with capabilities to host the service, if enough resoures are available.

The following scenarios are supported by the AllocateNssi operation:

- when the AllocateNssi operation does not include a sliceProfileId, the producer will treat the request as a create request and create a new sliceProfileId or;

- when the AllocateNssi operation includes a known sliceProfileId, the producer will treat the request as a modification request and the corresponding SliceProfile is updated provided that the (when applicable) constituent NSSIs can be modified or;

- when the AllocateNssi operation includes an unknown sliceProfileId, the producer will reject the request.

#### 6.5.2.2	Input parameters

| Parameter Name | Support Qualifier | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListIn | M | LIST OF SEQUENCE< attribute name, attribute value> | This parameter specifies the network slice subnet related requirements defined in SliceProfile in Clause 6.3.4 in TS 28.541 [6]. |

#### 6.5.2.3	Output parameters

| Parameter name | Support Qualifier | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListOut | M | LIST OF SEQUENCE< attribute name, attribute value> | This list of name/value pairs contains all of the attributes of the SliceProfile[6] which have been allocated or modified and the actual value assigned to each as a result of the input requirements. |
| status | M | ENUM (OperationSucceeded, OperationFailed) | An operation may fail because of a specified or unspecified reason. |
| networkSliceSubnetDN | M | The DN of NetworkSliceSubnet MOI uniquely identifying the network slice subnet instance. |  |

Second Change

## 7.6	Procedure of Network Slice Instance Modification

The Figure 7.6-1 illustrates the procedure of modifying an existing NSI.

Figure 7.6-1: Network Slice Instance Modification procedure

1)	Network Slice Management Service Provider (NSMS_P) receives an AllocateNsi operation defined in clause 6.5.1from Network Slice Management Service Consumer (NSMS_C) with the modifiednetwork slice related requirements including a value for the attribute serviceProfileId (see ServiceProfile defined in clause 6.3.3 in TS 28.541[6]).

2)	Based on the modified network slice related requirements, NSMS_P verifies the existence of the serviceProfileId and invokes the feasibility check procedure. If the modified modified network slice requirements can be satisfied, go to step 3), else go to step 5).

3) NSMS_P decomposes the serviceProfile modification request into sliceProfile modification request(s), i.e., generating the new network slice subnet related requirements for each NSSI if needed.

4) NSMS_P, as the role of Network Slice Subnet Management Service Consumer (NSSMS_C), invokes the NSSI modification procedure where required.

5) NSMS_P sends AllocateNsi result to NSMS_C.

## 7.7	Procedure of Network Slice Subnet Instance Modification

The Figure 7.7-1 illustrates the procedure of modifying an existing NSSI.

Figure 7.7-1: Network Slice Subnet Instance Modification procedure

1)	Network Slice Subnet Management Service Provider (NSSMS_P) receives an AllocateNssi operation defined in clause 6.5.2from Network Slice Subnet Management Service Consumer (NSSMS_C) with the modified new network slice subnet related requirements (see SliceProfile defined in clause 6.3.3 in TS 28.541[6]).

2)	Based on the new network slice subnet related requirements, NSSMS_S verifies the existence of the sliceProfileId and invokes the feasibility check procedure. If the modification requirements can be satisfied, go to step 3), else go to step 5).

3) NSSMS_P decomposes the sliceProfile modification request into modification requests for each NSSI constituent.

4a) If the requested NSSI constituent is constituent NSSI, NSSMS_P invokes NSSI modification procedure as described in clause 7.7.

4b) If the requested NSSI constituent is NF instance, NSSMS_SP invokes NF creation procedure as described in clause 7.10 or NF modification procedure as described in clause 7.11.

4c)	If the NSSI contains the virtualized part, NSSMS_SP invokes the NS instance scaling and/or NS instance updating and/or NS instance instantiation procedure as described in TS 28.526 [7].

4d) If the NSSI contains the TN part, NSSM_SP invokes the TN related coordination procedure as described in clause 7.9.

5) NSSM_SP sends AllocateNssi results  to NSSMS_C.

End of Changes