3GPP TSG-SA3 Meeting #106-e 	S3-221464

# e-meeting, 14 - 25 February 2022

# Source:	3GPP RAN2

# Title:	Reply LS on Beam measurement reports for the MDT measurements

# Document for:	Information, Discussion

# Agenda Item:	6.1

3GPP TSG-RAN WG2 Meeting #116-e 	R2-2111476

# Online meeting, 1st November – 12th November 2021

Title:	Reply LS on Beam measurement reports for the MDT measurements

Response to:	R2-2109352/R3-214519

Release:	Rel-17

Work Item:	NR_ENDC_SON_MDT_enh-Core

Source:	RAN2

To:	RAN3

Cc:	SA5

Contact Person:

#### Name:	Pradeepa Ramachandra

E-mail:	pradeepa.ramachandra@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN2 thanks RAN3 for their LS in (R2-2109352/R3-214519).

In the original LS, RAN3 has asked the following:

To RAN2 and SA5 group

ACTION: 	RAN3 kindly asks RAN2 and SA5 to indicate whether the proposals above, namely to include in the M1 Configuration signalled over the RAN interfaces (e.g. NG, Xn, F1) information describing whether and how beam measurements should be configured at the UE for M1 measurements, is feasible.

RAN2 has discussed this topic and the following agreements were made:

- RAN2 confirms that includeBeamMeasurements is mandatorily configured by gNB as part of ReportConfigNR with the value TRUE or FALSE.

- RAN2 confirms that the OAM could indicate operator’s preference the beam level measurement inclusion (includeBeamMeasurements) in the measurement reports associated to the M1 measurement so that the beam level coverage map can be generated by the OAM.

- RAN2 confirms that an indicator (includeBeamMeasurements) can be added in the M1 Configuration signaled over the RAN interfaces (e.g., NG, Xn, F1) to indicate to RAN whether the beam measurements are preferred be included for MDT purposes by the UE for M1 measurements or not.

- RAN2 confirms that for NR M1: Immediate MDT purposes involve reporting triggers and criteria utilized for RRM.

- RAN2 also wants to inform the RAN3 that the reference signal type (i.e. SSB and CSI-RS) used for reporting the beam level measurement information is the same as that of cell level measurement information.

2. Actions:

To RAN3:

ACTION: 	RAN2 kindly asks RAN3 to take the above feedback into account.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#116-bis-e	17 – 25 Jan 2021	Online e-meeting

RAN2#117-e	21 Feb – 3 Mar 2021	Online e-meeting