3GPP TSG SA WG5 Meeting 141-e		S5-221146

- Electronic meeting, online, 17 - 26 Nov. 2022

- Source:	Huawei

- Title:	Add network slice load analysis solution

- Document for:	Approval

Agenda Item:	6.4.15

# 1	Decision/action requested

This contribution is for approval.

# 2	References

[1]	TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	3GPP TS 28.104-020 “Management and orchestration; Management Data Analytics”

# 3	Rationale

Multiple SLS related attributes are defined in ServiceProfile, e.g., maxNumberofUEs, latency, throughput or etc, which are of great importance and should be satisfied. SLS analysis issues are studied in [1], including not only latency analysis, load analysis, jitter analysis.For different network services or vertical users, the requirements may varies a lot. This contribution is proposed to add the network slice load analysis capability of MDA in [2].

# 4	Detailed proposal

It is proposed to apply following changes in [2].

First of changes

- 8.4.x	SLS analysis

- 8.4.x.5	  Network slice load analysis

- 8.4.x.5.1	  MDA type

The MDA type for Capability- Network slice load analysis is: SLSAnalysis. NetworkSliceLoadAnalysis.

- 8.4.x.5.2	   Enabling data

The enabling data for network slice load analysis are provided in table 8.4.x.5.2-1.

Table 8.4.x.2.2-1: Enabling data for network slice load analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | Maximum number of PDU sessions of network slice | Maximum number of PDU sessions of network slice as defined in 6.4.5 in TS28.554 [5] |

- 8.4.x.5.3	Analytics output

The specific information elements of the analytics outputfor network slice load analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.x.5.3-1.

Table 8.4.x.5.3-1:  Network slice load analysis output

| Information | Description | Support qualifier | Properties |
| --- | --- | --- | --- |
| NetworkSliceLoadIssueId | The identifier indicates the output is for Network slice instance load analysis | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceLoadIssueDomain | Indicates the domain of the network slice instance load issue The allowed value is one of the enumerated values: RAN issue, CN issue | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceLoadIssuePhase | Indicates the phase of the network slice instance load issue The allowed value is one of the enumerated values: historic network slice load issue, ongoing network slice load issue, potential network slice load issue | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceLoadIssueType | Indicates the type of the network slice instance load issue The allowed value is one of the enumerated values: overload network slice load issue, underutilized network slice load issue | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AffectedObjects | The managed object instances involved in the network slice instance load problem | O | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceLoadDistribution | Describes the detailed load distribution or predictive distribution, e.g. load distribution for a network slice instance at a certain location or in a certain time period. | O | type: list multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of changes