3GPP TSG-SA5 Meeting #141-e 	S5-221190rev1

- e-meeting, 17 -26 January 2022

- Source:	Huawei

- Title:	pCR 28.104 Add the requirements for ML model training for MDA

- Document for:	Approval

Agenda Item:	6.4.15

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]		3GPP TS 28.104 Management and orchestration; Management Data Analytics (MDA)

# 3	Rationale

TS 28.104 [1] has defined the use case and requirement for ML model training for MDA in clause 7.4.1.

In the ML model training requested by consumer use case, the ML model training may be triggered by the request from the MLMT MnS consumer. In this case, the MLMT MnS producer needs to know some information about the model that consumers request so as to start the model training. One of the basic information is the model name which indicates the function or purpose of the model. For example, a model with the model name “CoverageProblemAnalysis” can be used by MDA MnS producer to generate the analytics output for coverage problem analysis. Thus, the consumer should specify the model name of the ML model to be trained in the model training request. The MLMT MnS producer can then perform the model training according to the designated model name. In addition, the consumer may have requirements for model performance, e.g., accuracy, etc. The MLMT MnS consumers can specify their model performance requirements in the model training request. According to the performance requirements, the MLMT MnS producer can determine how to perform model training to satisfy the requirement of consumers.

Based on above discussion, this contribution is proposed to add some description and requirement in draft TS 28.104 [1].

# 4	Detailed proposal

It is proposed to add the following chapter in TS 28.104[1].

1st modified section

#### 7.4.1.2	Use cases

##### 7.4.1.2.1	ML model training requested by consumer

The ML model training capabilities are provided by an MLMT MnS producer to one or more consumer(s).

Figure 7.X.1-1: ML model training requested by MLMT MnS consumer

The ML model training may be triggered by the request(s) from one or more MLMT MnS consumer(s).To trigger an ML model training, the MLMT MnS consumer requests the MLMT MnS producer to train the ML model. In the ML model training request, the consumer should specify the model name which indicates the function or purpose of the model, e.g., CoverageProblemAnalysis. The MLMT MnS producer can perform the model training according to the purpose. The consumer may provide the data source(s) that contain(s) the training data which are considered as inputs candidates for training. To obtain the valid ML model, consumers may also designate their requirements for model performance (e.g., accuracy, etc) in the training request.

The MLMT MnS producer provides a response to the consumer indicating whether the request was accepted.

If the request is accepted, the MLMT MnS producer decides when to start the ML model training with consideration of the request(s) from the consumer(s). Once the training is decided, the producer

-	selects the training data, with consideration of the consumer provided candidate training data. Since the training data directly influences the algorithm and performance of the trained model, the MLMT MnS producer may examine the consumer provided training data and select none, some or all of them. In addition, the MLMT MnS producer may select some other training data that are available.

-	trains the ML model using the selected training data, and

-	provides the training result (including the location of the trained model, etc) to the MLMT MnS consumer (s).

##### 7.4.1.2.2	ML model training initiated by producer

The ML model training may be initiated by the MLMT MnS producer, for instance as result of evaluation of performance of the ML model, based on the feedback received from the consumer, or when new training data describing the new network status/events are available.

When the MLMT MnS producer decides to start the ML model training, the producer

-	selects the training data,

-	trains the ML model using the selected training data, and

-	provides the training result (including the location of the trained model, etc) to the MLMT MnS consumer(s) who have subscribed to receive the ML model training results.

#### 7.4.1.3	Requirements for ML model training for MDA

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-MDA_ML-FUN-1 | The MLMT MnS producer shall have a capability allowing the consumer to request ML model training. | ML model training requested by consumer (clause 7.4.1.2.1) |
| REQ-MDA_ML-FUN-2 | The MLMT MnS producer shall have a capability allowing the consumer to specify the data sources containing the candidate training data for ML model training. | ML model training requested by consumer (clause 7.4.1.2.1) |
| REQ-MDA_ML-FUN-3 | The MLMT MnS producer shall have a capability allowing the consumer to specify the purpose of the ML model to be trained. | ML model training requested by consumer (clause 7.4.1.2.1) |
| REQ-MDA_ML-FUN-4 | The MLMT MnS producer shall have a capability to provide the training result (including the location of the trained model) to the consumer. | ML model training requested by consumer (clause 7.4.1.2.1), and ML model training initiated by producer (clause 7.4.1.2.2) |

End of modified section