- 3GPP TSG-SA5 Meeting #141e	S5-221336rev2

- 17 - 26 January 2022, E-meeting

- Source:	Intel

- Title:	pCR 28.105 Add NRMs for AI/ML model training

- Document for:	Approval

- Agenda Item:	6.4.15

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TS 28.xxx-000 “Management and orchestration; AI/ML management”.

# 3	Rationale

The AI/ML model training UCs and requirements were agreed. This pCR is to add the corresponding stage 2 NRMs to support the stage 1 requirements.

# 4	Detailed proposal

Start of modification

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[a]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[b]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) Model Repertoire".

[c]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[d]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[e]	3GPP TS 28.532: "Management and orchestration; Generic management services".

Next modification

# X	Information model definitions for AI/ML management

## X.1	Imported and associated information entities

### X.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [a], IOC, Top | Top |

### X.1.2	Associated information entities and local labels

| Label reference | Local label |
| --- | --- |

## X.2	Class diagram

### X.2.1	Relationships

This clause depicts the set of classes (e.g., IOCs) that encapsulates the information relevant to AI/ML model training. For the UML semantics, see 3GPP TS 32.156 [b].

Editor’s note: it is FFS which IOC should be the root container of the AI/ML related IOCs.

Figure X.2.1-1: NRM fragment for AI/ML model training

### X.2.2	Inheritance

Figure X.2.2-1: Inheritance Hierarchy for AI/ML model training related NRMs

## X.3	Class definitions

### X.3.1	AIMLTrainingRequests

#### X.3.1.1	Definition

The IOC AIMLTrainingRequests represents the container of the AIMLTrainingRequest IOC(s).

#### X.3.1.2	Attributes

None.

#### X.3.1.3	Attribute constraints

None.

#### X.3.1.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

### X.3.2	AIMLTrainingRequest

#### X.3.2.1	Definition

The IOC AIMLTrainingRequest represents the AI/ML model training request that is created by the MnS consumer.

The AIMLTrainingRequest MOI is contained under one AIMLTrainingRequests MOI.

#### X.3.2.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| aIMLModelId | M | T | T | F | T |
| candidateTraingDataSource | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |

#### X.3.2.3	Attribute constraints

None.

#### X.3.1.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

### X.3.3	AIMLTrainingReports

#### X.3.3.1	Definition

The IOC AIMLTrainingReports represents the container of the AIMLTrainingReport IOC(s).

#### X.3.3.2	Attributes

None.

#### X.3.3.3	Attribute constraints

None.

#### X.3.3.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

### X.3.4	AIMLTrainingReport

#### X.3.4.1	Definition

The IOC AIMLTrainingReport represents the AI/ML model training report that is provided by the MnS producer. This IOC contains the attributes for the information of a trained AI/ML model without disclosing the training (learning) algorithm and model’s inference algorithm.

The AIMLTrainingReport MOI is contained under one AIMLTrainingReports MOI.

#### X.3.4.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| aIMLModelId | M | T | F | F | T |
| aIMLModelPackageAddress | M | T | F | F | T |
| inferenceType | M | T | F | F | T |
| trainingAccuracyScore | M | T | F | F | T |
| areConsumerTrainingDataUsed | M | T | F | F | T |
| usedConsumerTrainingData | CM | T | F | F | T |
| Attribute related to role |  |  |  |  |  |
| trainingRequestRef | CM | T | F | F | T |
| lastTrainingRef | CM | T | F | F | T |

#### X.3.4.3	Attribute constraints

| Name | Definition |
| --- | --- |
| usedConsumerTrainingData Support Qualifier | Condition: The value of areConsumerTrainingDataUsed attribute is PARTIALLY. |
| trainingRequestRef Support Qualifier | Condition: The AIMLTrainingReport MOI represents the report for the AI/ML model training that was requested by the MnS consumer (via AIMLTrainingRequest MOI). |
| lastTrainingRef Support Qualifier | Condition: The AIMLTrainingReport MOI represents the report for the AI/ML model training that was not initial training (i.e., the model has been trained before). |

#### X.3.4.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

## X.4	Attribute definitions

### X.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| aIMLModelId | It identifies the AI/ML model. It is unique in each MnS producer. | type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: True |
| aIMLModelPackageAddress | It provides the address where the AI/ML model package is located. The AI/ML model package may contain the AI/ML model (e.g., software image or file) and the model descriptor. The model descriptor may contain more detailed information about the model, such as version, resource requirements, etc. The model descriptor is not specified in the present document. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| candidateTraingDataSource | It provides the address(es) of the candidate training data source provided by MnS consumer. The detailed training data format is vendor specific. | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| inferenceType | It indicates the type of inference that the AI/ML model supports. allowedValues: the values of the MDA type (see TS 28.104 [c]), Analytics ID(s) of NWDAF (see TS 23.288 [d]), RAN-intelligence inference function name(s), and vendor’s specific extensions. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| trainingAccuracyScore | It indicates the training accuracy score of the AI/ML model. It is on a scale from 0 to 100, where 100 indicates the best. The algorithm for calculating the accuracy score is vendor’s specific. allowedValues: { 0 .. 100 }. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| areConsumerTrainingDataUsed | It indicates whether the consumer provided training data have been used for the AI/ML model training. allowedValues: ALL, PARTIALLY, NONE. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| usedConsumerTrainingData | It provides the address(es) where lists the consumer provided training data which have been used for the AI/ML model training. | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| trainingRequestRef | It is the DN(s) of the related AIMLTrainingRequest MOI(s). | type: DN (see TS 32.156 [b]) multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| lastTrainingRef | It is the DN of the AIMLTrainingReport MOI that represents the reports for the last training of the AI/ML model. | type: DN (see TS 32.156 [b]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |

### X.4.2	Constraints

| Name | Affected attribute(s) | Definition |
| --- | --- | --- |

## X.5	Common notifications

### X.5.1	Configuration notifications

This clause presents a list of notifications, defined in TS 28.532 [e], that an MnS consumer may receive. The notification header attribute objectClass/objectInstance shall capture the DN of an instance of a class defined in the present document.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyMOICreation | O | -- |
| notifyMOIDeletion | O | -- |
| notifyMOIAttributeValueChanges | O | -- |
| notifyEvent | O | -- |

Next modifications

# Y	Service components

## Y.1	Service components for AI/ML model training MnS

The components for AI/ML model training MnS are listed in table Y.1-1:

Table Y.1-1: Components for AI/ML model training MnS

| Management service component type A | Management service component type B | Management service component type C |
| --- | --- | --- |
| The operations and notifications for generic provisioning management service (see clause 11.1.1 of TS 28.532[e]). | AIMLTrainingRequests IOC; AIMLTrainingRequest IOC; AIMLTrainingReports IOC; AIMLTrainingReport IOC. | None |

End of modifications