- 3GPP TSG-SA5 Meeting #141e	S5-221612d1

- 17 Jan to 26 Jan 2022, E-meeting                                                   revision of S5-221251

- Source:	China Telecom, China Mobile, HUAWEI

- Title:	Add MDA capability for MDA assisted energy saving

- Document for:	Approval

- Agenda Item:	6.4.15

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TS 28.104-030 “Management and orchestration; Management Data Analytics”

# 3	Rationale

This pCR is to add the MDA capability for MDA assisted energy saving analysis for TS 28.104 [1].

# 4	Detailed proposal

Start of modifications

### 8.4.X	MDA assisted energy saving

#### 8.4.X.1	MDA type

The MDA type for energy saving analysis is: MDAAssistedEnergySaving.EnergySavingAnalysis.

#### 8.4.X.2	Enabling data

The enabling data for coverage problem analysis are provided in table 8.4.X.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.X.2-1: Enabling data for energy saving analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | PNF Power Consumption: power consumed over the measurement period | Clause 5.1.1.19.2 of TS 28.552 [4] |
|  | PNF Energy consumption: energy consumed | Clause 5.1.1.19.3 of TS 28.552 [4] |
|  | SS-RSRP distribution per SSB (beam) of serving NR cell | Clause 5.1.1.22.1 of TS 28.552 [4]. |
|  | SS-RSRP distribution per SSB (beam) of neighbor NR cell | Clause 5.1.1.22.1 of TS 28.552 [4]. |
|  | PDCP Data Volume of NR cells: PDCP data volume delivered in the downlink and uplink; | Clause 5.1.2.1 and 5.1.3.6 of TS 28.552 [4] |
|  | Traffic load variation: PRB utilization rate, RRC connection number, etc | Clause 5.1.1.2 and 5.1.1.4 of TS 28.552 [4] |
|  | UE throughput: UE throughput in downlink and uplink | Clause 5.1.1.3 of TS 28.552 [4] |
|  | Delay related measurements of UPF | Clause 5.4 of TS 28.552 [4] |
|  | Data volume of UPF | Clause 5.4 of TS 28.552 [4] |
|  | Virtual resource usage of NF: The virtual CPU usage, virtual memory usage, virtual disk usage of virtual network functions; | Clause 5.7.1 of TS 28.552 [4] |
| MDT reports | The RSRPs of UE measurements. | RSRPs of M1 measurements in TS 32.422 [6] and TS 32.423 [7]. |
|  | The RSRQs of UE measuremnets. | RSRQs of M1 measurements in TS 32.422 [6] and TS 32.423 [7]. |
|  | The UE location information. | UE location of M1 measurements in TS 32.422 [6] and TS 32.423 [7]. |
| QoE Data | The measurements that are collected are DASH and MTSI measurements. | TS 28.406 [9] |
| Configuration data | MOIs of the cells, UPFs and SMFs. | TS 28.541 [15] |
| Network analytics data | The control plane analysis result from the NWDAF, e.g., observed service experience related network data analytics. | TS 23.288 [10] |

#### 8.4.X.3	Analytics output

The specific information elements of the analytics output for energy saving analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.X.3-1.

Table 8.4.X.3-1: Analytics output for energy saving analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| EnergyEfficiencyProblematicObject | Indication of NR cells or NFs where the energy efficiency issues occurred or potentially occur. | M | type: DN multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| EnergyEfficiencyProblemType | Indication of type of the energy efficiency issues. The allowed value is one of the enumerated values: HighEnergyConsumption, LowEenergyEfficiency, Other, Unknown. | M | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TrafficLoadTrends | The predictions of the trends of traffic load in a certain time period. The predictions include the traffic load of the issue cell(s) and neithboring cell(s). | M | type:TrafficLoadTrend multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| EnergySavingRecommendations | The recommendation shall contain the energy saving policy. For ES on NR cells. It may contain a set of - recommended NR Cell (ES-Cell) to enter energySaving state. - recommended candidate cells with precedence for taking over the traffic of the ES-Cell. - the time to enter and terminate the energy saving state. - the load threshold to enter and terminate the energy saving state for the ES-Cell For ES on UPFs. It contains a set of - recommended UPF (ES-UPF) to conduct energy saving; - recommended candidate UPFs with precedence for taking over the traffic of the ES-UPF. | M | type: EsRecommendation multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| StatisticsOfCellsEsState | The statistic result of current energy saving state of the cells at a certain time, which can be used by consumers to make analysis (e.g., observed service experience analysis made by NWDAF) or to make decision (e.g., enter/exit the energy saving state based on the current energy saving state). | O | type: StatisticOfCellEsState multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

## 8.X	Data type definitions

### 8.X.a	TrafficLoadTrend

#### 8.X.a.1	Definition

This data type specifies the type of TrafficLoadTrend.

#### 8.X.a.2	Information elements

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| CellId | It indicates the cell for which the traffic load prediction is performed. | M | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| StartTime | It indicates the start time that are used for predict. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| EndTime | It indicates the end time that are used for predict. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TrafficLoadList | It provides a list of PRB usage based on a specific granularity.. | M | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

### 8.X.b	EsRecommendation

#### 8.X.b.1	Definition

This data type specifies the type of energy saving recommendations in the analytics output.

#### 8.X.b.2	Information elements

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| EsRecommendationsOnNRcells | It contains the energy saving recommendations on NR cells. | M | type: EsRecommendationsOnNRcell multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| EsRecommendationsOnUPFs | It contains the energy saving recommendations on UPFs. | M | type: EsRecommendationsOnUPF multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

### 8.X.c	EsRecommendationsOnNRcell

#### 8.X.c.1	Definition

This data type specifies the type of energy saving recommendations on NR cells.

#### 8.X.c.2	Information elements

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| EsNRcells | It provides the DN of NR cells (ES-Cell) which are recommended to enter energySaving state. | M | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CandidateNRcells | It provides the DN of candidate NR cells which are recommended with precedence for taking over the traffic of ES-Cell. | M | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| EnterTime | It provides the recommended time to enter the energy saving state for the ES-Cell. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| EndTime | It provides the recommended time to terminate the energy saving state for the ES-Cell | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| EnterTrafficThreshold | It provides the recommended entering traffic threshold, which is related to EnterTime. When the traffic of ES-cell is lower than the threshold, the ES-cell can enter the energy saving state. | M | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| EndTrafficThreshold | It provides the recommended end traffic threshold, which is related to EndTime. When the traffic of ES-cell is higher than the threshold, the ES-cell can terminate the energy saving state. | M | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

### 8.X.d	EsRecommendationsOnUPF

#### 8.X.d.1	Definition

This data type specifies the type of energy saving recommendations on UPFs.

#### 8.X.d.2	Information elements

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| EsUPFs | It provides the DN of UPFs (ES-UPF) which are recommended to conduct energy saving. | M | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CandidateUPFs | It provides the DN of candidate UPFs which are recommended with precedence for taking over the traffic of ES-UPF. | M | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

### 8.X.e	StatisticOfCellEsState

#### 8.X.e.1	Definition

This data type specifies the type of statistics of cells energy saving state in the analytics output.

#### 8.X.e.2	Information elements

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| StartAnalyticsTime | It indicates the start time information that are used for analytics. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| EndAnalyticsTime | It indicates the end time information that are used for analytics. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CellsInEsState | It provides the DN of NR cells which are always in energy saving state between the StartAnalyticsTime and EndAnalyticsTime. | M | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CellsInNotEsState | It provides the DN of NR cells which are always in not energy saving state between the StartAnalyticsTime and EndAnalyticsTime. | M | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ChangedCellsInEsState | It provides the DN of NR cells whose energy saving state changed between the StartAnalyticsTime and EndAnalyticsTime, and is finally in energy saving state at EndAnalyticsTime | M | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ChangedCellsInNotEsState | It provides the DN of NR cells whose energy saving state changed between the StartAnalyticsTime and EndAnalyticsTime, and is finally in not energy saving state at EndAnalyticsTime. | M | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of modifications