3GPP TSG-SA5 Meeting #141-e 	S5-221238

e-meeting, January 17 – 26 2022

- Source:	Nokia, Asiainfo

- Title:	Update intent fulfillment tracking in information model

- Document for:	Approval

- Agenda Item:	6.4.9

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP draft TS 28.312: “Management and orchestration; Intent driven management services for mobile networks v0.7.0”.

# 3	Rationale

This contribution proposes to update the intent information model in clause 6.2.1 to support an elaborated tracking of the fulfillment of the different aspects of the intent . It proposes

- to expand filfillmentStatus to a fulfilmentinfo datatype which includes both the status as well as the reason behind or related to a particular status.

- to elaborate the fulfilment info such that it can eb provided for the whole intent, any of its expectations of even any of the specific targets in any one expectation.

- to expand the possible status values to includes the cases where the producer suspects an intent as well as the transient case for when the producer is still working on fulfilling the intent, expectation or target.

  - Relatedly, it proposes to remove the editor's note for the intentFulfilStatus attribute definition

# 4	Detailed proposal

It proposes to make the following changes to TS 28.312[1].

1st Change

- 6.2.1.2	Class definition

- 6.2.1.2.1 	Intent <<IOC>>

- 6.2.1.2.1.1	Definition

This IOC represents the properties of an Intent. The Intent IOC contains one or multiple intentExpectation(s) which includes MnS consumer’s requirements, goals and constraints given to a 3GPP system.

The Intent IOC includes the attribute objectClass and objectInstance from the TOP IOC. The value of attribute objectClass is “Intent” and the value of attribute objectInstance is the DN of the instance of Intent IOC.

- 6.2.1.2.1.2	Attributes

The Intent includes attributes inherited from TOP IOC (defined in TS 28.622) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| intentExpectations | M | T | T | F | T |
| intentId | M | T | T | F | T |
| intentContexts | O | T | T | F | T |
| intentFulfillmentinfo | M | T | T | F | T |

Editor’s Note: whether other the attributes are needed for the Intent IOC needs further discussion.

- 6.2.1.2.1.3	Attribute constraints

None

- 6.2.1.2.2	IntentExpectation

- 6.2.1.2.2.1	Definition

IntentExpectation class represent MnS consumer’s requirements, goals and constraints given to a 3GPP system.

Editor’s Note: more description for IntentExpectation will be added later based on the further discussion.

- 6.2.1.2.2.2	Attributes

The IntentExpectation includes the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| expectationId | M | T | T | T | T |
| expectationObject | MO | T | T | F | T |
| expectationObjectContexts | O | T | T | F | T |
| expectationTargets | O | T | T | F | T |
| expectationContexts | O | T | T | F | T |
| expectationfulfilmentInfo | O | T | T | F | T |

Note: The scenario/requirements-specific IntentExpectations are defined utilizing the constructs of this generic IntentExpectation model.

Editor’s Note: whether other the attributes are needed for the IntentExpectation IOC needs further discussion.

Editor’s Note: The naming of the terms may need further discussion.

Editor’s Note: whether both expectionObjectContexts and ExpetationTargets are mandatory is FFS.

- 6.2.1.2.2.3	Attribute constraints

TBD

##### 6.2.1.2.3	IntentReport

###### 6.2.1.2.3.1	Definition

IntentReport class represents intent fulfilment feedback information that MnS consumer can obtain from a 3gpp system. the intentReport will capture the intentFulfillmentInfo, ExpectationFulfillmentInfo and targetFulfillmentInfo as well as other information that the MnS producer needs to report to MnS consumer at the time instant as which the report is compiled.

Editor’s Note: more description for IntentReport will be added later based on the further discussion.

###### 6.2.1.2.3.2	Attributes

TBD

###### 6.2.1.2.3.3	Attribute constraints

TBD

2nd Change

- 6.2.1.3	DataType definition

- 6.2.1.3.1 	expectationTarget <<dataType>>

- 6.2.1.3.1.1	Definition

This <<dataType>> represents the  Targets of the IntentExpectation that are required to be achieved .

- 6.2.1.3.1.2	Attributes

The expectationTarget includes the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| targetName | M | T | T | T | T |
| targetCondition | M | T | T | F | T |
| targetValueRange | M | T | T | F | T |
| targetContexts | O | T | T | F | T |
| targetfulfillmentInfo | O | T | F | F | T |

- 6.2.1.3.1.3	Attribute constraints

None

- 6.2.1.3.n 	fulfillmentInfo << dataType >>

- 6.2.1.3.n.1	Definition

This dataType represents the properties of a specific fulfilment information for an aspect of the intent (i.e. either an expectation, a target or the whole intent). The fulfilment information describes the producer's assessment of the degree to which a specific aspect of the intent has been fulfilled. The consumer may however assess the fulfilment differently e.g., the consumer may evaluate the delivered outcome or network state to compute its fulfilment satisfaction.

The fulfillmentStatus field indicates whether the intent is fulfilled or not fulfilled. The possible values of the fulfilment include:

- NOTFULFILLED: This is the default status for any aspect of the intent and the fulfillmentStatus remains as " NOTFULFILLED " until the producer is satisfied that the undertaken actions meets the requirements as staed by the consumer.

- FULFILLED: This is the status if the producer considers that the intent, expectation or target has been fulfilled as desired by the consumer that created the intent. The consumer may provide a fulfilment satisfaction report that either confirms the fulfilment or describes its evaluation the fulfilment.

The degree of fulfilment of an intent with the NOTFULFILLED status may have multiple explanations and related states. These different progress states and conditions are recorded in the fulfillmentProgressState field. The possible values of the fulfillmentProgressState include:

- ONGOING: this is the default status and is the initial fulfillmentProgressState right after the intent has been activated

- SUSPENDED: this is the state if the producer decides to suspect the fulfilment of the intent, expectation or target for whatever reason. This is fulfillmentProgressState must be supported by a reason such as the event(s) that were observed when fulfilment was attempted.

- "CANCELED": This state is registered if the respective aspect of the intent (i.e. either an expectation, a target or the whole intent) shall not be considered for fulfilment e.g. when an authorized consumer sends an indication terminating the specific aspect of the intent. For instance, if the consumer sends an update of the intent in which a particular target is eliminated, then that target shall be marked as cancelled.

- FULFILLMENTFAILED: This is the state when the producer decides that the intent, expectation or target cannot be fulfilled. This is state must be supported by a reason such as the event(s) that were observed when fulfilment was attempted.

For some scenarios (in particular for the "suspended" and the "fulfillmentFailed" fulfillmentProgressStates ), the fulfillmentProgressState must be supported by extra information describing or related to the state. This extra information is recorded into the fulfillmentProgressStateReasons field.

- 6.2.1.3.n.2	Attributes

The fulfillmentInfo includes the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| fulfillmentStatus | M | T | F | F | T |
| fulfillmentProgressState | CM | T | F | F | T |
| fulfillmentProgressReasons | O | T | T | F | T |

- 6.2.1.3.n.3	Attribute constraints

None

3rd Change

#### 6.2.1.4	Attribute definition

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| userLabel | A user-friendly (and user assignable) name of the intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: F isUnique: F defaultValue: None isNullable: False |
| intentExpectation | It indicates the expectations including requirements, goals and context (including constraints and filter information) given to a 3GPP system. It states the list of specific outcomes desired to be realized for a certain object | type: FFS multiplicity: * isOrdered: F isUnique: F defaultValue: None isNullable: False |
| fulfillmentInfo | It describes status of fulfilment and the related reasons for that status. It can be applied to the intent, the intentExpectation or expectationTarget allowedValues: tuple of (fulfillmentStatus, fulfillmentProgressState, fulfillmentProgressState Reason) | type: fulfillmentInfo multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| fulfillmentStatus | It describes the current status of fulfilment for the intent, intentexpectation or expectationTarget. It is configured/written by MnS producer and can be read by MnS consumer. allowedValues: "FULFILLED", “NOT_FULFILLED Editor’s Note: whether other allowed values should be supported is FFS, and the name for the attribute intentFulfilStatus is FFS. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fulfillmentProgressState | It describes the current progress of or the reason for not achieving fulfilment for the intent, intentExpectation or expectationTarget. It is configured/written by MnS producer and can be read by MnS consumer. allowedValues: "ONGOING", "SUSPENDED", "CANCELED" "FULFILLMENTFAILED", | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| fulfillmentProgressState Reason | It describes the reasons/observations related to the specific noted fulfillmentProgressState allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: F isUnique: F defaultValue: None isNullable: False |
| intentContext | It describes the list of constraints and conditions that should apply for the entire intent even if there may be specific constraints and conditions defined for specific parts of the intent. allowedValues: triple of (attribute, condition, value range) | type: Context multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| expectationId | A user-friendly (and user assignable) name of the intentExpectation. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| expectionObject | expectation carries requirements (expectations, goals and constrains) on an expectationObject. expectionObject refers to an object (e.g. instance of managed object) to which the intentExpectation should apply. This means, which object the requirements specified by the expectation are meant for. expectionObject is optional. allowedValues: NA | type: DN multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| xpectionobjectContext | It describes the list of constraints and conditions to be used as filter information to identify the specific object to which a given intentExpectation should apply. Note there may be other constraints and conditions defined either for the entire intent, for the specific intentExpectation or for the expectationTarget of the considered intentExpectation. allowedValues: depends on Object in the IntentExpectation | type: Context multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| expectionTargets | It describes the list of specific outcomes on metrics and observables related to the Object (e.g. the metrics that characterize the performance of the object(s) or some abstract index that expresses the behavior of the object(s)) that are desired to be realized for a given intentExpectation. allowedValues: depends on Object in the IntentExpectation | type: expectationTarget multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| expectationContext | It describes the list of constraints and conditions that should apply for a specific intentExpectation. Note there may be other constraints and conditions defined for the entire intent or for specific parts of the intentExpectation. allowedValues: depends on Object in the IntentExpectation | type: Context multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| targetName | It describes a specific attribute of the object on which the outcomes are stated, either a configuration or observable of that object. The attributes may be a parameter, gauge, counter, KPI, weighted metric, etc. related to that object allowedValues: depends on Object in the IntentExpectation | type: string multiplicity: 1 isOrdered: False isUnique: False defaultValue: Null isNullable: True |
| targetCondition | It expresses the limits within which the targetName is allowed/supposed to be allowedValues: is equal to; is less than; is greater than; Note: Others conditions like "is within the range" or "is outside the range" can be expressed in terms of these basic conditions | type: enum multiplicity: upto 2 isOrdered: False isUnique: False defaultValue: "is equal to" isNullable: False |
| targetValueRange | It describes the range of values that applicable to the targetName and the TargetCondition. | type: FFS multiplicity: upto 2 isOrdered: False isUnique: False defaultValue: Null isNullable: True |
| targetContext | It describes the list of constraints and conditions that should apply for a specific expectationTarget. Note there may be other constraints and conditions defined for the entire intent or the intentExpectation. allowedValues: triple of (attribute, condition, value range) | type: Context multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |
| contextAttribute | It describes a specific attribute of or related to the object or to characteristics thereof (e.g. its control parameter, gauge, counter, KPI, weighted metric, etc) to which the expectation should apply or an attribute related to the operating conditions of the object (such as weather conditions, load conditions, etc). | type: DN multiplicity: 1 isOrdered: False isUnique: False defaultValue: Null isNullable: True |
| contextCondition | It expresses the limits within which the ContextAttribute is allowed/supposed to be allowedValues: is equal to; is less than; is greater than; Note: Others conditions like "is within the range" or "is outside the range" can be expressed in terms of these basic conditions | type: enum multiplicity: upto 2 isOrdered: False isUnique: False defaultValue: "is equal to" isNullable: False |
| contextValueRange | It describes the range of values that explicatable to the ContextAttribute and the ContextCondition. | type: FFS multiplicity: upto 2 isOrdered: False isUnique: False defaultValue: Null isNullable: True |

End of changes