# 3GPP TSG SA WG5 Meeting #142e	S5-222093

# Online, , 04 Apr 2022- 12 Apr 2022

# Source:	Samsung

# Title:	pCR 28.104 Paging Optimization

# Document for:	Approval

Agenda Item:	6.6.5

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

None

# 3	Rationale

This contribution provides solution for paging optimization capability.

# 4	Detailed proposal

First modification

#### 8.4.1.x	Paging Optimization

##### 8.4.1.x.1	MDA type

The MDA type for Capability-Paging Optimization: SLSAnalysis.PagingOptimization.

##### 8.4.1.x.2	Enabling data

The enabling data for paging optimization are provided in table 8.4.1.x.2-1.

Table 8.4.1.x.2-1: Enabling data for network slice traffic prediction analysis

| Data category | Description | References |
| --- | --- | --- |
| MDT Data | MDT reports with anonymous user identification (e.g C-RNTI) | MDT measurements defined in TS 32.422[6]. |
| Performance measurements | Measurement for 5G Paging from AMF | See clause 5.2.5.2 in TS 28.522[4]. |

##### 8.4.1.x.3	Analytics output

The specific information elements of the analytics output for paging optimization, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.2.5.3-1.

Table 8.4.1.x.3-1:  Analytics output for network slice load analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| UserPagingAnalytics | This specify the paging related analytics for a specific user | M | type: PagingAnalytics multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

Second modification

## 8.5	Data type definitions

### 8.5.y	PagingAnalytics <<dataType>>

#### 8.5.y.1	Definition

This data type specifies the paging related analytics for a specific user.

#### 8.5.y.2	Information elements

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| DailyOOCDuration | This specify the time window during which UE is out-of-coverage every day. | M | type: ProjectionDuration multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| DailyOOCLocation | This specify the last known location before UE goes out-of-coverage every day. | M | type: Integer multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| UserIdentification | This specify the user identification i.e C-RNTI | M | type: Integer multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |