3GPP TSG-SA5 Meeting #142-e 	S5-222262

# e-meeting, 4 - 12 April 2022

# Source:	Huawei

# Title:	Rapporteur clean up

# Document for:	Approval

Agenda Item:	6.6.3

1	Decision/action requested

The group is asked to discuss and approval.

2	References

[1]	3GPP draft TS 28.312: “Management and orchestration; Intent driven management services for mobile networks v1.0.0”.

3	Rationale

This contribution proposes to do the following rapporteur clean up:

- Add the missing reference for TS 28.541 and TS 28.622

-	 Change the capital of some attributes to lowercase

4	Detailed proposal

It proposes to make the following changes to TS 28.312[1].

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[3]	3GPP TS 28.532: " Management and orchestration; Generic management services".

[4]	3GPP TS 28.530: " Management and orchestration; Concept, use cases and requirements".

[X]	3GPP TS 28.541: " Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[Y]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM); Integration Reference Point (IRP); Information Service (IS)".

2nd Change

6.2.1.2.1.2	Attributes

The Intent IOC includes attributes inherited from TOP IOC (defined in TS 28.622 [Y]) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| intentExpectations | M | T | T | F | F |
| userLabel | M | T | T | F | F |
| intentContexts | O | T | T | F | F |
| intentFulfilStatus | M | T | F | F | T |

3rd Change

##### 6.2.1.3.3	ExpectationObject <<dataType>>

###### 6.2.1.3.3.1	Definition

The ExpectationObject <<dataType>> represents the Object of the IntentExpectation that are required to be applied on.

###### 6.2.1.3.3.2	Attributes

The ExpectationObject includes the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| objectType | CM | T | T | F | F |
| objectInstance | CM | T | T | F | F |
| objectContexts | O | T | T | F | F |

###### 6.2.1.3.3.3	Attribute constraints

| Name | Definition |
| --- | --- |
| objectType Support Qualifier | Condition: The intent expectation is not for a specific object instance or MnS consumer have no knowledge of the DN of this specific object instance. |
| objectInstance Support Qualifier | Condition: The intent expectation is for a specific object instance and MnS consumer have the knowledge of the DN of this specific object instance |

4th Change

6.4.1.3	ExpectationTargets

Following provides the concrete ExpectationTargets for Radio Network Expectation based on the common structure of ExpectationTarget. The properties of the attributes in the following table should be same with properties of ExpectationTargets defined in clause 6.2.1.3

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| weakRSRPRatioTarget | O | T | T | F | F |
| lowSINRRatioTarget | O | T | T | F | F |
| aveULRANUEThptTarget | O | T | T | F | F |
| aveDLRANUEthptTarget | O | T | T | F | F |
| lowULRANUEThptRatioTarget | O | T | T | F | F |
| lowDLRANUEThptRatioTarget | O | T | T | F | F |

4th Change

6.4.99 	Attribute definition

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| coverageAreaPolygonContext | It describes the coverage areas for the RAN SubNetwork that the intent expectation is applied in the form of polygon. CoverageAreaPolygonContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: -contextAttribute: "CoverageAreaPolygon" -contextCondition: "With the range" -contextValueRange: a list of CoverageArea defined in TS 28.541[X] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| coverageTACContext | It describes the coverage areas for the RAN SubNetwork that the intent expectation is applied in the form of TAC. CoverageTACContext is a Context including attributes: contextAttribute, contextCondition and contextValueRange. Following are the allowed values: -contextAttribute: "CoverageAreaTAC" -contextCondition: "With the range" -contextValueRange: a list of nRTAC defined in TS 28.541[X] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| plMNContext | It describes the PLMN(s) supported by the RAN SubNetwork that the intent expectation is applied. PLMNContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: -contextAttribute: "PLMN" -contextCondition:"With the range" -contextValueRange: a list of PLMNId defined in TS 28.541[X] | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| nRFqBandContext | It describes the nRFqBands supported by the RAN SubNetwork that the intent expectation is applied. nRFqBandContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: -contextAttribute: "NRFqBand" -contextCondition: "With the range" -contextValueRange: a list of NRFqBand expressed as string. Valid frequency band values are specified in sub-clause 5.4.2 in 3GPP TS 38.104 [Z]. | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| rATContext | It describes the RAT supported by the RAN SubNetwork that the intent expectation is applied. RATContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: -contextAttribute: "RAT" -contextCondition: "With the range" -contextValueRange: a list of ENUM with allowed value: UTRAN, EUTRAN and NR. | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| weakRSRPRatioTarget | It describes the downlink weak coverage ratio target for the RAN SubNetwork that the intent expectation is applied. WeakRSRPRatioTarget is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. Following are the allowed values: -targetName: "WeakRSRPRatio" -targetCondition: "is less than" -targetValueRange: integer with allowed value [0,100]. -targetContext: WeakRSRPContext | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| WeakRSRPRatioTarget.weakRSRPContext | It describes the threshold for downlink weak RSRP of the RAN SubNetwork that the intent expectation is applied. WeakRSRPContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: -contextAttribute: "WeakRSRPThreshold" -contextCondition: "is less than" -contextValueRange: Float. | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| LowSINRRatioTarget | It describes the low SINR ratio target for the RAN SubNetwork that the intent expectation is applied. LowSINRRatioTarget is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. Following are the allowed values: -targetName: "WeakRSRPRatio" -targetCondition: "is less than" -targetValueRange: integer with allowed value [0,100]. - targetContext: LowSINRContext | type:ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| LowSINRRatioTarget.lowSINRContext | It describes the threshold for low SINR for RAN SubNetwork that the intent expectation is applied. LowSINRContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: -contextAttribute: "LowSINRThreshold" -contextCondition: "is less than" -contextValueRange: integer. | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| aveULRANUEThptTarget | It describes the average UL RAN UE throughput target for RAN SubNetwork that the intent expectation is applied. AveULRANUEThptTarget is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. Following are the allowed values: -targetName: "AveULRANUEThpt" -targetCondition: "is greater than" -targetValueRange: integer | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| aveDLRANUEThptTarget | It describes the average DL RAN UE throughput target for RAN SubNetwork that the intent expectation is applied. AveDLRANUEThptTarget is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. Following are the allowed values: -targetName: "AveDLRANUEThpt" -targetCondition: "is greater than" -targetValueRange: integer | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| lowULRANUEThptRatioTarget | It describes the low UL RAN UE throughput ratio target for the RAN SubNetwork that the intent expectation is applied. LowULRANUEThptRatioTarget is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. Following are the allowed values: -targetName: "LowULRANUEThptRatio" -targetCondition: "is less than" -targetValueRange: integer with allowed value [0,100]. -targetContext: LowULRANUEThptContext | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| LowULRANUEThptRatioTarget.lowULRANUEThptContext | It describes the threshold for the low UL RAN UE throughput of the RAN SubNetwork that the intent expectation is applied LowULRANUEThptContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: -contextAttribute: "LowULRANUEThptThreshold" -contextCondition: "is less than" -contextValueRange: Float. | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| lowDLRANUEThptRatioTarget | It describes the low DL RAN UE throughput ratio target for the RAN SubNetwork that the intent expectation is applied. LowDLRANUEThptRatioTarget is an ExpectationTarget including attributes: targetName, targetCondition and targetValueRange. Following are the allowed values: -targetName: "LowDLRANUEThptRatio" -targetCondition: "is less than" -targetValueRange: integer with allowed value [0,100]. -targetContext: LowDLRANUEThptContext | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| owDLRANUEThptRatioTarget.lowDLRANUEThptContext | It describes the threshold for the low DL RAN UE throughput of the RAN SubNetwork that the intent expectation is applied LowDLRANUEThptContext is a Context including attributes: contextAtrribute, contextCondition and contextValueRange. Following are the allowed values: -contextAttribute: "LowDLRANUEThptThreshold" -contextCondition: "is less than" -contextValueRange: Float. | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| serviceStartTime | This describes the start time at which the service shall be available. This contributes to the selection of the appropriate edge data network to be used for service deployment. Following are the allowed values: -contextAttribute: "serviceStartTime" -contextCondition: "is equal than" -contextValueRange: start time stamp | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| serviceEndTime | This describes the end time after which the service shall not be available. This contributes to the selection of the appropriate edge data network to be used for service deployment. Following are the allowed values: -contextAttribute: "serviceEndTime" -contextCondition: "is equal than" -contextValueRange: end time stamp | type:Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| edgeIdenfiticationId | This identifies the edge network where the service need to be deployed. This should be used when the edge identification is known to the consumer Following are the allowed values: -contextAttribute: "edgeIdentificationId" -contextCondition: "is equal than" -contextValueRange: EDNidentifier as defined in 28.538. | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| edgeIdenfiticationLoc | This identifies the location where the service need to be deployed. This should be used when the edge identification is not known to the consumer Following are the allowed values: -contextAttribute: "edgeIdentificationTarget" -contextCondition: "is equal than" -contextValueRange: geographical target location. This will take a form of either single latitude & longitude or a TAI | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| dlThptPerUE | See clause 6.3.1 of TS 28.541 | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| UlThptPerUE | See clause 6.3.1 of TS 28.541 | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| dLLatency | See clause 6.3.1 of TS 28.541 | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| uLLatency | See clause 6.3.1 of TS 28.541 | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| coverageAreaTAList | See clause 6.3.1 of TS 28.541 | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| uEMobilityLevel | See clause 6.3.1 of TS 28.541 | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| resourceSharingLevel | See clause 6.3.1 of TS 28.541 | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| maxNumberofUEs | See clause 6.3.1 of TS 28.541 | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| activityFactor | See clause 6.3.1 of TS 28.541 | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| uESpeed | See clause 6.3.1 of TS 28.541 | type: Context multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |

End of Changes