3GPP TSG-SA5 Meeting #142-e S5-222314 # e-meeting, 4 - 12 April 2022 CR-Form-v12.1. CHANGE REQUEST. 32.277; CR; rev; -; Current version:; 17.1.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests..

| Proposed change affects: |  |  | UICC apps |  |  |  |  | ME |  | Radio Access Network |  |  |  |  |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Introduction of Detailed message format for converged charging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  |  |  |  |  | Date: |  |  | 2022-3-15 |  |  |  |
| Category: | B |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-17 |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |  |  |  |
| Reason for change: |  | The charging information over Nchf for 5G ProSe converged charging is missing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce 5G ProSe information over Nchf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No charging for 5G ProSe. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x.3(new) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| Other comments: |  | This CR needs to be implemented after CR#00xy (inside chapter 6.x of CR#00xy_S5-222313) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1st modified section

6.x.3	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data for 5G ProSe converged charging.

The Operation types are listed in the following order: I [Initial] / U (Update)/T [Termination]/E [event]. Therefore, when all Operation types are possible it is marked as IUTE. If only some Operation types are allowed for a node, only the appropriate letters are used (e.g. IUT or E) as indicated in the table heading. The omission of an Operation type for a particular field is marked with "-" (e.g. I-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.x.3.1 illustrates the basic structure of the supported fields in the Charging Data Request for 5G ProSe converged charging.

Table 6.x.3.1: Supported fields in Charging Data Request message

| Information Element | Node Type | Direct Discovery | Direct Communication |  |
| --- | --- | --- | --- | --- |
|  | Supported Operation Types | I/U/T/E | I/U/T/E |
| Session Identifier |  | ITE | ITE |  |
| Subscriber Identifier |  | ITE | ITE |  |
| NF Consumer Identification |  | ITE | ITE |  |
| Invocation Timestamp |  | ITE | ITE |  |
| Invocation Sequence Number |  | ITE | ITE |  |
| Retransmission Indicator |  | --- | --- |  |
| One-time Event |  | --E | --E |  |
| One-time Event Type |  | --E | --E |  |
| Notify URI |  | I-- | I-- |  |
| Triggers |  | ITE | ITE |  |
| Multiple Unit Usage |  | ITE | ITE |  |
| 5G ProSe Charging Information |  |  |  |  |

Table 6.x.3.2 illustrates the basic structure of the supported fields in the Charging Data Response for 5G ProSe converged charging.

Table 6.x.3.2: Supported fields in Charging Data Response Message

| Information Element | Node Type | Direct Discovery | Direct Communication |  |
| --- | --- | --- | --- | --- |
|  | Supported Operation Types | I/U/T/E | I/U/T/E |
| Session Identifier |  | ITE | ITE |  |
| Subscriber Identifier |  | ITE | ITE |  |
| NF Consumer Identification |  | ITE | ITE |  |
| Invocation Timestamp |  | ITE | ITE |  |
| Invocation Sequence Number |  | ITE | ITE |  |
| Retransmission Indicator |  | --- | --- |  |
| One-time Event |  | --E | --E |  |
| One-time Event Type |  | --E | --E |  |
| Triggers |  | ITE | ITE |  |
| Multiple Unit Usage |  | ITE | ITE |  |
| 5G ProSe Charging Information |  |  |  |  |

End of changes