3GPP TSG-SA5 Meeting #142-e 	S5-222355

# e-meeting, 4 - 12 April 2022

# Source:	Nokia

# Title:	Enhance the description of the requirement of network slice thourhgout analysis

# Document for:	Approval

Agenda Item:	6.6.5

# 1	Decision/action requested

This contribution is for approval.

# 2	Rationale

One of the requirements of “network slice throughput analysis” REQ-THR_MDA-04 discusses the capability of MDA to prompt when the network slice throughput exceeds or falls below a certain threshold.

But it is not clear whether the prompt is on the predicted network slice throughput or the currently observed network slice throughput. In case the intention is to prompt based on the current value, MDA services may not be needed as corresponding Managed Entity can perform that.

In case the intenion is to prompt based on the predicted network slice throughput, then it must be explicitly mentioned as the current text is ambiguous.

# 3	Detailed proposal

1st Modified Section

#### 7.2.2.2	Network slice throughput analysis

##### 7.2.2.2.1	Description

This MDA capability is for  the network slice throughput analysis.

##### 7.2.2.2.2	Use case

Throughput is of great importance which represents the end users' experiences and also reflects the network problems, e.g., low UE throughput may be caused by the resource shortage. In order to satisfy the requirements of dL/ulThptPerSlice in the ServiceProfile, MDAS may be utilized for throughput related analysis/predictions for network slice instance.

MDAS producer should have the capability to receive the request from the consumer to analyse the network slice throughput related issues and identify the root cause to assist throughput assurance. Network slice throughput analysis can be for a specific domain or for cross-domain. The two levels of MDAS producers worked in a coordinated way to assure the throughput performance. The producer of MDAS is able to provide the MDA report including the network slice throughput analytics output.

##### 7.2.2.2.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-THR_MDA-1 | MDA capability for network slice throughput analysis shall be able to identify the network slice throughput issue, including, RAN issue, CN issue. | Network slice throughput analysis |
| REQ-THR_MDA -2 | MDA capability for network slice throughput analysis shall be able to provide the root cause analysis of the network slice throughput issue. | Network slice throughput analysis |
| REQ-THR_MDA -3 | MDA capability for network slice throughput analysis shall be able to provide the analytics output of the network slice throughput which contain the following information: - Network slice throughput statistics; - Network slice throughput predictions. | Network slice throughput analysis |
| REQ-THR_MDA-04 | MDA capability for network slice throughput analysis shall be able to provide the prompt when the predicted network slice throughput exceeds or falls below a certain threshold. | Network slice throughput analysis |

End of Modified Sections