3GPP TSG-SA5 Meeting #142-e 	S5-222359

# e-meeting, 17 -26 January 2022

# Source:	China Unicom

# Title:	Add potential solution for performance measurements without PLMN ID at NRcellCU

# Document for:	Approval

Agenda Item:	6.5.13

1	1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	3GPP TR 28.835 v0.0.0: “Management Aspects of 5G MOCN Network Sharing Phase2”

3	Rationale

It was approved to add potential solution for performance measurements without PLMN ID at NRcellCU. This contribution proposes to add OperatorCU and NROperatorCU IOCs to support performance measurements without PLMN ID for 5G MOCN network sharing.

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]                        3GPP TS 28.541: "5G Network Resource Model (NRM); Stage 2 and stage 3".

2nd Change

### 4.Y.2	Potential solution

The following enhanced NR NRM solution is proposed:

Introduce new OperatorCU <<IOC>> name-contained by GNBCUCPFunction<<IOC >> to represent operator specific gNB level information (including operator specific gNB Id, gNBIdlength, pLMNId), and new NROperatorCellCU<<IOC>> name-contained by OperatorCU<<IOC>> to represent operator specific cell level information (including operator specific CelllocalId, PLMNInfoList). Which means individual OperatorCU instance and NROperatorCellCU needs to be created and configured for each POP.

Figure 4.Y.2-1 NR NRM enhancement to suport NG-RAN sharing MOCN network sharing with multiple cell identity broadcast feature

The OperatorCU <<IOC>> contains attributes to support operator specific gNB-CU level information (including gNB Id, gNBIdLength, PLMNId) to support 5G Multi-Operator Core Network (5G MOCN) network sharing with multiple cell identity broadcast feature. Individual instance of OperatorCU <<IOC>> should be created and configured for each POP. When configured the attributes override those in parent GNBCUCPFunction instance.

The OperatorCU<<IOC>> is only used to support MOCN with multiple cell identity broadcast feature. If the MOCN with multiple Cell Identity broadcast feature is not supported, this IOC is not used.

The OperatorCU<<IOC>> includes attributes inherited from TOP IOC  and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| gNBId | M | T | T | F | T |
| gNBIdLength | M | T | T | F | T |
| pLMNId | M | T | T | F | T |

NROperatorCellCU<<IOC>>

The NROperatorCellCU <<IOC>> contains attributes to support operator specific cell level information (including cellLocalId, pLMNInfoList) to support 5G Multi-Operator Core Network (5G MOCN) network sharing with multiple Cell Identity broadcast feature. Individual instance of NROperatorCellCU <<IOC>> should be created and configured for each POP. When configured the attributes override those in associated NRCellCU instance.

The NROperatorCellCU <<IOC>> is only used to support MOCN with multiple cell identity broadcast feature. If the MOCN with multiple cell identity broadcast feature is not supported, this IOC is not used.

The NROperatorCellCU <<IOC>> includes attributes inherited from TOP IOC  and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | M | T | T | F | T |
| pLMNInfoList | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| nRCellCURef | M | T | T | F | T |

The attributes of the NRCellCU<<IOC>> in TS 28.541[2] need to be modified. The changes of support qualifier in cellLocalId and PLMNInfoList are proposed as follows.

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellLocalId | CM | T | T | F | T |
| pLMNInfoList | CM | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| nRFrequencyRef | M | T | F | F | T |
| NOTE: Void |  |  |  |  |  |

End of Changes