3GPP TSG-SA5 Meeting #142-e 	S5-222436

# e-meeting, 4 - 12 April 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.256 | CR | draftCR | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcting support of charging scenarios for roaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-03-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current specification only states that in the case of roaming the AMF (in VPLMN) shall support PEC towards the CHF in VPLMN. There is however no statement that it cannot support IEC or ECUR, only that it may support these towards the CHF in HPLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying that the AMF in VPLMN may to use IEC and ECUR handling towards the CHF in VPLMN, but that must support having PEC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no requirement on what the AMF shall support at roaming towards the HPLMN, if it supports interaction with the CHF in HPLMN, and therefore there might be interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 5.2.2.5.1	General

In roaming, for registration management charging.

-	if the AMF in VPLMN supports interaction with one CHF it:

-	shall support PEC scenario with V-CHF in VPLMN

-	may support IEC or ECUR scenario with V-CHF in VPLMN

-	if the AMF in VPLMN supports interaction with two CHFs it:

-	shall support PEC scenario with H-CHF in HPLMN and V-CHF in VPLMN

-	may support IEC or ECUR scenario with H-CHF in HPLMN and V-CHF in VPLMN.

If the AMF in VPLMN interacts with two CHFs it may use the same or different converged charging scenarios towards H-CHF in HPLMN and V-CHF in VPLMN.

End of changes