3GPP TSG SA WG5 Meeting 142-e		S5-222500

# e-meeting, 4 -12 April 2022

# Source:	Nokia

# Title:	Problems in MDA output data

# Document for:	Discussion

Agenda Item:	6.6.5

# 1	Decision/action requested

The group is asked to discuss and endorse section 4.

# 2	References

[x]	TR 28.809 Study on enhancement of management data analytics

# 3	Rationale

In [x] 3GPP SA5 introduced several use cases for MDA that relate to specific MDA output. MDA output may represent statistics or predictions among other MDA output information elements. Our observation is the fact that in several cases the way statistics and predictions are not described sufficiently. There are cases where the time-period is just mentioned, e.g., in:

- Table 8.4.2.1.3-1: ServiceExperienceStatistics and ServiceExperiencePredictions,

- Table 8.4.2.2.3-1 NetworkSliceThroughputTimeStatistics NetworkSliceThroughputUserPredictions, NetworkSliceThroughputTimePredictions

- Table 8.4.2.5.3-1NetworkSliceLoadDistribution

For theses case the MDA MnS consumer has no clue for which time period the MDA output is about. Also, the MDA MnS consumer ha no indication of the quality of the statistics or perdition. In TR 28.809 the notion of confidence degree was introduced but in TS 28.104 it was not adopted any more.

Furthermore, certain analytics in a set of MDA output related to a MDA capability, i.e., use case, are simple statistics or predictions of an existing KPI for example NetworkSliceThroughputTimeStatistics in Table 8.4.2.2.3-1. Why we need to invent a new name and we do not use the one in TS 28.554 stating that this is statistics or predictions related to that particular KPI? The same thing may apply for PM related statistics or predictions. We believe that is clearer to adopt the existing PM/KPI names stating statistics or predictions. Otherwise, this may be confusing.

# 4	Detailed proposal

Observation: Certain analytics are not defined properly, i.e., have no time period related or validity time and have no related quality indication.

Proposal: Introduce a common way to provide time period and analytics quality related to the MDA output.

Observation: Certain MDA output information elements belonging in a set of data related to an MDA capability or use case, are simply KPI or PM statistics or predictions.

Proposal: Use the same PM or KPI name stating statistics or prediction in order to make clear the relation with the original PM and KPI.