3GPP TSG-SA5 Meeting #144-e 	S5-224103

# e-meeting, 27 June - 1 July 2022

# Source: 	Huawei

# Title:	pCR TR 28.908 Add use case on AI/ML deployment management

# Document for:	Approval

Agenda Item:	6.7.5.4

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

References

[1]		SP-211443 New Study on AI/ ML management

[2]		3GPP TR 28.908-020 Management and orchestration; Study on Artificial Intelligence/Machine Learning (AI/ML) management

[3]		3GPP TS 28.813 Management and orchestration; Study on new aspects of Energy Efficiency (EE) for 5G

3	Rationale

The approved new SI [1] proposed to study the AI/ML management capabilities and management services to support/coordinate AI/ML in 5GS (3GPP management system, 5GC and NG-RAN). At the beginning of the phase, AI/ML deployment management need to be considered so as to making an AI/MLenabled function available in the operational environments.

As described in clause 4.7.3 in TR 28.813[3], to provide centralized ES for RAN domain area, 3GPP management system performs the functionalities as monitoring, analysis, decision, execution and evaluation. With AI technology, 3GPP management system can provide more efficient ES for RAN domain area while keeping basic KPIs stable for SLA assurance. In order to support RAN domain Energy saving management scenario, the AI/ML deployment management for RAN domain ES need to be studied.

This contribution proposes to add a use case on AI/ML deployment management.

4	Detailed proposal

It is proposed to add the following chapter in TR 28.908 [2].

1st modified section

5.X	AI/ML deployment management for RAN domain ES

5.X.1	Description

At the beginning of the AI/ML enabled function operation, enabling the AI/ML entities should be controlled by AI/ML management. As described in clause 4.7 in TR 28.813[3], RAN domain ES can use AI to formulate energy saving solutions. Therefore, the AI/ML entities which enabled RAN domain ES function should be controlled by AI/ML management. The AI/ML entities need to be activated to enable RAN domain ES function. The AI/ML entities activating can be triggered by the AI/ML MnS consumer or initiated by the AI/ML MnS producer.

5.X.2.1	AI/ML entities activating initiated by consumer

The AI/ML entities activating may be triggered by the AI/ML MnS consumer of Cross domain management. AI/ML MnS Consumer monitor network performance and determine whether to activate the AI/ML entities. For example, for ES purpose, AI/ML MnS Consumer collects the information of the capacity booster cells and coverage cells inside the RAN domain area, then makes the decision for activation AI/ML enabled RAN domain ES function. In this case, the consumer can request the AI/ML entities to be activated so as to get better network performance.

5.X.2.2	AI/ML entities activating initiated by producer

The AI/ML entities activating may be initiated by the AI/ML MnS producer. MnS producer can determine whether to activate AI/ML entities based on network performan and service requirements. For example, after receiving an RAN domain ES requirement, the producer can decide to activate the AI/ML enabled RAN domain ES function. In this case, the producer responsible for AI/ML management shall have a capability to activate the AI/ML entities and inform an authorized consumer about the activation status.

5.X.3	Potential requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-AIML_Deploy-CON-1 | The AI/ML MnS producer should have a capability to allow the authorized consumer to activate AI/ML entities. | AI/ML entities activating initiated by consumer |
| REQ-AIML_Deploy-CON-2 | The AI/ML MnS producer responsible for AI/ML management shall have a capability to activate the AI/ML entities and inform an authorized consumer about the activation status. | AI/ML entities activating initiated by producer |

5.X.4	Possible solutions

TBD

End of modified section