3GPP TSG-SA5 Meeting #144-e 	S5-224141rev1

- e-meeting, 27th June– 1st July 2022

- Source:	Huawei

- Title:	Add the QoS solution for CHF rating

- Document for:	Discussion

Agenda Item:	7.5.2

# 1	Decision/action requested

Discuss the solution for CHF rating.

# 2	References

[1]	3GPP TS 32.291 5G system, charging service;Stage 3

# 3	Rationale

Comprehensively consider the input information for CHF rating consideration.

# 4	Detailed proposal

1st change

#### 5.3.5.x	Solutions #3.x The QoS information in RSU as the input for CHF rating

A possible solution for key issues 3a, 3b, and 3c, enhancement of input to CHF rating.

The possible solution supports the potential requirements REQ-3GPPCH-ER-01 and partial supports Key Issue #3a, #3b and #3c to describe the enhancement of input to CHF rating.

The QoS information is the measurement of the overall performance of a service. A solution could be to allow the QoS information in the RequestedUnit as well as the rating group in the request for quota specified in the TS 32.291[x], as the input for CHF rating consideration, to be able to assist to allocate the right amount of quota needed at that moment for that rating group.

###### 6.1.6.2.1.9	Type RequestedUnit

Table 6.1.6.2.1.9-1: Definition of type RequestedUnit

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| time | Uint32 | OC | 0..1 | This field holds the amount of requested time. |  |
| totalVolume | Uint64 | OC | 0..1 | This field holds the amount of requested volume in both uplink and downlink directions. |  |
| uplinkVolume | Uint64 | OC | 0..1 | This field holds the amount of requested volume in uplink direction. |  |
| downlinkVolume | Uint64 | OC | 0..1 | This field holds the amount of requested volume in downlink direction. |  |
| serviceSpecificUnits | Uint64 | OC | 0..1 | This field holds the amount of requested service specific units. |  |
| QosInformation | QosData | OC | 0..1 | This field describes the QoS applied for the RG. | CHFRatingInput |
| NOTE 1: if none of them is included, "RequestedUnit": {}, the category and amount is determined by CHF for online charging with centralized unit determination and rating scenario. |  |  |  |  |  |

End of change