3GPP TSG-SA5 Meeting #144-e 	S5-224142rev2

- e-meeting, 27th June– 1st July 2022

- Source:	Huawei

- Title:	Add the general for NB mechanism enhancement

- Document for:	Discussion

Agenda Item:	7.5.2

# 1	Decision/action requested

Add the general for NB mechanism enhancement.

# 2	References

[1]	3GPP TR 28.826 Study on Nchf charging services phase 2 improvements and optimizations

# 3	Rationale

Add the general description for NB mechanism enhancement.

# 4	Detailed proposal

First change

## 5.5	Non-blocking mechanism enhancement

### 5.5.1	General

The non-blocking mode scenario is introduced for the 5G charging in clause 5.3.2 Charging scenarios of TS 32.290 [x]. The Converged Charging service offers charging with quota management, including the support for both blocking mode and non-blocking mode. The non-blocking charging mode allow the service delivery before sending the request to the CHF for the service to be granted authorization to start. In TS 23.503 [3], the "Service Data flow handling while requesting credit" in cluase 6.3 Policy and charging control rule is introduced for indicating whether the service data flow is allowed to start while the SMF is waiting for the response to the credit request (Values: blocking or non-blocking).

In the TS 23.501[x], when the PCC Rule attribute Service Data flow handling while requesting credit indicates "non-blocking", the SMF shall request the report of the relevant usage information for the Charging key and Sponsor Identity (if applicable) and provide a default threshold value to the UPF while waiting for the quota from the CHF.

End of change