3GPP TSG SA WG5 Meeting 144-e	S5-224249rev1

- electronic meeting, online, 27th June – 1st July 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | draftCR | rev |  | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add QMC job (stage 3 YANG) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eQoE |  |  |  |  |  | Date: |  |  | 2022-06-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Submit stage3 (YANG) for QMC job. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add QoE measurement Collection Job, stage3 (YANG). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Functionality would be missing in stage3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | D.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR … CR… |  |  |
| Other comments: |  | Forge link: https://forge.3gpp.org/rep/sa5/MnS/-/tree/28.623_CR_Add_QMC_Job. Stage3 changes are per S5-223511 (28.622, CR0165). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## D.2.x	module _3gpp-common-qmcjob.yang

<CODE BEGINS>

module _3gpp-common-qmcjob {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-qmcjob";

prefix "qmc3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Quality of Experience Measurement Collection Job handling";

reference "3GPP TS 28.623

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.622

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)";

revision 2022-04-29 { reference "S5-224249"; }

revision 2022-04-29 { reference "CR-0171"; }

revision 2022-03-22 { reference "draftCR"; }

revision 2022-01-06 { reference "CR-0142"; }

grouping QMCJobGrp {

leaf serviceType {

description "Specifies an end user service type for QoE measurements.";

mandatory true;

type enumeration {

enum DASH;

enum MTSI;

enum VR;

}

}

leaf areaScope {

type string;

description "Defines the area in terms of NR cells or Tracking Area where the QMC shall take

place.";

}

leaf qoECollectionEntityAddress {

type union {

type inet:uri;

type inet:ip-address;

}

mandatory true;

description "Specifies the address to which the QMC records shall be transferred. Ipv4 or,

Ipv6 address(es) may be used.";

}

list pLMNTarget {

min-elements 0;

max-elements 1;

description "Defines the PLMN for which sessions shall be selected in the network request

session in case of area based QMC when several PLMNs are supported in the RAN (this means

that shared cells and not shared cells are allowed for the specified PLMN. Furthermore,

several PLMNs can be used for not shared RAN cases as well as for shared RAN cases.).

Only the sessions may be selected where the PLMN that the UE reports as selected PLMN is

the same as the PLMN Target.";

uses types3gpp:PLMNId;

key "mcc mnc";

}

leaf qMCTarget {

description "Specifies whether the QMC is area based or individual UE based.

allowedValues: AREA_BASED, UE_BASED ";

type enumeration {

enum AREA_BASED;

enum UE_BASED;

}

mandatory true;

}

leaf qOEReference {

type string;

mandatory true;

description "Specifies the network request session. The QoE reference shall be globally

unique therefore it is composed as follows:

MCC+MNC+QMC ID, where the MCC and MNC are coming with the QMC activation request from the

management system to identify one PLMN containing the management system, and QMC ID is a 3

byte Octet String.  The QMC ID is generated by the management system or the operator.

Used to identify the QoE measurement collection job in the traffic nodes and in the

measurement collection entity.";

}

list sliceScope {

min-elements 0;

description "Contains a list of S-NSSAIs (Single Network Slice Selection Assistance

Information).  A Network Slice end to end is identified by S-NSSAI ";

key "sd sst";

uses types5g3gpp:SNssai;

}

leaf qMCConfigFile {

type string;

description "Provides a reference to a file including the parameters for configuration of

application layer measurements, known as Container for Application Layer Measurement

Configuration.";

mandatory true;

}

}

grouping QMCJobSubTree {

description "Helps with augmenting QMBJob into multple places.";

list QMCJob {

description "Represents the QMCJob IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses QMCJobGrp;

}

}

}

augment "/subnet3gpp:SubNetwork" {

uses QMCJobSubTree;

}

augment "/me3gpp:ManagedElement" {

uses QMCJobSubTree;

}

}

<CODE ENDS>

End change