3GPP TSG-SA5 Meeting #144-e 	S5-224263

- e-meeting, 27 June - 1 July 2022

- Source:	Huawei, Deutsche Telekom, Nokia

- Title:	Update issue for intent confliction

- Document for:	Approval

Agenda Item:	6.7.3.2

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP draft TR 28.912: “Management and orchestration; Study on enhanced intent driven management services for mobile networks v0.2.0”.

[2]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks"

# 3	Rationale

This contribution proposes to update the description for Issue#4.2:	Intent conflicts and intents containing contradictions in the following aspects:

-	Clearly differentiate the following three intent conflict scenarios:

-  Conflict between two or more expectation targets within the same intent expectation.

-  Conflict between two or more multiple intent expectations with the same intent

- Conflict between two or more intents.

-	Alignment the terminologies with TS 28.312.

# 4	Detailed proposal

1st Change

## 4.2	Issue#4.2:	Intent conflicts

### 4.2.1	Description

The MnS consumer may create an intent containing two or more intent expectations, and each intent expectation may contain multiple expectation targets. For example, a Radio Network related intent may express an intent with targets on radio network parameters (like, downlink transmit power, remote electrical tilt) or on KPIs (like, DL UE throughput target, average RSRP target, coverage area). On receiving and after analysing the intent, the MnS producer may realize that the intent expectations or expectation targets in one intent are contradicted, i.e. the MnS producer may detect conflicts in the intent. Also an intent is considered to have conflict with other intents if the requirements (includes intent expectation and corresponding expectation targets) stated in one intent is conflicted with the requirements (includes intent expectation and corresponding expectation targets) stated in another intent. Following are the intent related conflict scenarios:

- Target conflict, which represents the conflict between two or more expectation targets within the same intent expectation.

- Expectation conflict, which represent the conflict between two or more intent expectations with the same intent

- Intent conflict, which represent the conflict between two or more different intents.

For example, consider two targets target_1=: throughput > threshold_1 and target_2 :=  interference <  threshold_2, and while trying to achieve target_1, target_2 gets degraded, so the producer will see that the targets are conflicting. The two are intent conflict if they are in different intents, but they are expectation conflict or target conflict if they are in the same intent.

When such conflicts are detected, the MnS producer needs to notify the MnS consumer about the conflict, indicating the intent, intent expectations or expectation targets which give rise to the conflict. Additionally, the MnS producer may also notify the MnS consumer about the additional information (e.g. the impact for other expectation targets when fulfill the specified expectation targets in the same or different intent) for the conflict.

Thereby, the MnS consumer may task actions (e.g. modify and delete the intent or intent expectation or expectation targets,) to address such intent conflict or MnS consumer may give some intent conflict handling guidelines (e.g. assign priority for such intent or intent expectation or expectation targets) to MnS producer to solve such intent conflict or eliminate the terminated state which is caused by the reason of conflict detected.

REQ-Intent_conflict-CON-1: The intent driven MnS should have the capability to inform the authorized MnS consumer about intent related conflicts, including intent conflict, expectation conflict and target conflict.

REQ-Intent_conflict-CON-2: The intent driven MnS should have the capability allow the authorized MnS consumer to give intent conflict handling guidelines to MnS producer to solve such intent conflict and potentially affect the terminated state which is caused by the reason of conflict detected.

### 4.2.2	Potential Solution

The MnS producer should notify the MnS consumer whenever such a conflict is detected with intent, intent expectations or expectation targets specified which give rise to the conflict. Additionally, the intent progress status should be marked as terminated with the reason as conflict detected.  Also MnS consumer can give intent conflict handling guidelines to MnS producer to solve such intent conflict and potentially affect terminated state which is caused by the reason of conflict detected.

End of changes