- 3GPP TSG-SA5 Meeting #	S5- 225042

- August 2022, E-meeting

- Source:	Nokia

- Title:	pCR 28.908 Use case on capability update

- Document for:	Approval

- Agenda Item:	6.7.5.7

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.908-010 “Study on Artificial Intelligence/Machine Learning (AI/ ML) management”.

# 3	Rationale

Depending on their configurations, AI/ML-enabled functions may learn new characteristics even during their utilization. his knowledge may be available to use but the AI/ML-enabled function may not be configured to apply this knowledge by default and but to only apply it when explicitly triggered This pCR is to add the UC and potential requirements on consumer requested capability update.

# 4	Detailed proposal

Start of modification

# 5	Use cases, potential requirements and possible solutions

# 5	Use cases, potential requirements and possible solutions

## 5.9	AI/ML update management

### 5.9.1	Description

Due to the complexity and time-varying nature of network, the AI/ML entities previously deployed may no longer be applicable to the current network after running for a period of time. Typically, the performance of a trained model may degrades over time. The AI/ML entitie needs to be updated timely to ensure the performance of inference and analysis.

### 5.9.2	Use cases

#### 5.9.2.2	AI/ML entities updating initiated by producer

The AI/ML entity updating may be initiated by the AI/ML MnS producer. In order to keep the model at a requested level, the AI/ML MnS producer may periodically conduct AI/ML retraining with new available training data. Once a new version AI/ML entity is obtained after the training is finished, it can be used to update the current AI/ML entity with this new version. In another condition, the AI/ML MnS producer may initiate AI/ML entity updating based on the running model performance. For example, if the performance of the running AI/ML model is decreased under a predefined threshold, the AI/ML MnS producer may decide to start ratraining and then update the AI/ML entity to a new version which performs better.

#### 5.9.2.N	Availability of new capabilities or MLEntities

Depending on their configurations, AI/ML functions may learn new characteristics even during their utilization, e.g. if they are configured to learn through reinforcement learning or if they are configured to download new versions of their MLEntities. This knowledge may be available to use but the AI/ML inference function may not be configured to apply this knowledge by default and but to only apply it when explicitly triggered. In such cases, the consumer of AI/ML services that are exposed by the AI/ML inference function (e.g. the operator, a management function, or a consumer network function) may wish to be informed when such capabilities are available.

#### 5. 9.2.M	requesting ML Entity update

When the capabilities of AI/ML inference functions degenerate, the typical action may be to trigger  re-training of the constituent MLEntities. It may be the case, however, that the AI/ML inference function only has inference capabilities and is not equipped to retrain its constituent MLEntities. Yet the consumer of AI/ML services may still need to request for improvements in the capabilities of the AI/ML inference function. In such cases, the consumer of AI/ML services that are exposed by the AI/ML inference function (e.g. the operator, a management function, or a consumer network function) may wish to trigger the MLEntities of the AI/ML inference function to be updated even without knowing what internal actions the AI/ML-enabled functions shall take.

The consumer needs to request the AI/ML inference function to update its constituent capabilities or MLEntites and the AI/ML inference function shall respond accordingly. For example, the AI/ML inference function may download new software that comes with the required updates, download from a remote server a file containing configurations and parameters for the one or more MLEntities, or it may trigger one or more remote or local AI/ML-related processes (including training, testing, etc.) needed to generate the required updates.

#### 5.9.2.L	Control of ML Entity update

Given the capability for an AI/ML inference function to receive and execute updates for MLEnties, a consumer may wish to manage that capability and its related process(es) even without knowing the details about the MLEntities being updated. For example, the operator may wish to define policies on how often the AI/ML capabilities may be updated. Alternatively, for a specific a request to update and the respective update process, the operator may wish to suspend or restart the capability update or to adjust the capability update conditions or characteristics, e.g., the times when the update may be executed. Correspondingly, the 3GPP management system needs to provide the capability to allow the AI/ML capability update process to be configured and managed.

### 5.9.3	Potential requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-AIML_UPD-CON-3 | The AI/ML MnS producer should have a capability to update the AI/ML entities and inform an authorized consumer about the update status. | AI/ML entities updating initiated by producer (clause 5.X.2.2) |

REQ-AI/MLUPDATE-1 the 3GPP management system shall have a capability for the provider of AI/ML inference to inform an authorized consumer of the availability of new AI/ML capabilities or MLEntities (e.g. as learning through a training process or as provided via a software update) and the readiness to update the AI/ML capabilities of the respective network function when triggered.

REQ-AI/MLUPDATE-2 the 3GPP management system shall have a capability for the provider of AI/ML inference to inform an authorized consumer of the different versions of newly available AI/ML capabilities or MLEntities that are candidates for applying towards updating the AI/ML capabilities of a network function when triggered.

REQ-AI/MLUPDATE-3 the 3GPP management system shall have a capability for the provider of AI/ML inference to inform an authorized consumer of the expected performance gain if/when the AI/ML capabilities or MLEntities of the respective network function are updated with/to the specific set of newly available AI/ML capabilities.

REQ-AI/MLUPDATE-4 the 3GPP management system shall have a capability to allow an authorized consumer to request the provider of AI/ML inference to update its MLEntities without stating the specific AI/MLEntity to be updated.

REQ-AI/MLUPDATE-5 the 3GPP management system shall have a capability to allow an authorized consumer to request the provider of AI/ML inference to update its MLEntities using a specific version of newly available AI/ML capabilities or MLEntities.

REQ-AI/MLUPDATE-6 the 3GPP management system shall have a capability to allow an authorized consumer to request the provider of AI/ML inference to update its MLEntities with specific performance conditions including a specific performance gain that must be met for the respective capability update

REQ-AI/MLUPDATE-7 the 3GPP management system shall have a capability for the provider of AI/ML inference to inform an authorized consumer about of the process or outcomes related to any request for updating the AI/ML capabilities or MLEntities.

REQ-AI/MLUPDATE-8 the 3GPP management system shall have a capability for the provider of AI/ML inference to inform an authorized consumer about of the achieved performance gain following the update of the AI/ML capabilities of a network function with/to the specific newly available MLEntities or set of AI/ML capabilities

REQ-AI/MLUPDATE-9 the 3GPP management system shall have a capability for an authorized consumer (e.g. an operator or the function/entity that generated the request for updating the AI/ML capabilities) to manage the request and subsequent process, e.g. to suspend, re-activate or cancel the request or process; or to adjust the characteristics of the capability update.

### 5.9.4	Possible solutions

TBD

End of modifications