- 3GPP TSG-SA5 Meeting #	S5-225054

- August 2022, E-meeting

- Source:	Nokia

- Title:	pCR 28.864 Potential solution on service usage of NWDAF

- Document for:	Approval

- Agenda Item:	6.7.6.2

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.864-020 “Study on Enhancement of the management aspects related to NWDAF”.

# 3	Rationale

This pCR is to add a potential solution on Performance Management of NWDAF related to service usage.

# 4	Detailed proposal

Start of modification

# 4.3	Key Issue #3: Performance Management of the NWDAF on the Interaction Aspect

### 4.3.1	Description

The NWDAF was firstly introduced in Rel-15 and it follows the SBA concept. The interaction between the producer and consumer of the services provided by NWDAF are defined in TS 23.288 [2].

As the requirement on network automation kept increasing, the NWDAF is expected to provide a variety of services to the other NFs and play a very important role in the future. Moreover, in the scenario where multiple NWDAFs are deployed, the NWDAF is also able to act as an Aggregator NWDAF to interact with one to multiple other NWDAFs.

From the perspective of management, it is necessary for the operator to monitor the performance of NWDAF based on the interaction between the producer and consumer of the services provided by the NWDAF. The interaction may include the requests, subscriptions, responses and notifications received and/or generated by NWDAF.

However, in the management domain, the performance management of NWDAF on this aspect is missing.

In this key issue, the potential solution(s) is provided on the metrics and KPIs for performance management of the NWDAF based on the interaction between the producer and consumer of the services provided by the NWDAF.

### 4.3.2	Potential solutions

#### 4.3.2.N	Potential solution #N:  correctness of service usage

##### 4.3.2.2.1	Introduction

This solution is proposed on how to provide the performance measurements of the NWDAF service usage. The proposed performance measurements are defined as a bolean that expresses whether the NWDAF is observing the “Expected service usage” or “Abnormal service usage”. “Expected service usage” represents the case where the distribution of requests is similar distribution of requests observed during normal operations, otherwise “Abnormal service usage” represents the case where there mare major variations in the number of requests, e.g., where the there is a denial-of-service attack that increases the number of requests.

##### 4.3.2.2.1	Description

The solution proposes to use as input the requests received by an NWDAF service producer, and to produces an analysis of the expected incoming requests distribution.

Then at any point during operations, the solution may take as input the expected behaviour of the service producers’ incoming requests and provides a comparison between prior-to established incoming requests distribution and the distribution of the arriving service requests.

The outcome is an indication whether the service is used as expected or is being used abnormally.

Note: the service usage performance indication can be computed for any of the services of the NWDAF.

End of modifications