- 3GPP TSG-SA5 Meeting #145e	S5-225056

- August 2022, E-meeting

- Source:	Nokia

- Title:	pCR 28. 912 Potential solution on Intent conflicts

- Document for:	Approval

- Agenda Item:	6.7.3.2

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	3GPP draft TR28.912: “Study on enhanced intent driven management services for mobile networks”.

# 3	Rationale

It is expected that there can be conflicts in intent-driven management either on intents, expectation or targets. The requirements to address such conflicts have been added to TR28.912. This contribution adds a potential solution for such conflicts.

# 4	Detailed proposal

Start of modification

## 4.2	Issue#4.2:	Intent conflicts

### 4.2.1	Description

The MnS consumer may create an intent containing two or more intent expectations, and each intent expectation may contain multiple expectation targets. For example, a Radio Network related intent may express an intent with targets on radio network parameters (like, downlink transmit power, remote electrical tilt) or on KPIs (like, DL UE throughput target, average RSRP target, coverage area). On receiving and after analysing the intent, the MnS producer may realize that the intent expectations or expectation targets in one intent are contradicted, i.e. the MnS producer may detect conflicts in the intent. Also an intent is considered to have conflict with other intents if the requirements (includes intent expectation and corresponding expectation targets) stated in one intent is conflicted with the requirements (includes intent expectation and corresponding expectation targets) stated in another intent. Following are the intent related conflict scenarios:

- Target conflict, which represents the conflict between two or more expectation targets within the same intent expectation.

- Expectation conflict, which represent the conflict between two or more intent expectations with the same intent

- Intent conflict, which represent the conflict between two or more different intents.

For example, consider two targets target_1=: throughput > threshold_1 and target_2 :=  interference <  threshold_2, and while trying to achieve target_1, target_2 gets degraded, so the producer will see that the targets are conflicting. The two are intent conflict if they are in different intents, but they are expectation conflict or target conflict if they are in the same intent.When such conflicts are detected, the MnS producer needs to notify the MnS consumer about the conflict, indicating the intent, intent expectations or expectation targets which give rise to the conflict. Additionally, the MnS producer may also notify the MnS consumer about the additional information (e.g. the impact for other expectation targets when fulfill the specified expectation targets in the same or different intent) for the conflict.

Thereby, the MnS consumer may task actions (e.g. modify and delete the intent or intent expectation or expectation targets,) to address such intent conflict or MnS consumer may give some intent conflict handling guidelines (e.g. assign priority for such intent or intent expectation or expectation targets) to MnS producer to solve such intent conflict or eliminate the terminated state which is caused by the reason of conflict detected.

REQ-Intent_conflict-CON-1: The intent driven MnS should have the capability to inform the authorized MnS consumer about intent related conflicts, including intent conflict, expectation conflict and target conflict.

REQ-Intent_conflict-CON-2: The intent driven MnS should have the capability allow the authorized MnS consumer to give intent conflict handling guidelines to MnS producer to solve such intent conflict and potentially affect the terminated state which is caused by the reason of conflict detected.

### 4.2.2	Potential Solution

When the MnS producer detects a conflict on an intent, an intent expectation or an expectation target,

- First, the MnS producer should notify the MnS consumer whenever such a conflict is detected with intent, intent expectations or expectation targets specified which give rise to the conflict.

- the MnS producer may have three options on how to handle the conflict and which could also be configurable:

  - The MnS producer may reject the intent and send the notification of the rejection message to MnS consumer providing the cause for rejection as the conflict. Additionally, the intent progress status should be marked as terminated with the reason as conflict detected.

  - MnS producer accepts the intent and executes the best possible solution it finds appropropriate or that it is configured to apply.

  - The MnS producer provides to the MnS consumer an indication of the best alternative solution asking the MnS consumer to either approve or reject the alternative solution.

- Multiple methods may be available on how to derive best possible or alternative solution.

- For example, the  MnS consumer can give intent conflict handling guidelines to MnS producer which the MnS producer then applies. As an example,  each Intent, intent expectation or expectation target may be characterized by a priority and the guideline from the MnS consumer may be to apply the highest priority intent, intent expectation or expectation target. The MnS producer then applies this guideline to accept one of the Intent, intent expectation or expectation targets. The other are rejected providing a notofcation with the reason as   conflict and potentially affect terminated state which is caused by the reason of conflict detected. Note: This may imply that some of the MnS consumers will accept the unwanted outcomes, e.g. those MnS consumers whose intents, intent expectations or expectation targets have the lowest priority.

- The alternative solution may for example be derived based on  compromise derived from information given by the MnS consumers whose Intents, intent expectations or expectation targets are conflicting. Example information to derive such a compromise may be the relative pririties of the Intenst, intent expectations and expectation targets or their relative utilities.

Note: Calculation of the compromised value may depend and vary based on the specified target(s). For example, consider the two intents: (i) intent 1 {target: TTT = t1} and (ii) intent 2 {target: TTT = t2}. In this case, the compromised value of TTT can be calculated as (t1 + t2)/2.

however, if we consider the contradiction example in Expectation conflict of 4.2.1 (target_1=: throughput > threshold_1 and target_2 :=  interference <  threshold_2), in this case MnS producer has to determine the common factor(s) such as the specific network parameters because of which these two targets cannot be satisfied simultaneously. After that, MnS producer may average the values of the common factor(s) which are needed to satisfy target_1 and target_2.

X.1.5	Evaluation

TBD

End of modifications