- 3GPP TSG-SA5 Meeting #	S5-225152

- August 2022, E-meeting

- Source:	Nokia

- Title:	pCR TR28.864 Key issue on NWDAF Analytics-related Timing

- Document for:	Approval

- Agenda Item:	6.7.6.2

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.864-020 “Study on Enhancement of the management aspects related to NWDAF”.

# 3	Rationale

This pCR is to add a potential solution on Performance Management of NWDAF Analytics-related Timing.

# 4	Detailed proposal

Start of modification

## 4.N	Key Issue #N: Performance Management of the NWDAF Analytics-related Timing

### 4.N.1	Description

One area for NWDAF performance management is to study NWDAF’s performance in case of errors due to timer expiration. The optional time “Time when analytics information is needed” could be specified during the analytics subscription/request by the consumer. It indicates to the NWDAF the latest time at which the analytics consumer expects to receive the requested analytics data. Meanwhile, the NWDAF has a delay per Analytics ID, called “Supported Analytics Delay”, that is specified in the NWDAF profile and indicates the amount of time per per Analytics ID that is expected to lapse between the instant that the NWDAF receives a request for analytics and the time point at which the NWDAF delivers the corresponding analytics report

Ideally, the time “Time when analytics information is needed” should not be set to a value lower than “Supported Analytics Delay”. Otherwise, if the time “Time when analytics information is needed” is reached but the analytics information is not ready, the UNSATISFIED_REQUESTED_ANALYTICS_TIME error message might be returned, while if the time “Time when analytics information is needed” is set to a value lower than “Supported Analytics Delay”, it is expected that the NWDAF might not be able to treat the Analytics ID on time.

On the other hand, the specifications indicate that NWDAF should trigger ML model training, but the time required for that operation (which can be significantly long under certain circumstances) is not taken into account in the “Supported Analytics Delay” attribute. In addition, the performance management of NWDAF related to errors generated by expiration of “Time when analytics information is needed” is missing. Thus, there is the need of introducing performance measurement and KPI for performance management of the NWDAF based on this aspect.

The 3GPP management system should introduces the enablers to manage NWDAF performance based on expiration of “Time when analytics information is needed” specified by analytics consumer in the analytics subscription/request. In particular, the 3GPP management system should introduce a mechanism to keep track of analytics production failures due to infringement of timing attributes specified in the analytics subscription/request.

### 4.N.2	Potential requirements

REQ-NWDAF_TIMER-1 the 3GPP management system shall have a capability to allow an authorized consumer to configure the NWDAF to track the number of events/scenarios for which all the times are appropriately configured, which is the case when the "time when Analytics is delivered to the consumer" is lower than “Supported Analytics Delay" and the “Supported Analytics Delay" is lower than  the "Time when Analytics is needed". ".

REQ-NWDAF_TIMER-2 the 3GPP management system shall have a capability to allow an authorized consumer to configure the NWDAF to track the number of events/scenarios for which the analytics is delivered after the supported delay but before the deadline, which is the case when the “Supported Analytics Delay" is lower than "time within which Analytics is delivered to the consumer" and "time within which Analytics is delivered to the consumer" is lower than “Time when Analytics is needed”,

REQ-NWDAF_TIMER-3 the 3GPP management system shall have a capability to allow an authorized consumer to configure the NWDAF to track the number of events/scenarios for which analytics computation is too late. which is the case when the “Time when Analytics is needed” time is lower than “Supported Analytics Delay" and “Supported Analytics Delay" is lower than "time within which Analytics is delivered to the consumer",

End of modifications