- 3GPP TSG SA WG5 Meeting #145-e	S5-225208

- Aug. 15 – Aug. 24, 2022, e-Meeting                                revision of S5-2abcde

- Source:	CMCC

- Title:	pCR 28.838 add use case of load balancing

- Document for:	Approval

Agenda Item:	6.7.8.1

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TR 28.838 “Study on measurement data collection to support RAN intelligence”

# 3	Rationale

This contribution proposes to add the load balancing use case to TR28.838 [1].

# 4	Detailed proposal

-

1st modified section

# 5	Use cases, potential requirements and solutions

## 5.x	Load balancing

### 5.x.1	Description

With the rapid traffic growth, multiple frequency bands were utilized in the commercial network. It is quite challenging to steer the traffic in a balanced distribution so that the network performance could be assured. Load balancing had been proposed to address the issue. The objective of load balancing is to distribute load evenly among cells and among areas of cells, or to transfer part of the traffic from congested cells or from congested areas of cells, or to offload users from one cell, cell area, carrier or RAT to improve network performance. This can be done by means of optimization of handover parameters and handover actions. AI/ML capabilities, e.g., traffic load prediction, prediction of selected UE  and/or target cell for handover, could be introduced to improve the load balance performance, in terms of the quality user experience and system capacity. It also enables the network automation which would help to reduce human intervention in the network management and optimization tasks

### 5.x.2	Use cases

To support the AI/ML based load balancing in RAN, the related AI/ML Model can be trained, validated and tested in OAM while AI/ML Model inference could be done in the gNB. In case of CU-DU split architecture, AI/ML Model Training can be located in the OAM and AI/ML Model Inference can be located in the gNB-CU.

To facilitate the AI/ML Model Training, validation and testing in OAM, the input data, output data as well as the feedback of AI/ML based load balancing need to be collected by the OAM. In [2], to predict the optimized load balancing decisions, the following input data, output data and feedback data are identified in Table 5.x.2.1, 5.x.2.2, 5.x.2.3 (see clause 5.2.2. in TR 37.817 [2]).

#### 5.x.2.1	Input data for AI/ML-based load balancing

The input data for AI/ML-based load balancing is provided in table 5.x.2.1.

Table 5.1.2.1: Input data for AI/ML-based load balancing

| Data category | Description |
| --- | --- |
| From the local RAN node | Current and predicted own resource status |
|  | UE trajectory prediction |
|  | Current and predicted UE traffic |
|  | Predicted resource status information of neighbouring NG-RAN node(s) |
|  | Inter-gNB conditional handovers |
|  | Intra-gNB conditional handovers |
| From neighbouring NG-RAN nodes | Current and predicted resource status |
|  | UE performance measurement at traffic offloaded neighbouring cell |
| From the UE | UE location information (e.g., coordinates, serving cell ID, moving velocity) interpreted by gNB implementation when available |
|  | UE Mobility History Information |
|  | UE measurement report (e.g., UE RSRP, RSRQ, SINR measurement, etc), including cell level and beam level UE measurements |

#### 5.x.2.2  Output of AI/ML-based Load Balancing

The output data for AI/ML-based load balancing is provided in table 5.x.2.2.

Table 5.1.2.2: output data for AI/ML-based load balancing

| Data category | Description |
| --- | --- |
| Load Balancing Optimization Decision | Selection of target cell for load balancing |
|  | The predicted UE(s) selected to be handed over to target NG-RAN node (will be used by RAN node internally) |
| Prediction results of resource status | Predicted own resource status information |
|  | Predicted resource status information of neighbouring NG-RAN node(s) |

#### 5.x.2.3    Feedback of AI/ML-based Load Balancing

The feedback data for AI/ML-based load balancing is provided in table 5.x.2.3.

| Data category | Description |
| --- | --- |
| UE performance information | UE performance information from target NG-RAN |
| Resource Status Information | Resource status information updates from target NG-RAN |
| System KPIs | throughput, delay, RLF of current and neighbour cells |

### 5.x.3	Potential requirements

REQ-LB-01: 3GPP management service producer should have the ability allowing the authorized consumer to collect the input data for AI/ML-based load balancing from the serving RAN node and the neighbouring RAN node.

REQ-LB-02: 3GPP management service producer should have the ability allowing the authorized consumer to collect the input data for AI/ML-based load balancing from the UE.

REQ-LB-03: 3GPP management service producer should have the ability allowing the authorized consumer to collect the output data for AI/ML-based load balancing.

REQ-LB-04: 3GPP management service producer should have the ability allowing the authorized consumer to collect the feedback data for AI/ML-based load balancing,

End of modified section