3GPP TSG-SA5 Meeting #145-e 	S5-225247

- e-meeting, 15 - 24 August 2022

- Source:	Huawei

- Title:	Add potential solution of performance measurements for SMF and UPF

- Document for:	Approval

Agenda Item:	6.8.4.3

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	        3GPP draft TR 28.833: "Study on management aspects of 5GLAN"

# 3	Rationale

This contribution is to add the potential solution to support the performance measurements for SMF and UPF, which can address the key issue agreed.

# 4	Detailed proposal

1st Change

### 5.3.4	Solutions

5.3.4.1		Performance measurements for SMF

A number of performance measurements for SMF are specified in clause 5.3 of TS 28.552.  For example, Number of PDU sessions, Number of successful PDU session creations, Number of failed PDU session creations and Number of successful PDU session modifications (UE initiated) are specified and can be found in clause 5.3 of TS 28.552.

A DNN and S-NSSAI is associated with a 5G VN group. Then, the DNN and S-NSSAI of a SMF can support distinguishing the a 5G VN group communication.

As described in clause 5.2.4	in this document, the SMF configured to serve a certain 5G VN group communication can hold the DNN and S-NSSAI information during PDU session establishment procedure. The performance measurement related to SMF serving a 5G VN group communication can be enhanced to support a sub-counter sorted by DNN and S-NSSAI in this potential solution.

5.3.4.2		Performance measurements for UPF

A number of performance measurements for UPF are specified in clause 5.4 of TS 28.552. For example, Number of incoming GTP data packets on the N3 interface, Number of outgoing GTP data packets of on the N3 interface, Data volume of incoming GTP data packets per QoS level on the N3 interface and Data volume of outgoing GTP data packets per QoS level on the N3 interface are specified and can be found in clause 5.4 of TS 28.552.

A DNN and S-NSSAI is associated with a 5G VN group. Then, the DNN and S-NSSAI of a SMF can support distinguishing the a 5G VN group communication.

Currently, the UPF is capable to maintain DNN and S-NSSAI during a PDU session for a 5G VN group communication. The performance measurements related to UPF is potentially to be enhanced to support a sub-counter sorted by DNN and S-NSSAI in this potential solution.

End of changes