3GPP TSG-SA5 Meeting #145e	S5-225514

- August 15 – 24, 2022, e-Meeting								       		 				             s5-2abcde

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.538 | CR | 0019 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add a use case for EAS discovery failure measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eECM |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The use case of EAS discovery failure measurement is needed to support the dynamic EAS instantiation triggering, as specified in TS 23.558. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a use case for EAS discovery failure measurement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Dynamic EAS instantiation triggering in TS 23.558 is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.2.6 (new), 5.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 5	Edge Computing Management (ECM) capabilities

## 5.1	Lifecycle management

### 5.1.14	Instantiation triggered by EAS discovery failure

EES may need to trigger dynamic EAS instantiation when EES fails to discover and select the EAS that matches the UE location and the requesting application characteristics EAS (see table 8.5.3.2-2 in TS 23.558 [2]) due to no available EAS during the EAS discovery (see clause 8.5 in TS 23.558 [2]).

A consumer would consume performance assurance MnS to request the ECSP management system to collect EAS discovery failure measurementthat will be used to determine whether  an EAS needs to be initiated, based on performance information, such as UE locations, application characteristics, and number of UEs that have failed in the EAS discovery.

### 5.1.15	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-EAS-INST-FUN-1 | Generic provisioning MnS producer should have a capability allowing an authorized consumer to request the deployment of EAS based on the given deployment requirements. | EAS Deployment |
| REQ-EAS-INST-FUN-2 | Generic Provisioning MnS Producer should have the capability to deploy EAS at a suitable EDN which can support the EAS requirements e.g. serving location, required latency, affinity/anti-affinity with other EAS, service continuity. | EAS Deployment |
| REQ-EAS-INST-FUN-3 | Generic provisioning MnS producer should have a capability to inform the authorized consumer about the progress of instantiation as the response to the deployment request. | EAS Deployment |
| REQ-EAS-INST-FUN-4 | Generic provisioning MnS producer should have a capability to notify the authorized consumer the result (e.g. success, failure) of instantiation operation. | EAS Deployment |
| REQ-EAS-TERM-FUN-1 | Generic provisioning MnS producer should have a capability allowing an authorized consumer to request the termination of the EAS VNF instance. | EAS Termination |
| REQ-EAS-TERM-FUN-2 | Generic provisioning MnS producer should have a capability to inform the authorized consumer about the progress of termination as the response to the termination request. | EAS Termination |
| REQ-EAS-TERM-FUN-3 | Generic provisioning MnS producer should have a capability to notify the authorized consumer the result (e.g. success, failure) of termination operation. | EAS Termination |
| REQ-EAS-QUERY-FUN-1 | Generic provisioning MnS producer should have a capability allowing an authorized consumer to obtain the EAS instance information. | Query EAS information |
| REQ-EAS-MOD-FUN-1 | Generic provisioning MnS producer should have a capability allowing an authorized consumer to request the modification of the EAS instance. | EAS Modification |
| REQ-EES-INST-FUN-1 | Generic provisioning MnS producer should have the capability to instantiate the EES, as per request from authorized consumers. | EES Deployment |
| REQ-EES-INST-FUN-2 | Generic provisioning MnS producer should have the capability to send the notification indicating the status of EES instantiation | EES Deployment |
| REQ-EES-INST-FUN-3 | Generic provisioning MnS producer should have the capability to relate instantiated EES with one or multiple served EAS(s). | EES Deployment |
| REQ-EES-TERM-FUN-1 | Generic provisioning MnS producer should have the capability to terminate the EES with the EES identifier, as per request from authorized consumers | EES Termination |
| REQ-EES-TERM-FUN-2 | Generic provisioning MnS producer should have the capability to send the notification indicating the status of EES termination | EES Termination |
| REQ-EES-QUERY-FUN-1 | Generic provisioning MnS producer should have a capability allowing an authorized consumer to obtain the EES instance information. | Query EES information |
| REQ-EES-MOD-FUN-1 | Generic provisioning MnS producer should have a capability allowing an authorized consumer to request the modification of the EES instance. | EES Modification |
| REQ-EES-TERM-FUN-3 | Generic provisioning MnS producer should have a capability to inform the authorized consumer about the progress of EES instantiation as the response to the deployment request. | EES Deployment |
| REQ-EES-TERM-FUN-4 | Generic provisioning MnS producer should have a capability to inform the authorized consumer about the progress of EES termination as the response to the termination request. | EES Termination |
| REQ-ECS-INST-FUN-1 | Generic provisioning MnS producer should have the capability to instantiate the ECS, as per request from authorized consumers. | ECS Deployment |
| REQ-ECS-INST-FUN-2 | Generic provisioning MnS producer should have the capability to send the notification indicating the status of ECS Instantiation. | ECS Deployment |
| REQ-ECS-INST-FUN-3 | Generic provisioning MnS producer should have the capability to relate instantiated ECS with one or multiple served EES(s). | ECS Deployment |
| REQ-ECS-TERM-FUN-1 | Generic provisioning MnS producer should have the capability to terminate the ECS with the ECS identifier, as per request from authorized consumers. | ECS Termination |
| REQ-ECS-TERM-FUN-2 | Generic provisioning MnS producer should have the capability to send the notification indicating the status of ECS termination. | ECS Termination |
| REQ-ECS-TERM-FUN-3 | Generic provisioning MnS producer should have a capability to inform the authorized consumer about the progress of ECS instantiation as the response to the deployment request. | ECS Termination |
| REQ-ECS-TERM-FUN-4 | Generic provisioning MnS producer should have a capability to inform the authorized consumer about the progress of ECS termination as the response to the termination request. | ECS Termination |
| REQ-ECS-QUERY-FUN-1 | Generic provisioning MnS producer should have a capability allowing an authorized consumer to obtain the ECS instance information. | Query ECS information |
| REQ-ECS-MOD-FUN-1 | Generic provisioning MnS producer should have a capability allowing an authorized consumer to request the modification of the ECS instance. | ECS Modification |
| REQ-EAS-DF-FUN-1 | Performance assurance MnS producer should have a capability allowing an authorized consumer to request the collection of EAS discovery failure measurements that contain UE location and the requesting application characteristics information. | Instantiation triggered by EAS discovery failure |
| REQ-EAS-DF-FUN-1 | 3GPP Management system should have a capability allowing an EES to trigger or request for an EAS deployment. | Instantiation triggered by EAS discovery failure |

End of Modified Sections