3GPP TSG-SA5 Meeting #145e	S5-225515

- August 15 – 24, 2022, e-Meeting								       		 			   s5-225515, S5-225531

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0383 | rev | - | Current version: | 17.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add a EAS discovery failure measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eECM |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | EAS discovery failure measurement is needed to enable the consumer to determine if new EAS VNF needs to be instantiated based on UE locations, EAS types, and the number of UE experiencing the EAS discovery failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add EAS discovery failure measurement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Dynamic EAS instantiation triggering in TS 23.558 is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.15.3.3 (new). 5.17.1.1, 5.17.1.2, Annex 100 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[3]	3GPP TS 32.404: "Performance Management (PM); Performance measurements - Definitions and template".

[4]	3GPP TS 23.501: "System Architecture for the 5G System".

[5]	IETF RFC 5136: "Defining Network Capacity".

[6]	3GPP TS 38.473: "NG-RAN; F1 Application Protocol (F1AP)".

[7]	3GPP TS 23.502: "Procedures for the 5G System".

[8]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[9]	3GPP TS 32.425: "Performance Management (PM); Performance measurements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[10]	3GPP TS 32.451: "Key Performance Indicators (KPI) for Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Requirements".

[11]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[12]	Void.

[13]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".[14]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[15]	Void.

[16]	3GPP TS 29.244: "Technical Specification Group Core Network and Terminals; Interface between the Control Plane and the User Plane Nodes; Stage 3".

[17]	ETSI GS NFV-IFA027 v2.4.1: "Network Functions Virtualisation (NFV); Management and Orchestration; Performance Measurements Specification".

[18]	Void.

[19]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[20]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[21]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[22]	3GPP TS 29.413: "Application of the NG Application Protocol (NGAP) to non-3GPP access".

[23]	3GPP TS 29.122: "Technical Specification Group Core Network and Terminals; T8 reference point for Northbound APIs".

[24]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[25]	ETSI ES 202 336-12 V1.2.1: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[26]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[27]	3GPP TS 29.274: "Evolved General Packet Radio Service (GPRS); Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[28]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[29]	3GPP TS 38.314: "NR; layer 2 measurements".

[30]	3GPP TS 38.313: "Self-Organizing Networks (SON) for 5G networks".

[31]	3GPP TS 38.415: "NG-RAN; PDU session user plane protocol".

[32]	3GPP TS 38.321: "NR MAC protocol specification".

[33]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[34]	3GPP TS 38.215: "NR; Physical layer measurements".

[35]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[36]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[37]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[38]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[39]	3GPP TS 29.507: "5G System; Access and Mobility Policy Control Service; Stage 3".

[40]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[41]	3GPP TS 29.531: "5G System; Network Slice Selection Services".

[42]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)".

[43]	3GPP TS 29.540: "5G System; SMS Services; Stage 3".

[44]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[45]	3GPP TS 29.541: "5G System; Network Exposure FunctionServices for Non-IP Data Delivery (NIDD); Stage 3".

[46]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[47]	3GPP TS 29.504: "5G System; Unified Data Repository Services; Stage 3".

[48]	3GPP TS 29.554: "5G System; Background Data Transfer Policy Control Service; Stage 3".

[49]	3GPP TS 38.300: "NR and NG-RAN Overall description; Stage-2".

[50]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".

[51]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[52]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[53]	3GPP TS 23.273: "5G System (5GS); Location Services (LCS); Stage 2".

[54]	3GPP TS 29.572: "5G System (5GS); Location Management Services; Stage 3".

Next Modified Sections

- 5.15.1	EAS Discovery procedure related measurements

- 5.15.1.1	Number of discovery requests

a)	This measurement provides the number of EAS discovery requests (see clause 8.5.2 of TS 23.558 [52]) received by the EES.

b)	CC

c)	On receipt by the EES from the EEC of EAS Discovery Request. Each discovery request is added.

d)	A single integer value

e)	DIS.EasDisReq

f)	EESFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this performance measurements is for EES performance assurance.

- 5.15.1.2	Number of successful discovery request

a)	This measurement provides the number of successful EAS discovery request at the EES.

b)	CC

c)	On transmission of EAS Discovery Response (see clause 8.5.2 of TS 23.558 [52]) by the EES to the EEC that sent the registration request. Each accepted request is added.

d)	A single integer value

e)	DIS.EasDisSucc

f)	EESFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this performance measurements is for EES performance assurance.

### 5.15.1.3	EAS discovery failure

a)	This measurement provides the number of EAS discovery failures when EES fails to discover and select a suitable EAS.

b)	CC

c)	On transmission of EAS discovery response (see clause 8.5.2 in TS 23.558 [52]) by the EES to the EEC that indicates the EAS discovery failure, the measurement is obtained by incrementing the appropriate subcounters, shown below:

- UeLocation (see table 8.5.3.2-1 in TS 23.558 [52]) and EasType (see table 8.5.3.2-2 in TS 23.558 [2]), as contained in the corresponding EAS discovery request (see clause 8.5.2 in TS 23.558 [52]).

- Cause of EAS discovery request failure (see table 8.5.3.3-1 in TS 23.558 [52]), as contained in the corresponding EAS discovery response (see clause 8.5.2 in TS 23.558 [52]).

d)	Each subcounter is an integer value

e)	DIS.EasDisFail.UeLocation.EasType

where UeLocation: identifies where the UE is connected to the network or the position of the UE (see clause 7.3.2 in TS 23.558 [2]), and is represented by the cell ID.

EasType: represents the category or type of required EAS (e.g., V2X) (see table 8.5.3.2-2 in TS 23.558 [2])

DIS.EasDisFail.cause, where cause indicates the cause of EAS discovery request failure

f)	EESFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this performance measurements is for EES performance assurance.

Next Modified Sections

## 5.17	Performance measurements for ECS

### 5.17.1	Service provisioning procedure related measurements

#### 5.17.1.1	Number of service provisionig requests

a)	This measurement provides the number of Service provisioning requests (see clause 8.3.3 of TS 23.558 [52]) received by the ECS.

b)	CC

c)	On receipt by the ECS from the EEC of Service provisioning request. Each provisioning request is added.

d)	Each subcounter is an integer value

e)	SP.SerProvReq

f)	ECSFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this performance measurements is for ECS performance assurance.

#### 5.17.1.2	Number of successful discovery

a)	This measurement provides the number of successful Service provisioning request at the ECS.

b)	CC

c)	On transmission of Service provisioning response (see clause 8.3.3 of TS 23.558 [52]) by the ECS to the EEC that sent the provisioning request. Each accepted request is added.

d)	Each subcounter is an integer value

e)	SP.SerProvSucc

f)	ECSFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this performance measurements is for ECS performance assurance.

Next Modified Sections

# A.100	Use case of measurements for EES.

EES related measurements are used to measure the performance of an EES on each of the supported functionality.

Discovery procedures enable entities in an edge deployment to obtain information about EAS and their available services, based on specified criteria of interest. It is useful to analyse the EAS discovery success rate in order to assess EES performance. The discovery procedure may fail due to the unavailability of the relavant EAS. If the failure rate increases beyond a defined threshold, corrective actions (e.g instantiating the required EAS) can be taken by the OAM systems. Hence, it is necessary to collect measurement related with EAS discovery procedures. EAS discovery failure measurement can be used to mitigate the EAS discovery failure issue, based on the UE locations, application characteristics, and number of UEs that have failed in the EAS discovey during the EAS docsovery failure.

An EEC performs registration with an EES in order to provide information that can be used by the EES in Edge Computing services. It is useful to analyse the EEC registration success rate in order to assess EES performance. If the failure rate increases beyond a defined threshold, corrective actions can be taken by the OAM systems. Hence, it is necessary to collect measurement related with EEC Registration procedures.

The EAS Registration procedure allows an EAS to provide its information to an EES in order to enable its discovery. It is useful to analyse the EAS registration success rate in order to assess EES performance. If the failure rate increases beyond a defined threshold, corrective actions can be taken by the OAM systems. Hence, it is necessary to collect measurement related with EAS Registration procedures.

End of Modified Sections