3GPP TSG SA WG5 Meeting 144-e	S5-225564

- electronic meeting, online, 15th – 24th August 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0132 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove example from network slice subnet instance modification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netslice |  |  |  |  |  | Date: |  |  | 2022-06-10 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use case for network slice subnet instance modification includes an example. As the example is just that it would be better to remove the example from the use case text and keep the use case description crisp and clean. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Example has been removed - Begins when has been rephrased to “The network slice subnet management service producer receives request from its authorized consumer…” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use case is not entirely correct and and may cause confusion leading to incompatible implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No impact on the procedures |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.1.12	Network slice subnet instance modification

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To modify an existing network slice subnet instance |  |
| Actors and Roles | Network slice subnet provisioning management service consumer. |  |
| Telecom resources | Network slice subnet instance Network SliceSubnet slice subnet provisioning management service provider. |  |
| Assumptions | N/A |  |
| Pre-conditions | N/A |  |
| Begins when | The network slice subnet provisioning management service provider receives request from its authorized consumer with new sets of network slice subnet related requirements and decides to modify an existing NSSI. |  |
| Step 1 (M) | The network slice subnet provisioning management service provider identifies the NSSI constituents as well as the transport network (TN) part within the NSSI that needs to be modified, and generates new sets of requirements for the NSSI constituents and transport network if needed. |  |
| Step 2 (M) | The network slice subnet provisioning management service provider checks whether the requirements for the identified NSSI constituents managed by itself could be satisfied, and then triggers the modification of the corresponding NSSI constituents if needed. |  |
| Step 3 (M) | If the NSSI consists of constituent NSSI managed by other network slice subnet provisioning management service provider, and the constituent NSSI is identified to be modified, the network slice subnet provisioning management service provider sends modification request to other network slice subnet provisioning management service provider which manages the constituent NSSI with new sets of constituent NSSI requirements. | Network slice subnet instance modification use case |
| Step 4 (M) | If the NS instance associated with the NSSI needs to be modified, the network slice subnet provisioning management service provider derives the new sets of NS related requirements and triggers corresponding NS instance request to NFVO with Os-Ma-nfvo interface as described in clause 6.4.3 in TS 28.525 [2]. | TS 28.525 [2] Clause 6.4.3 NS instance use cases |
| Step 5 (M) | If the related TN part of the NSSI is identified to be modified, the network slice subnet provisioning management service provider derives new sets of requirements for the TN part and coordinates with the corresponding TN management system. |  |
| Step 6 (M) | The network slice subnet provisioning management service provider generates the modification result based on the received response and send response to its authorized consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The NSSI is modified. |  |
| Traceability | REQ-PRO_NSSI-FUN-11 |  |

End of changes