3GPP TSG-SA5 Meeting #145-e 	S5-225603

e-meeting, 15 - 24 August 2022

Title:	Reply LS on Transport Network Slicing Enhancement IM/DM TS28.541

Response to:	LS S5-225025 Resubmitted LS on O-RAN – Transport Network Slicing Enhancement IM/DM TS28.541 (O-RAN)

Release:	Rel-17

Work Item:	ad_NRM

Source:	3GPP SA5#145-e

To:	O-RAN

Cc:	3GPP SA, RAN

Contact person:	Deepanshu Gautam

Deepanshu.g@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA5 thanks O-RAN for the LS Transport Network Slicing Enhancement IM/DM TS28.541 and would like to provide the responses as follows:

Question 1: O-RAN WG9 kindly asks 3GPP SA5 to provide guideline on how transport network information may be reflected in TS 28.541 6.3.18.2  EP_Transport.

Answer 1: EP_Transport is defined in relation with network slicing. The attribute epTransportRef, in NetworkSliceSubnet is used to specify a list of EP_Transport instance as transport resources to be aggregated to a NetworkSliceSubnet instance. The consumer determines the EP_Transport instance(s) to support EP_Application (EP_N3, EP_NgU etc.) instances as part of the NetworkSliceSubnet instance and request the producer to configure the attribute epTransportRef of the NetworkSliceSubnet. EP_Transport includes transport topology related info, reachability info, QoS requirement, etc. as defined as part of EP_Transport IOC and the related attributes definition in TS 28.541. SA5 would like understand what information, in addition to what is already defined as part of EP_Transport IOC, may be reflected in EP_Transport. Once that is determined we will be able to suggest on how to include that as part of NRM in TS 28.541.

Question 2: O-RAN WG9 kindly asks 3GPP SA5 to provide guideline on how to extract the following key values information on per network slice fashion, such as for example:

- Unique ID of the slice  [local attribute or proxy_class]. Index, related to SR/Label

- Throughput constraints: Uplink_Demand and Downlink_Demand, Traffic direction

- Latency constraint demand

- Transport protection constraint demand

- Service Level Agreement Availability value

- Maximum MTU size [Payload of interest]

- Maximum Jitter constraint

- Temporary values [activation / deactivation time stamp – activate values from XX:XX to YY:YY]

- Security / Policy constraint to encode the propagation of the connectivity per domain/geography

Answer 2: SA5 would like to provide response as follows. All responses are based on the definitions in TS 28.541.

- Unique ID of the slice is the NetworkSlice DN

- Throughput constraints, Latency constraints, Maximum MTU size, Maximum Jitter, SLA Availability and Security constraints are defined as part of ServiceProfile/SliceProfile.

- Activation and deactivation time stamp can be ascertained by the attributes of operationalState and administrativeState defined as part of NetworkSliceSubnet IOC.

# 2	Actions

To O-RAN

ACTION: 	SA5 kindly asks O-RAN to take the above information into account.

# 3	Dates of next TSG SA WG 5 meetings

SA5#146		14 - 18 November 2022	Europe (TBC)

SA5#146 Ad-Hoc	16 - 20 January 2023	Electronic meeting