3GPP TSG-SA5 Meeting #145-e 	S5-225125

# e-meeting, 15 - 24 August 2022

# Source:	MATRIXX Software

# Title:	pCR TR 32.847 Solve Editor’s Notes on solution#1.3

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

This pCR is to Solve Editor’s Notes on solution#1.3

# 2	References

[1]		3GPP TR 32.847 "Study on Charging Aspects for Network Slicing Phase 2"

# 3	Rationale

This pCR is to Solve Editor’s Notes on solution#1.3.

- The first Editor's Note: based on the architecture of solution#1.3, the NF is the NSACF

Editor’s Note: The NF that will hold the CTF is FFS

- The second Editor's Note: the usual capability of CHF to supply new set of triggers is used, and

Editor’s Note: How the thresholds and triggers are to be handled is FFS.

# 4	Detailed proposal

The following changes are proposed to be incorporated into TR 32.847 [1]

First change

#### 6.1.3.6	Charging scenarios

Determination of passing a threshold for a network slice using Session based charging (SCUR).

Figure 6.1.3.6-1: SCUR – UE addition or removal to network slice

0. Threshold configured as initial trigger: N1.

1)	UE to be added: A UE is to be added to the network slice.

2)	Trigger evaluated: the NSACF (CTF) determines that the addition of the UE would cause the N1 threshold to be passed.

3)	Charging Data Request [Initial]: the NSACF (CTF) starts the charging session by sending a request to the CHF for the passing of the threshold.

4)	Account, Rating Control: the CHF may use the information to rate and make deductions from customer's account balance.

5)	 Create CDR: the CHF creates a CDR related to the service.

6)	Charging Data Response [Initial]: the CHF informs the NSACF (CTF) on the result of the request, the CHF may provide new thresholds indicated with corresponding operation "update or termination": e.g. N2 (update), N3 (termination).

7)	 UE added: The UE is added to the network slice.

8)	UE to be added or removed: a UE is to be added to or removed from the network slice.

9)	Trigger evaluated: the NSACF (CTF) determines that the change in number of UE would cause a CHF provided (e.g. N2) to be passed.

10)	Charging Data Request [Update]: the NSACF (CTF) updates the charging session by sending a request to the CHF for the passing of the threshold.

11)	Account, Rating Control: the CHF may use the information to rate and make deductions from customer's account balance.

12)	 Update CDR: the CHF updates the CDR related to the service.

13)	Charging Data Response [Update]: the CHF informs the NSACF (CTF) on the result of the request. The CHF may provide new thresholds indicated with corresponding operation "update or termination" overriding step 6 triggers.

14)	 UE added: if the trigger was addition of UE then the UE is added to the network slice.

15)	UE removed: a UE is to be removed from the network slice.

16)	Trigger evaluated: the NSACF (CTF) determines that the removal of UE would cause the termination threshold trigger (e.g. N3) to be passed.

17)	Charging Data Request [Termination]: the NSACF (CTF) terminates the charging session by sending a request to the CHF for the passing of the threshold.

18)	Account, Rating Control: the CHF may use the information to rate and make deductions from customer's account balance.

19)	 Close CDR: the CHF closes the CDR related to the service.

20)	Charging Data Response [Termination]: the CHF informs the NSACF (CTF) on the result of the request.

End of changes