3GPP TSG-SA5 Meeting #145-e 	S5-225127

# e-meeting, 15 - 24 August 2022

# Source:	MATRIXX Software

# Title:	pCR TR 32.847 Evaluation and conclusion for Key issue#1

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

This pCR is to introduce the evaluation and conclusion for Key issue#1

# 2	References

[1]		3GPP TR 32.847 "Study on Charging Aspects for Network Slicing Phase 2"

# 3	Rationale

This pCR is to introduce the evaluation and conclusion for Key issue#1.

# 4	Detailed proposal

The following changes are proposed to be incorporated into TR 32.847 [1]

First change

### 6.1.x	Evaluation

This clause evaluates the solutions for Key issue#1, they are all based on the new Nchf_NSConvergedCharging service.

Solution #1.1 partially solves Key issue#1:

Pro:

- Use of CCS capability of Network Slice quota management to dynamically grant additional NS_quota of "Nb of Reg UEs", to go beyond the NSACF configured "max Nb of Reg UEs", based on charging criteria.

Cons:

- Not possible to apply specific rate and costs for different ranges of "Nb of Reg UEs" (before and the NSACF configured "max Nb of Reg UEs" is reached).

Solutions #1.2 and #1.4 partially solves Key issue#1:

Pro:

- Possible to apply specific rate and costs for different ranges of "Nb of Reg UEs" (before and the NSACF configured "max Nb of Reg UEs" is reached) using pre-configured static thresholds.

- Event-based to CHF (No charging session).

Cons:

- Not possible to go beyond the NSACF configured "max Nb of Reg UEs" for the Network Slice.

- Not possible to dynamically tune the ranges of "Nb of Reg UEs" for specific rate and costs.

Solutions #1.3 and #1.5 partially solve Key issue#1:

Pro:

- Possible to apply specific rate and costs for different ranges of "Nb of Reg UEs", under the NSACF configured "max Nb of Reg UEs".

- Possible to dynamically tune the ranges of "Nb of Reg UEs" for specific rate and costs.

Cons:

- Not possible to go beyond the NSACF configured "max Nb of Reg UEs" for the Network Slice.

Solutions #1.5 (CEF) compared to solution #1.3 does not allow the CHF (see 6.1.3.6	Charging scenarios) to act before a UE is added or removed.

Solution #1.x, by combining solutions#1.1 and #1.3, fully solves Key issue#1:

Pro:

- Use of CCS capability of Network Slice quota management to dynamically grant additional NS_quota of "Nb of Reg UEs", to go beyond the NSACF configured "max Nb of Reg UEs", based on charging criteria.

- Possible to apply specific rate and costs for different ranges of "Nb of Reg UEs".

- Possible to dynamically tune the ranges of "Nb of Reg UEs" for specific rate and costs.

Cons:

- Charging session to CHF.

For all solutions, the particular REQ-NSCH-06 is covered under the NSACF behavior which counts UEs connected in EPS when EPS counting is required. In this case, the network description of the solution relates to interworking with EPC procedures instead of 5GC procedures involving the AMF.

### For all solutions, the particular REQ-NSCH-04 cannot be fulfilled: based on clause 5.15.11.0 of TS 23.501 [11], counts in the NSACF also include emergency registered UEs undifferentiated from registered UEs.

Next change

### 6.1.y	Conclusion

Based on evaluation in clause 6.1.x, it is concluded for normative work:

- on solution #1.x as a complete solution;

- on solution #1.2 for the event-based scenario.

With the explicit description of the limitation on emergency registered UEs which cannot be excluded from the quota.

End of changes