3GPP TSG-SA5 Meeting #145-e 	S5-225128

# e-meeting, 15 - 24 August 2022

# Source:	MATRIXX Software

# Title:	pCR TR 32.847 New NSACF (CTF) - NS quota management solution for Key issue#2

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

This pCR is to introduce a New NSACF (CTF) - NS quota management solution for Key issue#2

# 2	References

[1]		3GPP TR 32.847 "Study on Charging Aspects for Network Slicing Phase 2"

[2]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

# 3	Rationale

This pCR is to introduce a new NSACF (CTF) - NS quota management solution for Key issue#2 similar as solution #1.x. with "PDU sessions" replacing "Number of UEs".

# 4	Detailed proposal

The following changes are proposed to be incorporated into TR 32.847 [1]

First change

### 6.2.x	Solution#2.x NSACF (CTF) – NS quota management

This solution is the same as solution#1.x with:

-	NS_quota management between the NSACF and CHF of solution#1.1 combined with solution #2.2;

-	"number of UEs" and "Nb of Reg UEs" replaced by "number of the PDU Sessions";

-	NS admission control on number of PDU sessions triggered from SMF (instead of AMF) per 3GPP TS 23.502 [11] clause 4.2.11.4 Number of PDU sessions per network slice availability check and update procedure.

End of changes