3GPP TSG-SA5 Meeting #145-e	S5-225178

# e-meeting, 15 Aug - 24 Aug 2022

# Source:	Samsung

# Title:	pCR 28.831 Add Key Issue for attribute addition and deletion

# Document for:	Approval

Agenda Item:

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below

# 2	References

[1]		3GPP TS 28.831: " Management and orchestration; Study on basic Service-Based Management Architecture (SBMA) enabler enhancements"

# 3	Rationale

None.

# 4	Detailed proposal

The following changes are proposed for TR 28.831[1].

First Modification

## 4.x	Issue #1: Attribute Addition and Deletion

### 4.x.1	Issue description

The provisioning MnS consumer may wish to add an attribute in an existing MOI due to some new requirements and/or some internal policies. This Key Issue the creation and deletion of an attribute in and from the existing IOC and its MOI(s).

The consumer may wish to add attributes in an existing IOC and related MOI(s). The example would be; the addition of attribute “maxNumberOfPDUSessions” in the MOI(s) of NetworkSliceSubnetProviderCapabilities, in order to enhance the provider capabilities in terms of maximum number of PDU session it can handle. The requirement of this attribute may not be available at the time of MOI creation. The consumer may wish to add this attribute at later point of time in every MOI of the NetworkSliceSubnetProviderCapabilities IOC. The consumer may also choose to add the same attribute in the related IOC. This is to enable the existence of this attribute in all new instances of this IOC. Further, it may also be needed for implementations to add additional implementation specific attributes in NetworkSlice IOC and in related MOI(s) to cater additional requirements.

The consumer may wish to just delete one single attribute from an MOI instead of deleting the entire MOI. An example would be; the updated slice SLA may invalidate the requirement of uEMobilityLevel for a NetworkSlice MOI(s). The consumer may also choose to delete the same attribute from the related IOC. This is to enable the non-existence of this attribute in all new instances of this IOC.

### 4.x.2	Analysis

The existing Generic Provisioning management service doesn’t allow consumer to create or delete a specific attribute for a managed object instance.

### 4.x.3	Potential requirements

- The provisioning MnS Producer shall allow authorized consumers to request for the addition of attribute(s) in an existing MOI.

- The provisioning MnS Producer shall allow authorized consumers to request for the deletion of attribute(s) from an existing MOI.

- The provisioning MnS Producer shall allow authorized consumers to request for the addition of attribute(s) in an existing IOC.

- The provisioning MnS Producer shall allow authorized consumers to request for the deletion of attribute(s) from an existing IOC

### 4.x.4	Potential solution

TBD

### 4.x.5	CR proposal

TBD

### 4.x.6	Conclusion

TBD