3GPP TSG-SA5 Meeting #145-e 	S5-225188

# e-meeting, 15 - 24 August 2022

# Source:	China Mobile

# Title:	Rel-18 pCR 28.828 New charging scenarios and key issues for PNI-NPN topic 1

# Document for:	Approval

Agenda Item:	7.5.4

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	3GPP TR 28.828: "Charging management; Study on charging aspects for enhanced support of non-public networks".

3	Rationale

This pCR proposes to add charging scenarios and key issues for PNI-NPN topic 1 to TR 28.828 [1].

4	Detailed proposal

The following changes are proposed to be incorporated into TR 28.828.

First change

6	Charging scenarios and key issues for PNI-NPN

6.1	Topic 1: Converged charging for access connection in PNI-NPN

6.1.1	Use cases

6.1.1.1	Use case #1a: Network access control

Relying on PLMN network, PNI-NPN provides logically independent 5G non-public network for vertical industry customers through network slice instances/ dedicated DNNs, and supporting of CAG feature. CAG is used for access control to prevent unauthorized UEs from trying to access a PNI-NPN.

The NPN-OP provides the PNI-NPN to NPN-SP (e.g. vertical industry customer), and NPN-SP provides NPN services to NPN-SC. It is important for NPN-OP to collect usage for PNI-NPN resources associated to access connection for the purpose of charging or statistics.

The potential charging requirements for this UC are: REQ-eNPN_CH_PNI-NPN_AC-01, REQ-eNPN_CH_PNI-NPN_AC-02.

6.1.1.2	Use case #1b: Converged charging for number of UEs

For vertical industry consumers, some enterprise UEs may be restricted to only access the non-public network of the enterprise, even if PLMN coverage is available in the same geographic area. To use Closed Access Group, enterprise UE may subscribe the CAG information, indicating CAG Identifiers the UE is allowed to access.

The maximum number of UEs, corresponding to the maximum number of UEs associated with CAG Identifier, represents the maximum number of UEs using the PNI-NPN network simultaneously in restricted area. If the enterprise has one or more CAG Identifiers, the maximum number of UEs can be calculated per CAG Identifier.

For the PNI-NPN with CAG scenarios, it is important for NPN-OP to collect number of UEs per CAG Identifier of vertical industry customers for the purpose of charging or statistics. In addition, NPN-OP can perform tiered charging for different number of UEs threshold.

The potential charging requirements for this UC are: REQ-eNPN_CH_PNI-NPN_AC-03, REQ-eNPN_CH_PNI-NPN_AC-04.

6.1.1.3	Use case #1c: Converged charging for number of PDU sessions

The maximum number of PDU sessions, corresponding to the maximum number of PDU sessions associated with CAG Identifier, represents the maximum number of PDU sessions supported by the PNI-NPN network in restricted area. If the enterprise has one or more CAG Identifiers, the maximum number of PDU sessions can be calculated per CAG Identifier.

For the PNI-NPN with CAG scenarios, it is important for NPN-OP to collect number of PDU sessions per CAG Identifier of vertical industry customers for the purpose of charging or statistics. In addition, NPN-OP can perform tiered charging for different number of PDU sessions threshold.

The potential charging requirements for this UC are: REQ-eNPN_CH_PNI-NPN_AC-03, REQ-eNPN_CH_PNI-NPN_AC-05.

6.1.2	Potential charging requirements

REQ-eNPN_CH_PNI-NPN_AC-01: The 5G system should support converged charging per UE for PNI-NPN network access and usage.

REQ-eNPN_CH_PNI-NPN_AC-02: The 5G system should support converged charging per vertical industry consumer for PNI-NPN network access and usage.

REQ-eNPN_CH_PNI-NPN_AC-03: The 5G system should support converged charging per CAG Identifier for PNI-NPN network access and usage.

REQ-eNPN_CH_PNI-NPN_AC-04: The 5G system should support converged charging per CAG Identifier for PNI-NPN network access and usage, based on "number of UEs".

REQ-eNPN_CH_PNI-NPN_AC-05: The 5G system should support converged charging per CAG Identifier for PNI-NPN network access and usage, based on "number of PDU sessions".

6.1.3	Key issues

This key issue is for investigating how to support converged charging for access connection in PNI-NPN considering REQ-eNPN_CH_PNI-NPN_AC-01, REQ-eNPN_CH_PNI-NPN_AC-02, REQ-eNPN_CH_PNI-NPN_AC-03, REQ-eNPN_CH_PNI-NPN_AC-04 and REQ-eNPN_CH_PNI-NPN_AC-05. This investigation covers the following:

-	Key Issue #2a: Identification of the charging information for converged charging for PNI-NPN network access and usage;

-	Key Issue #2b: Identification of the main interactions with the NFs to obtain the charging information;

-	Key Issue #2c: The charging mechanism supporting converged inter-provider charging for PNI-NPN network access and usage.

End of changes